Supplementary Information: Accelerating Monoclonal Antibody Quality Control: The Role of LC–MS in Upstream Bioprocessing

Table S1: Applied dilution factors for LC–MS analysis

| Day of cultivation | Dilution factor | Injection volume (μL) | Target concentration (ng) |
| --- | --- | --- | --- |
| Day 0-5 | undiluted | 3 | 200-1500 |
| Day 6-8 | 1:3 | 3 | 600-1800 |
| Day 9-13 | 1:5 | 3 | 1000-2000 |
| Purified mAb |  |  |  |
| Elution sample | 1:5 | 3 |  |

Table S2: Eluents used for LC–MS analysis

| Eluent | Composition |
| --- | --- |
| Column + System storage | 100 % Acetonitrile (ACN) |
| LC Wash | 25/25/25/25 diH2O/MeOH/IPA/ACN + 0.1% FA |
| Mobile Phase A | diH2O + 0.1% FA |
| Mobile Phase B | ACN + 0.1% FA |
| Seal Wash | 90/10 ACN/diH2O |
| Wash | 50/50 ACN/diH2O |

Table S3: LC parameters used for sample analysis

| LC Parameters |  |
| --- | --- |
| Column temperature | 60 °C |
| Sample temperature | 8 °C |
| MS Parameters |  |
| Cone voltage | 95 V |
| Measurement range | 400 - 7000 m/z |
| Capillary voltage | 1.5 kV |

Table S4: LC gradient used for sample analysis

| Time (min) | Flow rate (mL/min) | Mobile phase A (%) | Mobile phase B (%) |
| --- | --- | --- | --- |
| 0.00 | 0.4 | 80 | 20 |
| 2.85 | 0.4 | 59 | 41 |
| 3.02 | 0.4 | 15 | 85 |
| 3.22 | 0.4 | 15 | 85 |
| 3.32 | 0.4 | 80 | 20 |
| 3.42 | 0.4 | 80 | 20 |
| 3.52 | 0.4 | 15 | 85 |
| 3.72 | 0.4 | 15 | 85 |
| 3.82 | 0.4 | 80 | 20 |
| 3.92 | 0.4 | 80 | 20 |
| 4.02 | 0.4 | 15 | 85 |
| 4.22 | 0.4 | 15 | 85 |
| 4.32 | 0.4 | 80 | 20 |
| 8.00 | 0.4 | 80 | 20 |

Table S5: Mass errors for the duplicated LC–MS measurements (deconvoluted spectra) of different glycoforms in the varied cultivation conditions

| Mass Error (ppm) | G0F G0F | G0F G1F | G1F G1F | G1F G2F | G2F G2F |
| --- | --- | --- | --- | --- | --- |
| Standardreference | -0.8 | -0.8 | 0.3 | 4.4 | -4.6 |
| Standardreference | -2.0 | -0.6 | -2.7 | 2.7 | -28.7 |
| Man Substitution | -3.7 | -1.6 | -3.6 | -6.0 | -12.5 |
| Man Substitution | -3.9 | 0.8 | -4.3 | -7.3 | 12.4 |
| Glc Deficiency | 4.2 | 39.1 | 5.8 | 19.2 |  |
| Glc Deficiency | 53.2 | 33.8 | 22.5 | 0.9 | - |