Tender specification concrete skin

Glassfibre reinforced concrete panel "concrete skin" for facade cladding

Status: April 2024

General

Information in the text of the invitation for tender must be reviewed and, if necessary, supplemented for the object.

Further liability by Rieder Facades GmbH is excluded. This also applies to printing and writing mistakes and to any subsequent changes. The respective national laws, standards and preliminary remarks for advanced rear-ventilated façades amended from time to time shall apply. Loading, storage and handling provisions of the manufacturer must be observed.

General construction technique permit

Glassfibre reinforced concrete panels "fibreC" to cover exterior walls are certified with a general construction technique permit (allgemeine Bauartgenehmigung) “Z-31.4-166".

Certificate of conformance Austria

Austrian Certificate of Conformance No.: Z-6.1.2-04-4772

concrete skin façade									__________________ m²

Glassfibre reinforced concrete "concrete skin" for covering outer walls, delivery and installation on aluminium substructure.

- fibreC glassfibre reinforced concrete panels produced of mineral raw materials

- Standard panel dimensions 2500 x 1310 x 13 mm, 3100 x 1310 x 13 mm and 3600 x 1310 x 13 mm

- Texture: standard

- Reinforcement by alkaline-resistant glass fibres in the matrix (CEM-Fil, approval Z-3.72-1731), 2-layer textile glass fabric (each longitudinally and cross-wise) and short fibres

- Production method by extrusion

- Dead weight / area weight 26 - -31.5 kg/m² depending on shade

- Bending tension resilience > 18 N/mm² (MOR)**

- Elasticity modulus approx. 10,000 N/mm²

- Frost resistance pursuant to EN 12467

- Building material class A1 not flammable pursuant to DIN 4102 | EN 13501

- Water impermeability pursuant to EN 12467

- Wet-dry test pursuant to EN 12467

- Crystalline silica < 0% - not detectable

- Surface protection: The surface is applied with a transparent hydrophobic coating by default.

** MOR: Modus of Rupture; design values deviate from MOR according to national provisions. The national approvals and provisions on calculation of the rated resistance shall apply

Loading, storage and handling provisions of the manufacturer must be observed.

Ventilated façade covers on existing substructure according to technical provisions and applicable standards.

Supplier concrete skin:	Rieder Facades GmbH

Glemmerstraße 21, 5751 Maishofen, Austria

T +43 6542 690 844, office@rieder.cc, www.rieder.cc

COLOURS

Colour collection greyscale:

polar white 				_____________m²

off-white 				_____________m²

ivory 				_____________m²

silvergrey 				_____________m²

chrome 				_____________m²

anthracite 				_____________m²

liquid black 				_____________m²

Colour collection bricky:

coralline				_____________m²

terracotta 				_____________m²

oxide red				_____________m²

burgundy				_____________m²

merlot				_____________m²

Colour collection timber:

larch				_____________m²

oak					_____________m²

walnut				_____________m²

ebony				_____________m²

terra				_____________m²

Colour collection pietra:

cotton				_____________m²

vanilla				_____________m²

sahara				_____________m²

sandstone				_____________m²

almond				_____________m²

Other colours:

green				_____________m²

Surfaces

ferro: sandblasted surface								_____________m²

ferro light: slightly sandblasted surface (only available on request for formparts)		_____________m²

matt: smooth, brushed surface (not available for formparts)				_____________m²

SPECIAL TEXTURES

terrazzo black ferro plus		_____________m²

vintage matt				_____________m²

groove ferro soft			_____________m²

groove ferro light			_____________m²

groove ferro				_____________m²

twine ferro soft			_____________m²

twine ferro light			_____________m²

twine ferro				_____________m²

glossy solo (interior)			_____________m²

salt’n’pepper ferro plus		_____________m²

salt’n’pepper ferro			_____________m²

luce silver ferro			_____________m²

lumber ferro				_____________m²

slate ferro soft			_____________m²

SPECIAL SIZES

Width: max. 1500 mm

Length: max. 5000 mm

Thickness: 13 – 18 mm

___________ mm width x ___________ mm length x ___________ mm thickness	_____________m²

FORMPARTS

U-shape: 	Leg x width x leg x length: ______________________		______________pcs.

L-shape: 	Leg x leg x length: _____________________________		______________pcs.

Round arc formed part: 	Unwinding length x unwinding width: _________________

Radius: ___________°					______________pcs.

SERVICE

Cut rectangular according to measurements: 				_______________m²

Special cut (waterjet):				 			_______________rm

Mitre cut (45° for edges):							_______________rm

Small cuttings (< 0,7 m²):							_______________pcs.

Undercut anchor drilling for KEIL hs=8.5					_______________pcs.

Though drilling: Diameter 8 mm for rivet attachment 				_______________pcs.

ACCESSORIES/FASTENING

Exposed fastening with rivet

Visible attachment with rivets for aluminium substructure.

Attachment with fixed and sliding points, fixed points must be produced with sleeves.

Attachments: Façade rivet of aluminium with head painting, head diameter 14/16mm, length depending on substructure and panel thickness.

Material: AIMg3 (material no. EN AW-5754 according to DIN EN 573-3:2003-10)

Rivet mandrel: non-corrosive steel (material no. 1.4541)

Fixed point sleeve of aluminium, outer diameter 7.7mm, length depending on panel thickness.

Material: AIMg3 (material no. EN AW-6060 according to DIN EN 573-3:2003-10)

Supplier rivets:	Rieder Facades GmbH

Glemmerstraße 21, 5751 Maishofen, Austria

T +43 6542 690 844, office@rieder.cc, www.rieder.cc

Concealed fastening with undercut anchor

Concealed attachment with undercut anchor from Keil with setting depth 8.5mm. Screw length depending on substructure.

Approval number: ETA-06/0220

Supplier undercut anchor:		KEIL Befestigungstechnik GmbH
				PO box 1158, 51751 Engelskirchen, Germany

T +49 2263 807 0, mail@keil.eu, www.keil.eu

Concealed fastening with RPA Rieder Power Anchor

Approval number: Z-31.4-166

Suplier RPA Rieder Power Anchor:		Rieder Facades GmbH

Glemmerstraße 21, 5751 Maishofen, Österreich

T +43 6542 690 844, office@rieder.cc, www.rieder.cc

Concealed fastening by bonding with Sika Tack panel system

Concealed attachment by bonding with the SikaTack panel system from Sika on aluminium.

Approval number: Z-10.8-408 (supplementary notice on the general building-supervisory approval)

Supplier Sika Tack panel system:	www.sika.com

CONTACT

Sales: 		Rieder Facades GmbH

Glemmerstraße 21

5751 Maishofen

Austria

T +43 6542 690 844

office@rieder.cc

www.rieder.cc

Production:	Rieder Faserbeton-Elemente GmbH

Bergstraße 3a

83059 Kolbermoor

Germany

T +49 8031 90167 0

office@rieder.cc

www.rieder.cc