| Bottle Description | Parameters Analysed | ## Preservation | Filling Instructions | Refrigerate? |
| --- | --- | --- | --- | --- |
| ## BACT Disposable 500mL PET Sterilised DRINKING WATER SAMPLES ONLY | Total coliforms E coli Colony Counts Enterococci Pseudomonas spp. Pseudomonas aeruginosa Clostridium perfringens | Supplied by manufacturer with sodium thiosulphate added Must be used before expiry date shown on bottle Do not use if seal is broken | Fill to shoulder Do not rinse Do not overflow | Yes |
| ## BACT Disposable 500mL PET Sterilised | Total coliforms E coli Colony Counts Enterococci Pseudomonas spp. Pseudomonas aeruginosa Clostridium perfringens | Supplied by manufacturer with sodium thiosulphate added Must be used before expiry date shown on bottle Do not use if seal is broken | Fill to shoulder Do not rinse Do not overflow | Yes |

| Bottle Description | Parameters Analysed | ## Preservation |  | Filling Instructions | Refrigerate? |
| --- | --- | --- | --- | --- | --- |
| ## BACT_1L Disposable 1L PET Sterilised | Total coliforms (Large Volume) E coli (Large Volume) Clostridium perfringens (Large Volume) | Supplied by manufacturer with sodium thiosulphate added Must be used before expiry date shown on bottle Do not use if seal is broken |  | Fill to shoulder Do not rinse Do not overflow | Yes |
| ## LEGIONELLA ## Disposable 1L PET ## Sterilised | Legionella |  | Supplied by manufacturer with sodium thiosulphate added Must be used before expiry date shown on bottle Do not use if seal is broken | Fill to shoulder Do not rinse Do not overflow | If transported on day of sampling, store at ambient temperature ideally between 6°C - 20°C otherwise store at 5±3°C. Store all samples in the dark, separate hot and cold samples. |
| Bottle Description | Parameters Analysed | ## Preservation |  | Filling Instructions | Refrigerate? |
| ## Crypto_10L 10L Grab | Cryptosporidium Giardia |  | None | Fill to shoulder Rinse with sample before filling | No |
| ## Crypto_10L 10L Grab Disposable | Cryptosporidium Giardia |  | None | Fill to shoulder Rinse with sample before filling | No |

| Bottle Description | Parameters Analysed | ## Preservation | Filling Instructions | Refrigerate? |
| --- | --- | --- | --- | --- |
| # PLAS_1L 1L Disposable PET | Algae Chlorophyll a Oil & Grease Biological Oxygen Demand Chemical Oxygen Demand Total Suspended Solids Dry Residues Total Dissolved Solids Detergents | None | Fill to shoulder | Yes |
| # GREEN 250mL disposable PET with green lid | Alkalinity Hardness Ammonium (Ammonia and Ammonium Ions) Chloride Nitrate Nitrite TON Sulphate Silica Phosphorus-SRP Chromium VI | None | Fill to shoulder | Yes |

| Bottle Description | Parameters Analysed | ## Preservation | Filling Instructions | Refrigerate? |
| --- | --- | --- | --- | --- |
| # RED 500mL disposable PET with red lid | Hydrogen Ion (pH) Conductivity Colour Turbidity | None | Fill to top No headspace should be present | Yes |
| # PURPLE 125ml disposable PET with purple lid | Bromate Fluoride Bromide Chlorate Chlorite | None | Fill to shoulder | Yes |

| Bottle Description | Parameters Analysed | ## Preservation | Filling Instructions | Refrigerate? |
| --- | --- | --- | --- | --- |
| # TOC 125ml disposable PET with opaque lid | Total Organic Carbon Dissolved Organic Carbon UV Absorbance % Transmission OES Metals (Process and Effluent Waters) – Ag, Be, Se, Sn, As, Cr, Cd, Ba, Ni, Pb, Mn, Na, Ca, B, K, Mg, P & Total Phosphate | None | Fill to shoulder | Yes |
| # BLACK 125ml disposable PET with black lid | Dissolved Metals Fe, Mn, Al, Cu, Pb, Zn, Cd OES-BSRIA Metals (Process Water) – Molybdate, Mo, Free & Total Fe, Zn, Cu & Al | None | Fill to shoulder | No |

| Bottle Description | Parameters Analysed | ## Preservation | Filling Instructions | Refrigerate? |
| --- | --- | --- | --- | --- |
| # BROWN 250mL brown PET | Total Cyanide | 0.4g Sodium Hydroxide CORROSIVE (Gloves should be worn) Must be used before expiry date shown on bottle | Fill to shoulder Do not rinse Do not overflow | Yes |
| # METAL 125mL polythene | METAL Metals- Fe, Al, Mn, Pb, Cu, Ni, Zn Toxics- Cd, Cr, Ba, P, Ag, Sr, U METAL_2 Hydrides- As, Se, Sb Alk-Metals- Ca, Mg, Na, B,K Li, Be, V, Co, Mo, Sn, Tl | None | Fill to 100ml mark | No |
| # MERCURY 100ml Glass | Hg | None | Fill to shoulder | No |

| Bottle Description | Parameters Analysed | ## Preservation | Filling Instructions | Refrigerate? |
| --- | --- | --- | --- | --- |
| # FIRST-DRAW 1L polythene | First Draw Metals Cu, Pb, Fe, Ni, Zn, Cr | None | Fill to just below shoulder | No |
| # RADIOACT 1L polythene | Gross Alpha Gross Beta | None | Fill to shoulder | No |

| Bottle Description | Parameters Analysed | ## Preservation | Filling Instructions | Refrigerate? |
| --- | --- | --- | --- | --- |
| T-O-GLASS Glass Duran 500ml | Odour – Qualitative Taste – Qualitative Odour – Quantitative Taste – Quantitative | None | Fill to top – no headspace should be present | Yes |
| # PAH 500ml Amber glass | PAH | 0.5mL of 6% Brij / 3% Sodium thiosulphate Must be used before expiry date shown on bottle | Fill to shoulder Do not rinse Do not overflow | Yes |

| Bottle Description | Parameters Analysed | ## Preservation | Filling Instructions | Refrigerate? |
| --- | --- | --- | --- | --- |
| # THM 40ml Amber Vial | THMs, HHC, Benzene, 1,2-Dichloroethane, BTEX, MTBE | 0.1ml of 3% sodium thiosulphate Must be used before expiry date shown on bottle | Fill to top- no headspace should be present Do not rinse Do not overflow Fill 2 bottles | Yes |
| PEST 1L Amber Glass | Pesticides (Azuron, Acid Herbicides, Metaldehyde, PEST-GC-01, PHERBS, Chlorthal, OCP) Hydrocarbons, Geosmin, MIB | 1mL of 3% sodium thiosulphate Must be used before expiry date shown on bottle | Fill to shoulder Do not rinse Do not overflow | Yes |

| Bottle Description | Parameters Analysed | ## Preservation | Filling Instructions | Refrigerate? |
| --- | --- | --- | --- | --- |
| RADON 500ml plastic | Radon | None Analysis MUST be carried out within 48 hours of sample being taken The container should be stored and transported inverted after filling, Sampling of Radon MUST be agreed with the Laboratory prior to sampling due to stability | Bottles should be filled with minimal amount of air present in bottle to prevent loss of gas from bottle from a slow-moving water source. | Yes |
| TRITIUM 125ml disposable PET with blue lid | Tritium | None | Fill to top | Yes |

| Bottle Description |  |  | Parameters Analysed |  | Preservation |  | Filling Instructions |  | Refrigerate? |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PLAS_500ML 500ml clear plastic |  |  | Waste Water Metals- Molybdate, Mo, Free & Total Fe, Zn, Cu & Al As, Cr, Cd, Ba, Ni, Pb, Mn Na, Ca, B, K, Mg, P & Phosphate Ag, Be, Se, Sb, Sn. Hg Free Cr, Free Ni, Free Pb, Free Cd |  | None |  | 250ml Minimum Volume Please subsample if customer does not provide separate bottle |  | No |  |
| STL17 1ltr clear glass |  |  | Tributyl Tin Pesticides & PCBs |  | None |  | Fill to shoulder |  | Yes |  |
| Bottle Description | Parameters Analysed |  | Preservation |  | Filling Instructions |  | Refrigerate? |  |
| STL33 250mL clear glass | TPH Glycol |  | None |  | Fill to shoulder |  | Yes |  |
| STL101 500ml Plastic | PV Colloidal Silica |  | None |  | Fill to shoulder |  | Yes |  |
| STL51 40ml clear glass vial | VOC BTEX VPH |  | Supplied by ALS with 0.5ml Hydrochloric Acid (50% v/v) solution added CORROSIVE (Gloves should be worn) Must be used before expiry date shown on bottle |  | Fill to top- no headspace should be present Do not rinse Do not overflow |  | Yes |  |

| Bottle Description | Parameters Analysed | ## Preservation | Filling Instructions | Refrigerate? |
| --- | --- | --- | --- | --- |
| STL53 40ml amber glass vial | Vinyl Chloride Epichlorohydrin | Supplied by ALS with 0.3ml 1% (w/v) Sodium Thiosulphate solution added Must be used before expiry date shown on bottle | Fill to top- no headspace should be present Do not rinse Do not overflow | Yes |
| STL430 250ml Amber Glass | HAAs | Supplied by ALS with 0.25ml 10% (w/v) Ammonium Chloride solution added Must be used before expiry date shown on bottle | Fill to shoulder Do not rinse Do not overflow | Yes |

| Bottle Description | Parameters Analysed | Preservation | Filling Instructions | Refrigerate? |
| --- | --- | --- | --- | --- |
| STL701 1ltr Plastic | NDMA | Supplied by ALS with 40mg Ascorbic Acid added Must be used before expiry date shown on bottle | Fill to shoulder Do not rinse Do not overflow | No |
| ALS036 White plastic | Glyphosate Chlormequat PFAS Wakefield | Supplied by ALS with 0.125mL of 18mg/L Thiosulphate added Must be used before expiry date shown on bottle | Fill to the top Do not rinse Do not overflow | Yes |

| Bottle Description | Parameters Analysed | Preservation | Filling Instructions | Refrigerate? |
| --- | --- | --- | --- | --- |
| ALS058 500ml Amber Glass | Acrylamide ALS WPC43 Pesticides ALS WPC45 Pesticides ALS WPC46 Pesticides | Supplied by ALS with Sodium Thiosulphate solution added Must be used before expiry date shown on bottle | Fill to shoulder Do not rinse Do not overflow | Yes |
| PFAS 125ml polythene | PFAS 48 | None | Fill to 100 ml mark | Yes (Freeze on arrival at laboratory) |

| Bottle Description | Parameters Analysed | Preservation | Filling Instructions | Refrigerate? |
| --- | --- | --- | --- | --- |
| ALS060 100ml Amber Glass | Phenols | 0.1 ml of 3% (w/v) Ascorbic Acid solution Must be used before expiry date shown on bottle | Fill to the top Do not rinse | Yes |
| STL24 125ml Plastic | PFAS | None | Fill to shoulder 2 full containers | Yes |

| Bottle Description | Parameters Analysed | ## Preservation | Filling Instructions | Refrigerate? |
| --- | --- | --- | --- | --- |
| STL81 1.5kg Round Container | One container required Particle Size Distribution | None | Fill to lid If sample contains large stones, please fill two sample pots | No |
| STL200 250ml Glass with Red Stopper | One container required Redox Potential | None | Carefully fill the bottle with sample with minimum aeration Place the stopper in the bottle to displace air and excess liquid, and then remove the stopper Replace the stopper avoiding inclusion of air bubbles | No |
| STL105 250ml Round Plastic | One container required Iodide | None | Fill to shoulder | Yes |

| Bottle Description | Parameters Analysed | ## Preservation | Filling Instructions | Refrigerate? |
| --- | --- | --- | --- | --- |
| ALE204 | Effluent: Silver Total Silver - Filtered | Bottle contains 0.57ml (40% Nitric Acid) SDS 11 Contains a hazardous Preservative. Shelf life: 18 months | Total Analysis Carefully fill to the neck of the bottle only Filtered Analysis Place the filter on the end of the syringe. Fill the syringe with sample. Carefully insert plunger and apply pressure to filter sample into the bottle, ensuring to only fill to the neck of the bottle. When filtering, rest the syringe and filter disc on the neck of the bottle to aid with filtering, ensuring to hold the bottle. The bottles are date stamped and must be used by the expiry date shown Do not rinse | Yes |
| ALE001 60ml plain DIGI tube | Effluent PFAS | None | Fill to lid | No |