CPCINS3006 Install foamed in-situ insulation products

CPCINS3006 Install foamed in-situ insulation products

# Modification History

| Release | Comments |
| --- | --- |
| 1 | This unit of competency was first released in CPC Construction, Plumbing and Services Training Package Release 11.0. |

# Application

This unit specifies the outcomes required to safely undertake foamed in-situ insulation installation work. The unit includes identifying legislative, regulatory, and job requirements; planning and preparing to install insulation; installing insulation; and, completing installation, including preparation of a Statement of Insulation Installation.

The unit requires a person undertaking this work to comply with legislative, regulatory, safety, technical, organisational and site requirements while planning and carrying out the work.

The unit applies to insulation installers working primarily in residential class 1, 2 or 4 buildings, although there may be application in some instances in commercial buildings.

Persons undertaking this unit should be aware that coverage of this unit extends only to foamed in-situ insulation to roofs, ceilings, walls and subfloors. The work context involves working at heights, dust control and mitigation of risk from asbestos and aerosol particles.

# Licensing/Regulatory Information

At the time of endorsement there was no requirement by any state or territory regulator for insulation installers to complete this unit for licensing or registration purposes.

# Pre-requisite unit(s)

No pre-requisite units

# Competency field(s)

Insulation

# Unit sector(s)

Construction

# Elements and Performance Criteria

| Elements | Performance Criteria |
| --- | --- |
| Elements describe the essential outcomes. | Performance criteria describe the performance needed to demonstrate achievement of the element. |
| 1. Define job requirements. | 1.1 Identify scope of required installation activities through review of job requirements, work instructions and job site specifications, and inspection of work site readiness. 1.2 Assess and control all site risks and hazards and communicate and clarify any issues with supervisor or occupant. 1.3 Identify required safety equipment, processes and safe systems of work by review of work order, risk assessment, Safety Data Sheets (SDS), Safe Use Instruction Sheet (SUIS), and jurisdictional work health and safety (WHS) legislation. 1.4 Identify required Respiratory Protective Equipment (RPE) and safe systems of work according to Australian Standards for the specific Spray Polyurethane Foam (SPF) system. 1.5 Identify and confirm that foamed-in-situ SPF manufacturer requirements are met prior to installation. 1.6 Identify and review workplace environmental requirements that relate to the installation activities and foamed-in-situ SPF manufacturer product guidelines. 1.7 Identify and provide ventilation re-entry and re-occupancy guidance to site supervisor or occupant according to manufacturer product guidelines. 1.8 Identify and review workplace emergency response and evacuation procedures as they apply to the site of the installation activities. 1.9 Identify and review condensation management requirements of the National Construction Code (NCC) and their site-specific climate zone. |
| 2. Plan and prepare for installing insulation. | 2.1 Where electrical circuits and wiring exist, undertake a general safety inspection in accordance with Australian Standards and jurisdictional requirements. 2.2 Confirm site specific requirements for electrical isolation and safety procedures. 2.3 Document and submit for approval a Safe Work Method Statement (SWMS) for the worksite, incorporating unexpected finds protocol, identified risks and hazards, electrical isolation procedure and the hazard hierarchy of control. 2.4 Plan job incorporating identified requirements of work order, work site, safety legislation, environmental requirements, work instructions, job site specifications and SWMS. 2.5 Confirm that foamed in-situ insulating material meets work instructions and thermal and acoustic performance specifications. 2.6 Put in place all requirements of the SWMS including work and spray zones. 2.7 Select Personal Protective Equipment (PPE), RPE, forced ventilation, tools, equipment and associated materials, check for serviceability and report for repair or replacement where unserviceable. |
| 3. Install foamed in-situ insulation. | 3.1 Isolate worksite electrical circuits and install safety signage and lock-out tag where required in accordance with the Australian Standards and SWIMS. 3.2 Isolate site area from other trade works according to manufacturer guidelines and training. 3.3 Clean, prepare and dry site substrate according to foamed in-situ manufacturer work instructions. 3.4 Assess site temperature and humidity conditions according to foamed in-situ manufacturer work instructions. 3.5 Undertake installation work in accordance with the SWMS and foamed in-situ manufacturer work instructions. 3.6 Undertake installation work in accordance with NCC condensation management requirements and their site-specific climate zone. 3.7 Ensure correct placement of vapour barrier membranes and vent cards where required. 3.8 Setup and calibrate insulation plural mix equipment according to product specifications. 3.9 Apply insulation material to specified thickness and density, ensuring uniform coverage. 3.10 Monitor and adjust equipment settings to maintain consistent application according to manufacturer product guidelines. 3.11 Install insulation without damage or distortion of the surrounding environment, electrical and other services and in a manner that maximises safety of self and others. 3.12 Conduct final inspection to ensure installed foamed in-situ insulation conforms to work instructions, thermal and acoustic performance specifications, and manufacturer specifications. 3.13 Complete job sheet outcome including ventilation period prior to reoccupation. |
| 4. Complete installation. | 4.1 Reenergise electrical circuits and remove lock-out tags and safety signage where required in accordance with workplace requirements, Australian Standards and SWMS. 4.2 Clean work area and dispose of materials following workplace and environmental procedures. 4.3 Clean, check, maintain and store tools and equipment in accordance with manufacturer specifications. 4.4 Document details of malfunctions, faults, wear or damage to tools and equipment and report for repair or replacement. 4.5 Document, distribute and store Statement of Insulation Installation and site outcome record, including reoccupation ventilation requirements. 4.6 Inform building occupants of timing for safe reoccupation of building according to manufacturer guidance. |

# Foundation skills

Foundation skills essential to performance are explicit in the performance criteria of this unit of competency.

# Range of conditions

N/A

Assessment Requirements for CPCINS3006 Install foamed in-situ insulation products

# Performance Evidence

A person who demonstrates competency in this unit must:

- Conduct a general safety inspection in accordance with Australian Standards, and luminaire classifications/clearances according to Australian Standards, and jurisdictional requirements.

- Demonstrate an understanding of the key building element assembly outcome needed with selected product to deliver condensation solution with air infiltration elimination.

- Plan and apply foamed in-situ insulation in:

  - at least two different building constructions areas (e.g., roof cavities, walls and subfloors) using appropriate materials such as spray polyurethane foam (SPF).

- Set up and calibrate insulation foam in-situ equipment according to manufacturer specifications and workplace procedures.

- Apply a minimum area of 9 square metres of insulation material to specified thickness and density, ensuring uniform coverage and compliance with thermal and condensation performance specifications.

- Apply manufacturer product guidelines, including:

  - pre-spray checks

  - Daily Work Record (DWR) set points, batch ID’s, lift log, ventilation log

  - field density log

  - field tests per shift and lot change

  - exotherm control

  - ventilation and re-entry.

- Provided a Statement of Insulation installation, with re-entry guidance.

The person must provide evidence from each of the above activities that:

- satisfies the elements and associated performance criteria and foundation skills of this unit; and

- confirms the activities to be in accordance with legislative and workplace requirements specified in Knowledge Evidence.

# Knowledge Evidence

A person demonstrating competency in this unit must provide evidence to demonstrate knowledge and understanding of the following knowledge evidence items. Each item defines the type of knowledge required to meet the demands of one or more performance criteria.

- Jurisdictional Work Health and Safety (WHS) and environmental legislation and regulatory requirements

- Workplace requirements for undertaking all aspects of foamed in-situ insulation installation, including interpreting work orders, work instructions, and job site specifications, and reporting problems relating to foamed in-situ insulation installation and instructions to households on the safe re-occupation of homes.

- Procedures to safely use equipment, shift and handle products and materials, and work at heights to Install foamed in-situ insulation in common situations

- Organisational procedures for responding to site risks and hazards including the presence of pests/vermin, asbestos and dust within an installation workspace

- Risks of asbestos containing materials (ACM), and their use in common building materials used in insulation spaces

- Key requirements of legislation, regulation and building codes related to foamed in-situ insulation

- Manufacturer product guidelines, including:

  - pre-spray checks

  - Daily Work Record (DWR) set points, batch ID’s, lift log, ventilation log

  - field density log

  - field tests per shift and lot change

  - exotherm control

  - ventilation and re-entry.

- Appropriate Personal Protective Equipment (PPE), Respiratory Protective Equipment (RPE) and ventilation, and its use in foamed in-situ insulation installation

- Safety data sheets (SDS) and Safe Work Method Statements (SWMS) commonly used in foamed in-situ insulation installation

- Common health and safety risks associated with handling foamed in-situ insulation products

- Emergency response and evacuation procedures relating to foamed in-situ insulation installation

- Work instructions and specifications relating to foamed in-situ insulation installation

- Electrical risk assessment process, hazard identification and reporting according to Australian Standards for common foamed in-situ insulation installation

- Hierarchy of hazard control as it relates to foamed in-situ insulation installation

- Organisational requirements and procedures relating to foamed in-situ insulation installation, including requirements for a systematic approach to planning own work

- Key requirements of Australian Standards relating to foamed in-situ insulation, including:

  - confirming material label states that it complies with   Australian Standards; and

  - Australian Standards wiring rules.

- Requirements relating to foamed in-situ insulation installation from product manufacturer training for:

  - personal safety;

  - electrical risk assessment;

  - approved processes; and

  - energy efficiency – new products and technologies.

- Requirements of the National Construction Code (NCC) relating to fire property testing and Australian Standards relating to smoke and group numbers

- Types of common foamed in-situ insulation materials, and their properties:

  - open cell SPF

  - closed cell SPF

- Manufacturer specifications of common foamed in-situ installation materials, including R rating and thickness/depth.

- Types, safety, characteristics, uses and limitations of common foamed in-situ insulation installation tools and equipment

- Type and purpose of lock out tags in foamed in-situ insulation installation

- Requirements of Australian Standards for luminaires, in foamed in-situ insulation installation including:

  - operating temperature limit of electrical cables;

  - effect on cables partially surrounded by thermal insulation and fully surrounded by thermal insulation;

  - common wiring systems used in domestic premises indicating the age of the installation; and

  - electrical hazards in foamed in-situ spaces, including unenclosed connections, unenclosed conductors, damaged cable sheaths and exposed conductors.

- Product and process knowledge to identify common problems and predict consequences in foamed in-situ insulation installation

- Quality of installation of foamed in-situ insulation material to achieve thermal and acoustic performance

- Energy efficiency of common foamed in-situ insulation material types

- Methods for measuring, cutting and installing foamed in-situ insulation material

- Procedures for conducting a final inspection of a foamed in-situ insulation installation

- Workplace procedures for environmental requirements for waste, including waste management and recycling relating to foamed in-situ insulation installation

- Procedures for cleaning, checking and maintaining tools and equipment used for foamed in-situ insulation installation

- Procedures for dealing with faulty foamed in-situ insulation installation tools and equipment

- Procedures for documenting, distributing and storing Statements of Insulation Installation.

# Assessment Conditions

Assessors must satisfy the requirements for assessors contained in the Standards for Registered Training Organisations.

Assessment of performance must be undertaken in the workplace and/or under realistic workplace conditions which typically reflect:

- the use of industry standard equipment;

- performing tasks/activities to the level of proficiency and within timelines that would be expected in a workplace; and

- following standard and authorised work practices, safety requirements and environmental constraints.

Assessment must include work in a space with obstructions including framing and services, including; electrical, plumbing and air conditioning.

Assessors are responsible for ensuring that candidates have access to specifications for:

- foamed in-situ insulation installation tasks for at least two different building constructions areas (e.g., roof cavities, walls and subfloors) using appropriate materials such as spray polyurethane foam (SPF).

Assessors are responsible for ensuring that candidates have access to:

- wall frames;

- ceiling spaces;

- sub-floors;

- insulation foamed in-situ equipment and associated materials and tools;

- relevant sections of the Australian Standards/National Construction Code;

- relevant sections of WHS/OHS legislation;

- electrical risk assessment report;

- SWMS template;

- Statement of Insulation Installation template;

- PPE, RPE and ventilation; and

- technical documentation.

# Mandatory Workplace Requirements

Assessment of performance evidence may be in a workplace setting or an environment that accurately represents a real workplace.

# Unit Mapping Information

| Previous Code and Title | Equivalence | Comments |
| --- | --- | --- |
|  | Newly created | This unit has been created to address a skill or task required by industry that is not covered by an existing unit of competency |

# Links

Companion volumes, including implementation guides, are found on the national training register - https://training.gov.au/training/details/CPC.