CPPSPS3014 Maintain swimming pool and spa water circulation and filtration systems

CPPSPS3014 Maintain swimming pool and spa water circulation and filtration systems

# Modification History

| Release | Comments |
| --- | --- |
| 1 | This unit of competency was first released in CPP Property Services Training Package Release 19.0. |

# Application

This unit of competency specifies the skills and knowledge required to carry out routine maintenance and installation tasks for swimming pool and spa water circulation and filtration systems.

It includes:

- preparing for and carrying out routine maintenance and installation tasks for swimming pool and spa circulation and filtration systems

- assessing hazards and determining appropriate risk controls

- completing routine maintenance, installation, and repair activities within scope of own authority, including work completion records.

This unit applies to swimming pool and spa technicians, and related industry professionals and specialists, with limited supervisory responsibilities, who perform work alone or as members of a team.

Work is undertaken in accordance with worksite procedures and manufacturer instructions, and within the limits of the individual’s role and authority.

Technicians perform tasks permitted under relevant state or territory legislation and are required to coordinate with or defer to licensed gas, electrical and plumbing trades for restricted work.

# Licensing/Regulatory Information

Technicians perform tasks permitted under relevant state or territory legislation and are required to coordinate with or defer to licensed gas, electrical and plumbing trades for restricted work.

# Pre-requisite unit(s)

Nil

# Competency field(s)

N/A

# Unit sector(s)

Swimming pool and spa servicing

# Elements and Performance Criteria

| Elements describe the essential outcomes. | Performance criteria describes the performance needed to demonstrate achievement of the element. Required knowledge, skills and application should be considered and clearly articulated. |
| --- | --- |
| 1. Prepare for routine maintenance and installation of swimming pool and spa water circulation and filtration systems | 1.1 Confirm routine maintenance and installation task requirements with relevant personnel and/or customers in accordance with Work Health and Safety (WHS) requirements, manufacturer specifications and worksite instructions. 1.2 Obtain relevant work instructions, manufacturer specifications and task documentation in accordance with worksite instructions. 1.3 Review relevant Australian Standards and legislation in accordance with WHS and worksite instructions. 1.4 Implement risk control measures specified in relevant safety documentation in accordance with WHS and worksite instructions. 1.5 Select tools, equipment and materials required to complete tasks efficiently in accordance with WHS and worksite instructions. 1.6 Check serviceability of tools and equipment and rectify or report faults in accordance with WHS requirements, manufacturer specifications and worksite instructions. 1.7 Confirm licensed electrical, plumbing or gas trades for any work outside scope of own authority, in accordance with worksite and regulatory requirements. |
| 2. Work safely and sustainably | 2.1 Apply relevant procedures when operating, installing, commissioning and maintaining pool and spa systems in accordance with WHS, manufacturer specifications and worksite instructions. 2.2 Use personal protective equipment (PPE) appropriate to the task after checking its serviceability in accordance with WHS and worksite instructions. 2.3 Follow environmental procedures for the safe handling of chemicals, filters and wastewater in accordance with WHS requirements, manufacturer specifications and worksite instructions. 2.4 Minimise environmental impact during work activities in accordance with worksite instructions. |
| 3. Carry out routine maintenance and installation | 3.1 Inspect and assess swimming pool and spa water circulation and filtration system/s to identify deviations from normal operating criteria and safety hazards, in accordance with manufacturer specifications and workplace instructions. 3.2 Carry out routine maintenance and repairs, including replacement of worn or defective components, within scope of own authority, according to manufacturer specifications and worksite instructions. 3.3 Carry out installation tasks within scope of own authority including like-for-like component changeovers, according to manufacturer requirements and worksite instructions. 3.4 Refer and coordinate repair, installation or maintenance tasks outside of scope of own authority with licensed trades where required, in accordance with worksite and regulatory requirements. 3.5 Verify system operation following maintenance, repair or installation tasks to confirm it functions according to manufacturer specifications and worksite instructions. |
| 4. Finalise work and hand over circulation and filtration system | 4.1 Reinstate the work area and dispose of waste and contaminated materials in accordance with environmental and WHS requirements. 4.2 Ensure systems are returned to safe operational condition and notify relevant personnel of completion in accordance with worksite instructions. 4.3 Ensure all chemical containers are safely secured, labelled, and stored in accordance with legislative requirements, WHS requirements and worksite and/or manufacturer instructions. 4.4 Provide client or responsible person with instructions for safe operation and routine maintenance in accordance with WHS requirements and worksite and/or manufacturer instructions. 4.5 Record installation and maintenance work activities in the digital or manual field service documentation system in accordance with WHS requirements and worksite and/or manufacturer instructions. |

# Foundation skills

This section describes the language, literacy, numeracy and employment skills essential to performance in this unit but not explicit in the performance criteria:

- reading skills to:

  - interpret task documentation

  - understand written instructions, procedures and signs

  - access information in safety data sheets (SDSs)

  - interpret manufacturers’ instructions and workplace requirements

- writing skills to report faults and record work completion information

- numeracy skills to interpret numerical information relating to routine swimming pool and spa water circulation and filtration systems maintenance

- oral communication skills to:

  - use questioning to identify and confirm requirements

  - provide client with information

  - report work site hazards.

# Range of conditions

N/A

Assessment Requirements for CPPSPS3014 Maintain swimming pool and spa water circulation and filtration systems

# Performance Evidence

To demonstrate competency, a candidate must meet the performance criteria for this unit by carrying out routine maintenance and installation tasks limited to like-for-like component changeovers for circulation and filtration systems for at least two of the following:

- swimming pools

- spas

- aquatic environments.

and:

- at least one installation task involving a like-for-like component changeover, and

- routine maintenance tasks on existing circulation and filtration systems, including:

  - cleaning strainers

  - cleaning filters

  - inspecting visible pipework and seals

  - replacing worn components

While completing the above work, the candidate must also:

- interpret work instructions, specifications, and manufacturer requirements for circulation and filtration systems

- identify and comply with relevant Australian Standards, regulatory and environmental requirements

- plan and prepare work, including confirming access, tools, materials, and coordination with licensed trades where required

- inspect and assess system condition to identify deviations from normal operating criteria

- carry out routine maintenance activities, including cleaning strainers, cleaning filters, inspecting visible pipework and seals, and replacing worn components

- carry out like-for-like component changeovers within scope of own authority

- verify system operation following maintenance or installation tasks

- reinstate the work area and ensure systems are left in a safe condition

- provide relevant information to the client or responsible person regarding system operation and routine maintenance

- complete required documentation.

# Knowledge Evidence

To be competent in this unit, a candidate must demonstrate knowledge of:

- relevant provisions of work health and safety (WHS), environmental and dangerous goods legislation

- hydraulic and electrical safety requirements as they relate to swimming pool and spa circulation and filtration systems

- Australian Standards applicable to installation and maintenance of swimming pool and spa circulation and filtration systems

- environmental management principles, including waste disposal, chemical handling and water discharge requirements

- entrapment-prevention requirements for swimming pool and spa cleaning and vacuuming systems, including common entrapment hazards, required controls and relevant Australian Standards, codes of practice or regulatory requirements

- circulation and filtration system principles, including:

  - purpose and basic operating principles of circulation and filtration systems

  - main components and configuration of suction and return lines, pumps, filters, valves and control devices

  - types of filters, filter media and routine changeover procedures

  - typical system faults and symptoms, such as loss of suction, pressure fluctuations, poor filtration and leaks

- installation and maintenance practices, including:

  - interpreting manufacturer specifications and work instructions relevant to installation and maintenance tasks

  - inspection and assessment procedures to identify deviations from normal operating criteria

  - routine maintenance and servicing procedures for filters, strainers, valves and associated components

  - replacement of worn or defective components

  - verification of basic system operation following maintenance or installation tasks

- worksite communication and documentation, including:

  - coordination requirements with licensed trades where electrical or plumbing work is required

  - procedures for reporting hazards, faults and non-compliance issues

  - documentation requirements for installation, maintenance and service records

  - client communication relating to system operation, routine maintenance and safety information.

# Assessment Conditions

Assessment must include access to:

- manufacturer specifications and installation diagrams

- relevant Australian Standards and regulatory documentation

- tools, materials, components, and PPE appropriate to installation and maintenance tasks

- digital or manual field service documentation system and procedures used for installation, testing, commissioning, and maintenance records

# Mandatory Workplace Requirements

Assessment of performance evidence may be in a workplace setting or an environment that accurately represents a real workplace.

# Unit Mapping Information

| Previous Code and Title | Equivalence | Comments |
| --- | --- | --- |
| CPPSPS3004 Maintain swimming pool and spa water circulation and filtration systems | Not equivalent | Changes have been made towards the application, elements, performance criteria, performance evidence and assessment conditions. |

# Links

Companion volumes, including implementation guides, are found on the national training register - https://training.gov.au/training/details/CPP.

CPP Property Services Training Package:  https://training.gov.au/training/details/CPP/summary