1.RS-232 Command:

Baudrate：19200

Data width：8bit

Parity: none

Stop: 1bit

Checksum =(0x100-(CMD1+CMD2+.....CMD13))&0xFF

Port switch command package length is 13byte:

[0x5b+0xa5+0x02+0x03+input port(1~4)+0x00+output port(1~4)+0x00+0x00+0x00+0x00+0x00+checksum]

All you need to change is just “input port”, ”output port”, ”checksum”

Checksum = 0x100 – (0x5b+0xa5+0x02+0x03+input port+0x00+output port+0x00+0x00+0x00+0x00+0x00)

For example: Set output 1 from input 2 command:

TX: 5B A5 02 03 02 00 01 00 00 00 00 00 F8

RX: A5 5B 02 03 00 00 01 00 00 00 00 00 FA

Port switch query package length is 13byte:

This is a query command which mean you must send query package and then receive an answer.

For example: Query output A input port (1~4)

TX: 5B A5 02 01 01 00 00 00 00 00 00 00 FC

RX: A5 5B 02 01 01 00 01 00 00 00 00 00 FB

The red 01 mean the output port number, it should be 1~4.

The blue 01 mean the input port number, it should be 1~4.

Edid set command package length is 13byte:

[0x5b+0xa5+0x03+0x02+Edid index(1~15)+0x00+input port(1~4)+0x00+0x00+0x00+0x00+0x00+checksum]

[0x5b+0xa5+0x03+0x01+Edid index(1~15)+0x00+0x00+0x00+0x00+0x00+0x00+0x00+checksum]

Means: set edid mode to all input port

For example: set SE_1080P_20 mode to all input port

TX: 5B A5 03 01 04 00 00 00 00 00 00 00 F8

RX: A5 5B 03 01 00 00 00 00 00 00 00 00 FC

Edid index list:

SE_1080I_20 		= 1

SE_1080I_51 		= 2

SE_1080I_71 		= 3

SE_1080P_20 		= 4

SE_1080P_51 		= 5

SE_1080P_71 		= 6

SE_3D_20 			= 7

SE_3D_51 			= 8

SE_3D_71 			= 9

SE_4K2K_20 		= 10

SE_4K2K_51 		= 11

SE_4K2K_71 		= 12

SE_DVI_1024_768 	= 13

SE_DVI_1920_1080 	= 14

SE_DVI_1920_1200 	= 15

Edid copy command package length is 13byte:

[0x5b+0xa5+0x03+0x04+output port (1~4)+0x00+input port(1~4)+0x00+0x00+0x00+0x00+0x00+checksum]

Means: copy output port X edid to input port X

[0x5b+0xa5+0x03+0x03+output port (1~4)+0x00+0x00+0x00+0x00+0x00+0x00+0x00+checksum]

Means: copy output port X edid to all input port

For example: copy output port 1 edid to input port 1

TX: 5B A5 03 04 01 00 01 00 00 00 00 00 F7

RX: A5 5B 03 04 00 00 01 00 00 00 00 00 F8

Output HDP status query package is 13byte:

This is a query command which mean you must send query package and then receive an answer.

[0x5b+0xa5+0x01+0x05+output port (1~4)+0x00+0x00+0x00+0x00+0x00+0x00+0x00+checksum]

Query HPD status of the output X.

For example: Query output 1(1~4) HPD status

TX:	5B A5 01 05 01 00 00 00 00 00 00 00 F9

RX: A5 5B 01 05 01 00 FF 00 00 00 00 00 FA

The red 01 mean the output port number, it should be 1~4.

The blue FF mean this port’s HPD is LOW, if 00 mean HIGH.

Input port status query package is 13byte:

This is a query command which mean you must send query package and then receive an answer.

[0x5b+0xa5+0x01+0x04+input port (1~4)+0x00+0x00+0x00+0x00+0x00+0x00+0x00+checksum]

For example: Query input 1(1~4) status

TX: 5B A5 01 04 01 00 00 00 00 00 00 00 FA

RX:  A5 5B 01 04 01 00 FF 00 00 00 00 00 FB

The red 01 mean the input port number, it should be 1~4.

The blue FF mean this port is plug out, if 00 mean plug in.

Beep on/off command package length is 13byte:

[0x5b+0xa5+0x06+0x01+Beep onoff(0x0f:ON; 0xf0:OFF)+0x00+0x00+0x00+0x00+0x00+0x00+0x00+checksum]

For example:

Set beep on:

TX:5B A5 06 01 0F 00 00 00 00 00 00 00 EA

RX:A5 5B 06 01 00 00 00 00 00 00 00 00 F9

Beep on/off query package is 13byte:

This is a query command which mean you must send query package and then receive an answer.

For example:

TX: 5B A5 01 0B 00 00 00 00 00 00 00 00 F4

RX: A5 5B 01 0B 00 00 FF 00 00 00 00 00 F5

The blue FF mean Beep off, if 00 mean Beep on.

IR command：

NEC code

#define SYSTEM_CODE 	0x00

#define IR_KEY_POWER               0x14

#define IR_KEY_OUTPUT_1_FROM_1     0x09

#define IR_KEY_OUTPUT_1_FROM_2     0x1D

#define IR_KEY_OUTPUT_1_FROM_3     0x1F

#define IR_KEY_OUTPUT_1_FROM_4     0x0D

#define IR_KEY_OUTPUT_1_PRE        0x1B

#define IR_KEY_OUTPUT_1_NEXT       0x11

#define IR_KEY_OUTPUT_2_FROM_1     0x17

#define IR_KEY_OUTPUT_2_FROM_2     0x12

#define IR_KEY_OUTPUT_2_FROM_3     0x59

#define IR_KEY_OUTPUT_2_FROM_4     0x08

#define IR_KEY_OUTPUT_2_PRE        0x55

#define IR_KEY_OUTPUT_2_NEXT       0x48

#define IR_KEY_OUTPUT_3_FROM_1     0x5e

#define IR_KEY_OUTPUT_3_FROM_2     0x06

#define IR_KEY_OUTPUT_3_FROM_3     0x05

#define IR_KEY_OUTPUT_3_FROM_4     0x03

#define IR_KEY_OUTPUT_3_PRE        0x07

#define IR_KEY_OUTPUT_3_NEXT       0x40

#define IR_KEY_OUTPUT_4_FROM_1     0x18

#define IR_KEY_OUTPUT_4_FROM_2     0x44

#define IR_KEY_OUTPUT_4_FROM_3     0x0f

#define IR_KEY_OUTPUT_4_FROM_4     0x51

#define IR_KEY_OUTPUT_4_PRE        0x1E

#define IR_KEY_OUTPUT_4_NEXT       0x0E