# Overview:

This document describes the procedure to use Windows PC to connect to JTECH-VW-4KPT web UI. The steps are:

- Copy ‘node_list for Windows’ from disk

- Setup PC’s IP address to the same IP domain as JTECH-VW-4KPT devices.

- Use node_list to find the JTECH-VW-4KPT device in interest.

- Connect to the Web UI.

# Procedure:

Step 1. copy ‘node_list for Windows’ from disk

User can use this tool to list all available JTECH-VW-4KPT devices in the same IP domain. You can download the windows program from:

- Execute file:

It is a Windows console program. There is no installation procedure needed. Just un-zip the downloaded files.

Step 2. Setup PC’s IP address

J devices use 169.254.xxx.xxx private IP domain by default. In order to connect to JTECH-VW-4KPT, the Windows PC must use the same IP domain as JTECH-VW-4KPT. Here is the example steps of setup static IP address to 169.254.111.111 in Windows XP.

- Open Windows’s “Network Connections” control panel by press “Win+R” then type “control netconnections” and Enter.

- Select the network interface that will be used for network-access (typically Local Area Connection). Right-Click shows up the context-menu and select properties.

- Setup static IP to 169.254.111.111 as following steps:

Step 3. Run node_list

After un-zip the node_list file. Just run ‘node_list.bat’. Please note:

- Disable or Configure Windows firewall to accept node_list.exe.

- The PC’s IP domain must be the same as JTECH-VW-4KPT’s IP domain.

Following is a sample result where there are two JTECH-VW-4KPT devices. One is using IP 169.254.10.146. The other is using 169.254.196.156.

Step 4. Connect to the web UI using the found IP address

Open Web browser of your choice and enter the IP address of the JTECH-VW-4KPT device you want to connect to. If you can’t find any JTECH-VW-4KPT devices, make sure these JTECH-VW-4KPT devices are connected to the same LAN as your PC. And make sure your PC’s IP address is in the same IP domain as JTECH-VW-4KPT devices (See Step 2).