INTERNATIONAL ORGANISATION FOR STANDARDISATION

ORGANISATION INTERNATIONALE DE NORMALISATION

ISO/IEC JTC1/SC29/WG11

CODING OF MOVING PICTURES AND AUDIO

ISO/IEC JTC1/SC29/WG11 MPEG/m54732

July 2020, on-line

| Source | Interdigital |
| --- | --- |
| Status | proposal |
| Title | Fan content proposal for MIV (update) |
| Author | Renaud Doré, Gérard Briand, Julien Fleureau |

# Context

MPEG Video group does not have a CGI content with fine geometry and it misses in our current portfolio.

In presence of very small geometry as seen from the camera rig, some pixels blend the results of several depth level, while the depth is one of them only. Therefore it raises specific issues which need to be addressed in MIV investigation.

## Fan content

We propose the content named Fan. The scene is indoor and is shot with perspective cameras. A fan on the right starts slowly to rotate and produces wind blow which eventually moves a little bit different dynamic fine elements like plants, posters, pennants, .. Other static elements have been chosen with untrivial geometry.

The content on 97 frames, 3 pose files (Fp01.csv, Fp02.csv, Fp03.csv), the 3 related R97 pose traces, this contribution, the excel file and license are to be found in:

- MPEG server MPEG-I/Interdigital/Interdigital_Fan_Content/

We made such that there is very little specularity here and very little transparencies, so that investigation with that content can focus on fine geometry (with a easy to remember content name: Fan => Fine geometry)

Fig 1: 3D view of InterdigitalFan0 with the 5x3 rig

Here is one view example where the following can be shown:

- Static fine geometry: rodchairs, chairs,  bonzaï (on the very bottom left)

- Dynamic fine geometry (rotating fan with rotating blades, birdcage, plants in the background, ..)

- Other dynamic parts like posters, pennants moving with the air flow

The v2 updates from content shown at last Ad Hoc meeting are the following:

- Crescendo in the air flow intensity

- Some fixes on mesh artefacts

- Slight texture improvement

- Slight motion blur, becoming apparent in the blade and pennant objects at the sequence end

- Rig of 5x3 spaced by 10cm horizontally and 10cm vertically, leading to a visualization box of 40cm x 20 cm

Note that no depth of field has been introduced on purpose: although it would be more photo-realistic, it would have prevented the investigations to be carried on the out-of-focus zones.

Figure 1: top right view v00

Figure 2: bottom left view v14

# Trial on TMIV5.0

In the first version of this contribution, a simple trial with TMIV5.1 was made without specific tweaking with the following parameters of the TMIV encoder configuration file :

…

"geometryScaleEnabledFlag": true,

"intraPeriod": 32,

"maxAtlases": 2,

"maxEntities": 1,

"maxLumaSampleRate": 1069547520.0,

"maxLumaPictureSize": 8912896,

"numGroups": 1,

…

In this update, the depth downscaling has been devalidated

"geometryScaleEnabledFlag": false,

## Objective results

There is no Anchor to compare with, but the objective figures are interesting per se. A template has been provided with Fan content taking the place of SJ, and putting downscaled geometry on the left and unscaled geometry on the right.

The objective results are low, way below 30dB. Unscaling the geometry degrades slightly the performance by 19.1% and 25.1% high and low BD rate respectively.

Figure 3: objective figures with downscale geometry in red, and unscaled geometry in green

## Subjective results

We have made available on the mpeg content server the following

- The 3 pose traces with R97

- P01 pose traces @ QP1 & QP3 and A97 scaled geometry

- P01 pose traces @ QP1 & QP3 and A97 with geometry unscaled.

- Side by side video on the

Figure 4: A97 scaled (left) versus A97 unscaled (right) @QP1

As expected:

- R97 exhibits videos of correct quality, proving that the 5 by 3 views sampling of this scene is adapted

- There are severe artefacts especially in the fine geometry of the fan. A97 is not good at QP1 and horrible at QP3 on those particular fine parts.

- Removing depth downscaling help a little, but this is not sufficient.

# Conclusion

Adding this content as optional content, since it focuses on fine geometry, something present in real scene. We think that this scene can correspond to a real life situation and is far to be extravagant.

We also encourage proponents to generate depth on the textures alone independently of the provided depth for the reasons explained in the other contrib m54420 [1] “Consideration on MIV v1 versus beyond” presented in MPEG131).

# Annex: Copyright Group & Fan contents

WARNING: DO NOT REMOVE THIS NOTICE - AUTHORIZATION SUBJECT TO RESTRICTIONS

This copyright document relates to the following image contents:

All files included in the directory Interdigital_Group[X] (with X as index or suffix or absent)

All files included  in the directory Interdigital_Fan[X] (with X as index or suffix or absent)

with the names nnnn_####.yuv or nnnn_#### .png files for the color,

with the names mmmm_####. depth for the depth

with #### indicating any frame number

This video content (“Content”) may only be used for the purpose of developing, testing and promulgating technology standards developed by the MPEG, JPEG, VCEG, JVET standardization groups and for academic research without any commercial use or intent (a commercial use including without limitation the creation of any intellectual property right)(“Purpose”), under InterDigital R&D France (“InterDigital”) owned or controlled copyrights, by individual(s) or organization(s) that participates, contributes or is part of such standardization groups (“User(s)”).

RESTRICTIONS: No license whatsoever is implied except from this expressed limited personal, non-sublicensable, non-transferrable authorization. In particular no right under any patent of InterDigital or its affiliates is granted. Any other use is strictly prohibited. This Content cannot be distributed or otherwise disposed of or made available (via internet or otherwise), broadcasted, , used for providing any services to third parties or for any commercial use, nor copied (except as technically necessary for the Purpose), modified, adapted, translated, exchanged, integrated, without limitation. This Content and all intellectual property rights, titles and interests therein, are and remain the exclusive property of InterDigital or its affiliates.

The Content may be disclosed to third Parties or publicly performed for the Purpose only. In no circumstances the Content may be delivered to any third Party. In any demonstration of the content, the following publication must be mentioned:

Renaud Doré, Gérard Briand, Franck Thudor, Interdigital, Group content for MIV, ISO/IEC JTC1/SC29/WG11 MPEG131/M54731, June 2020

or

Renaud Doré, Gérard Briand, Franck Thudor, Interdigital, Fan content for MIV, ISO/IEC JTC1/SC29/WG11 MPEG131/M54732, June 2020

NO WARRANTY: This Content is supplied "as is" and without any warranty of any kind, expressed or implied, including but not limited to any warranty for accuracy or performance, fitness for a general or particular purpose, or any warranty arising by statute or by law, or non-infringement of any third parties’ rights. In no event will InterDigital be liable for damages of any kind, including without limitation, any special, indirect, incidental, or consequential damages even if InterDigital has been advised of the possibility of such damages. This authorization is effective as of the date of use of the Content. InterDigital may nevertheless terminate it at any time for any reason upon notice to User(s). It is governed by the laws of France without regard to conflict of laws. It shall be exclusively submitted to the first instance civil court of Paris, France in the event of a dispute not settled amicably.

BY USING THIS CONTENT USER(S) AGREES AND ACCEPTS THE ABOVE MENTIONNED TERMS AND CONDITIONS. IF USER(S) DOES NOT AGREE TO THESE TERMS AND CONDITIONS, IT SHALL NOT ACCESS OR OTHERWISE USE THIS CONTENT. DO NOT REMOVE OR MODIFY INTERDIGITAL OR ITS AFFILIATES’ NAMES, SIGNS, LOGOS, NOTICES OF/IN THE CONTENT.

Any use (including citation) of the Content must explicitly contain the following sentence "copyright: InterDigital. All rights reserved"

Copyright © 2017-2018 – InterDigital

All Rights Reserved “