INTERNATIONAL ORGANISATION FOR STANDARDISATION

ORGANISATION INTERNATIONALE DE NORMALISATION

ISO/IEC JTC1/SC29/WG11

CODING OF MOVING PICTURES AND AUDIO

ISO/IEC JTC1/SC29/WG11 MPEG2019/M52413

January 2020, Brussels, Belgium

Source Interdigital

Status Input contribution

Title CE2.6.1: Synthesizer

Author Julien Fleureau, Renaud Doré, Franck Thudor, Thierry Tapie, Gérard Briand, Bertrand Chupeau

# Introduction

This contribution presents a description of Interdigital’s experiment on Immersive Video CE2 (CE2.6 Investigation on graph-based pruning method) [1]. The sentence below is recopied from [1] as a remainder of the objective of this experiment:

“[…] This CE will investigate another method for efficient pruning. For that experiment, it will be checked the correlation with the synthesizer used at rendering stage. Hence experiments will be conducted considering the coupling that may exist between both modules. […]”

Experiment depicted in the current document CE2.6.1 is about an enhanced synthesizer at decoder side, that improves both objective and subjective metrics and makes it also possible the use of graph-based pruning related metadata generated at encoder side. Investigation on graph-based pruning presented in document CE2.6.2 [2] uses this synthesizer.

Experiment is conducted following the CTC rules [3].

# Synthesizer presentation

Synthesizer at decoder side (cf. Figure 1) has been modified to improve quality of the rendered views. With respect to the TMIV3.0 synthesizer, the main difference is the use of splat rendering per pixel rather than using a triangulation method.

The synthesis still works frame by frame.

Experiment is done with C++ code integrated directly in TMIV3.0 implementation, with GCC 9.1.0 as compiler.

Figure 1 : decoder within TMIV3.0.

An atlas deconstructor has been embedded inside the synthesizer (cf. Figure 2), in order to recover the pruned views. The renderFrame procedure of the synthesizer works on these recovered pruned views. This procedure can be decomposed into three main stages:

- Per-camera global value initialization

- Visibility map computation

- Shading map generation

Each stage will be explained below.

Figure 2 : schematic of the synthesizer at decoder side.

- Per-camera global value initialization

In this first stage, metadata information dealing with the rig of source cameras and the target viewport is used to compute two vectors named camera weight and camera visibility.

Camera weight: for each source camera, a weight is computed as a function of the distance between the camera position and the target viewport.

Camera visibility: for each camera, a boolean is computed specifying if the camera is/is not visible by the target viewport. It allows to discard some cameras that are not seen hence decreasing the processing time.

- Visibility map computation

The aim of this second stage is to generate a depth map for the target viewport, based on the recovered pruned views. This stage is decomposed into three steps: warping, selection and filtering.

Warping: a warped depth map is generated for each camera, from the pruned depth map. During this step, a splat is generated for each pixel, which is then rasterized. Splat dimensions are based on the neighborhood of the pixel. Angular distortion of neighbors reprojections drives the splat size, parameterized by the angularScaling parameter. Degeneracy of splats is limited by the stretchFactor parameter.

At the end of this step, one warped depth map is generated per source camera.

|  | … |  | … |  |
| --- | --- | --- | --- | --- |

Figure 3: example of pruned depth maps (v0, v18, v23); only pixels that re-project inside the viewport camera are shown here.

|  | … |  | … |  |
| --- | --- | --- | --- | --- |

Figure 4: example of warped depth maps (v0, v18, v23).

Selection: from the warped depth maps, a single depth map is generated. The selection process is based on a majority voting process, pixel-wise. Depth candidate values are sorted by increasing order. These values are walked through. The minimal depth value is selected, except if a further depth presents a high enough weight. This selection is controlled by the overloadFactor parameter. Weight is computed during the Per-camera global value initialization step. This procedure is particularly efficient for natural content sequences where depth is not that coherent between cameras.

At the end of this step, one selected depth map is generated.

Figure 5: example of selected depth map

Filtering: a median filtering is finally applied to filter out the depth. This step is driven by the filteringPass parameter.

- Shading map generation

Finally, using the filtered depth map and the recovered pruned views, the color texture of the viewport is generated. To improve visual quality of the rendered texture, a bilinear filtering is applied. Moreover, pixels coming from contours of an object in a source camera are detected and discarded during the shading step, to avoid aliasing.

# Json parameters

The parameters needed for this synthesizer are given in the example below. The values are the same whatever the content (CG-A, CG-B, CG-C, CG-J, CG-L, NC-D, NC-E, NC-L). The .json TMIV parameters files are  generated by running the Python script “src\ctc_config\make_tmiv_idcc_01_configs.py”.

"Decoder": { "AtlasDeconstructor": {}, "AtlasDeconstructorMethod": "AtlasDeconstructor", "GroupBasedRenderer": { "IDCCSynthesizer": { "angularScaling": 1.5, "blendingFactor": 0.03, "filteringPass": 1, "overloadFactor": 2.0, "stretchFactor": 100.0 }, "Inpainter": {}, "InpainterMethod": "Inpainter", "SynthesizerMethod": "IDCCSynthesizer" }, "RendererMethod": "GroupBasedRenderer" }, "DecoderMethod": "Decoder",

Table 1: extract from TMIV_A17_SA.json

Parameters needed for the synthesizer are presented below:

- angularScaling (float): this parameter is used to compute the angular distortion per source, used at the warping step.

- blendingFactor (float): this parameter is used to test the proximity of a point in space at a depth “z” with respect to a reference point at a depth zref with the following test:

zref-z<z*blendingFactor

This test is performed each time the neighborhood of a pixel is to be considered.

- filteringPass (int): this parameter specifies the number of passes of visibility filtering.

- overloadFactor (float): this parameter is used for the best candidate selection within the selection step.

- stretchFactor (float): this parameter is used in the rasterization process of the splat within the warping step, to limit the degeneracy of a splat.

# Results

Tests done are summarized in the two templates “reporting_template_ff_Anchors3.0_vs_IDCCSynth.xlsm” and “reporting_template_rf_Anchors3.0_vs_IDCCSynth.xlsm”, respectively in the full number of frames (i.e. 97 frames) and in the reduced number of frames (i.e. 17 frames) anchors configuration. All test sequences of the template were tested during the validation. The encoded bitstreams used for the anchors have been used for this validation.

For “ff” configuration, a snapshot of the summary is presented in Figure 6 and the BD-curves are presented in Figure 7. Please refer to the attached templates for more information.

As presented in Figure 6, synthesizer developed at Interdigital performs well with respect to Y-PSNR metrics, both for natural content (NC) and computer-generated content (CG). Some rendering improvements can be seen in Figure 8 for SA and in Figure 9 for SJ, showing the benefit of Interdigital’s synthesizer (cf. red circles). But improvement in objective results are not always linked to improvement in subjective results, as shown in Figure 10 where holes are appearing in Kermit’s back, while the objective figure is better. All results cannot be shown in current document; hence we strongly recommend checking the side-by-side pose traces during a viewing session.

Results on content SN are not that good, and visualization of pose traces leads to the same conclusion. It seems that the source depth maps are not using the right dynamic of value to efficiently encodes the scene, leading to small difference in quantized normalized disparity for very distant values in depth. Please refer to [4] for deeper investigation.

Figure 6: snapshot of reporting_template_ff with respect to TMIV3.0 anchors.

Figure 7: BD-curves from reporting_template_ff with respect to TMIV3.0 anchors.

Figure 8: example of rendered frame from A97_SA_QP1_Tt_p01_1920x1080_yuv420p10le: TMIV3.0 anchors (top) versus ours (bottom).

Figure 9: example of rendered frame from A97_SJ_QP1_Tt_p01_1920x1080_yuv420p10le: TMIV3.0 anchors (top) versus ours (bottom).

Figure 10: example of rendered frame from A97_SE_QP1_Tt_p01_1920x1080_yuv420p10le: TMIV3.0 anchors (top) versus ours (bottom).

Now, considering the full frames R97 anchors, Interdigital’s synthesize behavior is slightly better than TMIV3.0, as can be seen in images below for SA content and for SB content. In ClassroomVideo sequence, contours are sharper (cf. Figure 11). In TechnicolorMuseum sequence, some holes have been filled and traces near contours have disappeared (cf. Figure 12).

Figure 11: example of rendered frame from R97_SA_QP1_Tt_p03_2048x2048_yuv420p10le: TMIV3.0 anchors (top) versus ours (bottom).

Figure 12: example of rendered frame from R97_SB_QP1_Tt_p01_2048x2048_yuv420p10le: TMIV3.0 anchors (top row) versus ours (bottom row).

# Recommendations

Interdigital has developed a synthesizer that gives good subjective results and good objective results when using the encoded bitstreams of TMIV3.0 anchors. Hence, we recommend adopting the synthesizer in the TMIV 4.0 as, at least, an additional synthesizer.

We also encourage proponents to use this synthesizer at encoder side whenever a virtual view needs to be synthesized.

# References

- A. Dziembowski, “Description of Immersive Video Core Experiments 2: Pixel Pruning”, ISO/IEC JTC1/SC29/WG11, N18933, October 2019, Geneva, Switzerland.

- J. Fleureau, R. Doré, F. Thudor, T. Tapie, G. Briand, B. Chupeau, “CE2.6.2: Graph-based pruning for natural content”, ISO/IEC JTC1/SC29/WG11, M52414, January 2020, Brussels, Belgium.

- J. Jung, B. Kroon, J. Boyce, “Common Test Conditions for Immersive Video”, ISO/IEC JTC1/SC29/WG11, N18789, October 2019, Geneva, Switzerland.

- R. Doré, G. Briand, “Comment on quantification of Nokia Chess content depth for MIV CTC”, ISO/IEC JTC1/SC29/WG11, M52424, January 2020, Brussels, Belgium.