INTERNATIONAL ORGANISATION FOR STANDARDISATION

ORGANISATION INTERNATIONALE DE NORMALISATION

ISO/IEC JTC1/SC29/WG11

CODING OF MOVING PICTURES AND AUDIO

ISO/IEC JTC1/SC29/WG11 MPEG2019/m52425

January 2020, Brussels, Belgium

Source Interdigital

Status Input contribution

Title New Depth QP proposal for CTC

Author Renaud Doré, Gérard Briand

# Introduction

MPEG MIV working group is at a point where subjective results should now matter even more than objective results and should converge to a satisfactory level. While many efforts have been done – and are still on-going - to improve subjective performance at lowest QPs (QP1 and QP2), the results at lowest quality QPs are rarely viewed.

As a matter of facts, the pose traces for QP4 and QP5 are just disastrous, while still being accounted in the objective metrics for the low BD rate, for a majority of contents.

This contribution proposes to change the values of the QP Depth on the left part of the curves (highest QP values, low bit rates). There has been relevant remarks at last meetings that QP Depth could also be changed advantageously for the right part of the curves (lowest QPs, high bitrates) for some content like D without any noticeable effect, but that is another topic and this contribution does not address this.

# Current status with TMIV3.0 CTC

Here is the list of QP5 snapshots, sorted by level of increasing “horribleness”. One should have in mind that a still picture does not reflect dynamic artefacts which makes the biggest damages:

Figure 1: snapshot of QP5 contents

The subjective quality is so bad that it does invalidate any MIV use cases at these lower bitrates, even if these most compressed bitrate configurations are supposed to be viewed on small screen like on a tablet or mobile phone,.

Also, this very bad quality is a threat to the reputation of the work done in this group.

# Proposal to improve depth quality

The reason why most of largest QPs are unviewable is because the contours are extremely affected by the depth compression and show extremely visible dynamic artefacts. Saliency is at the contours. Here is TMIV3.0 CTC QPs [1, table15]:

Table 1: QPs used for depth and texture

|  | QP1 | QP2 | QP3 | QP4 | QP5 |
| --- | --- | --- | --- | --- | --- |
| Texture QP | 22 | 27 | 32 | 37 | 42 |
| Depth QP | 9 | 14 | 21 | 30 | 38 |

We should actually acknowledge the fact that MIV deals with many large Field of View contents. For these contents, there is no way to stream related contents in a couple of Mbits, and we should accept to increase the lowest bitrates.

Also, we would like to readdress the question of putting the same QPs for all 8 contents. In our opinion, the contents are so different in 1) Field of View 2) depth quality (synthetic vs natural) 3) input pixel rate 4) texture and depth grain 5) rig cameras divergence .. that there is no reason to converge to a single set of QP’s. We proposal in fact to have different Depth QP configurations but to limit that number to 3.

In short, proposal 1 consists in

- Keeping untouched the depth QP1, so that the (PSNR, bitrate) higher part will remain unchanged

- For all contents except D and L, re-boost the other depth QPs, so that it is still possible to visualize the QP3, QP4 and QP5 pose traces, mechanically making the lower curve part steeper.

- Keep unchanged the depth for D and L (but see the proposal 2 in this contribution)

The 3 QP configurations are as follows:

Table 2:  QPs used for depth

|  | Contents | QP1 | QP2 | QP3 | QP4 | QP5 |
| --- | --- | --- | --- | --- | --- | --- |
| Depth QP configuration | B, N | 9 | 9 | 9 | 14 | 14 |
| Depth QP configuration 2 | A, C, E, J | 9 | 9 | 14 | 14 | 21 |
| Depth QP configuration 3 | D, L | 9 | 14 | 21 | 30 | 38 |

Explanations:

- B and N are extremely sensitive to the quality of contours, because of texture contrasts and presence of depth cuts at close distance

- A, C, E and J are less sensitive but get benefits of better depths as well

- D and L are not sensitive by the depth

  - D rig cameras are co-aligned, making depth defaults less impacting.

  - L depth is very noisy by itself, hiding depth compression noise

## Objective results

As expected, we get overall worse Low BR rates, with the exception of B and N which gets better results.

All changed curves (profile 1 & 2) exhibit steeper slopes to the left, as expected.

These results are still satisfactory in the sense that

- The span in bitrate (Max bitrate / Min bitrate) is still large enough

- The span in PSNR (Max PSNR – Min PSNR) is still large enough, although below 2dB for A and N

## Subjective results

Interdigital will bring side-by-side videos to visualize the pose traces in the viewing session TMIV3.0 vs new Depth Proposal. To our opinion, the QP5 (and QP4) becomes “viewable”. Here is our own assessment table:

| Content | A | B | C | E | J | N |
| --- | --- | --- | --- | --- | --- | --- |
| QP5 pose trace improvement | More viewable (cf p03) | Much more viewable | More viewable | A bit more Viewable | More viewable | Much more viewable |

Here is a snapshot of the most impressive content improvement (B):

It is amazing to see how a MIV content with horrible texture but good contours is still valuable for  human vision.

# Conclusion

We propose an adjustment of the CTC Depth QPs values by considering subjective performances and not objective performances. The obtained objective results degradation for most of contents is the price to pay to have MIV contents which mean something in all bitrate configurations.

We recommend to take our proposed QP depths in the next CTC and for the next Anchors computation.

Generally speaking, we recommend to put more and more emphasis to subjective performances, at this latter stage of MIV project. We believe that

- the focus on subjective quality will force the MIV group to reactivate the question of the depth bitrate cost which is necessary to have the targeted quality. What was valid for video compression activity is no more true for immersive contents.

- We should be sure that MIV technology is able to deliver near to perfect images in the best configuration, which is not the case at QP1 Depth= 9. We should therefore also wonder whether the lowest QP value is low enough to have this “near perfect” images

# References

- J. Jung, B. Kroon, J. Boyce, Common Test Conditions for Immersive Video, ISO/IEC JTC1/SC29/WG11 MPEG/N18789, October. 2019, Geneva, Switzerland.