INTERNATIONAL ORGANISATION FOR STANDARDISATION

ORGANISATION INTERNATIONALE DE NORMALISATION

ISO/IEC JTC1/SC29/WG11

CODING OF MOVING PICTURES AND AUDIO

ISO/IEC JTC1/SC29/WG11 MPEG2019/m50783

October 2019, Geneva, CH

| Source | Interdigital |
| --- | --- |
| Status | Input contribution |
| Title | Report on new synthetiser |
| Author | Julien Fleureau, Franck Thudor, Renaud Doré |

# Introduction

It is a fact that the quality of the current pose traces produced by TMIV2.02 do not meet the expectations of the group so far. It was Interdigital intention to carry out investigations to improve this synthesizer. 
Unfortunately, time was short to come up with convincing results, so that Interdigital will not propose a new synthesizer for this Geneva meeting.

Still, we believe relevant to report our – very time consuming – work on the synthesizer.

# Design option

Roughly speaking, the new synthesizer is a CPU port of a proprietary GPU synthesizer already in use within Interdigital. Its main architecture is quite classical and is compound a two main stages: i) a visibility pass which goal is to estimate a depth map for the current viewport to synthesize, and ii) a shading pass which aims at defining a color for each pixel of the viewport to synthesize.

The visibility stage is a splat based pipeline. It first computes the geometric contribution of each input views and then weights and blends these individual contributions to define a final depth information per viewport pixel. For each individual (per view) contribution, a splat is projected on the viewport for each pixel with a size which depends on its neighbourhood average distance (to avoid holes in the depth maps) and on its local depth gradient (to avoid geometric contour artifacts). The final depth selection is a voting scheme which basically favors cameras “closer” from the viewport to synthesize but also lower depth values.

The shading stage occurs directly in the viewport space and makes use of the latter visibility map to weight and blend the source color contributions. More precisely, the color of all source view pixels being reprojected at the current position to shade are epsilon-blended i.e. blended depending on their compatibility with the depth information of the visibility map at this position. Additionally, pixels coming from depth contours of input views which often suffer from aliasing effects are also weighted with lower values to avoid “ghost” contours in the rendered viewport.

# Subjective performances on plain source views

As a first important step, we have tried to compare the performances of the proposed synthesizer with the current TMIV ones making use of all plain source views for the synthesis of pose traces. Such a comparison is meaningful because:

- It gives in principle the limit which the TMIV can not outperform: if there are already some artifacts at this stage, they are very likely to be reported after the TMIV chain.

- The capability of creating a new view from all plain source views may be important in some configurations like when a central view is re-synthesized.

In all following side-by-side figures, the left part will be related to the current TMIV 2.02 synthesizer while the right one will be related to the Interdigital (IDC) current synthesizer.

## ClassroomVideo: similar performances

On Classroom video, visually comparable results have been observed on the synthesized pose traces. The synthesis is however less “noisy” for the IDC synthesizer as the shading pass (which takes into account all camera contributions) tends to filter the speckle artifacts present in the original source images and due to the ray-tracing configuration used when the content has been generated.

## TechnicolorMuseum: less contour artifacts

With the TMIV 2.02 synthesizer,  one can observe contour artifacts due to bad triangle rasterization but also due to group of points spread around contours. None of these artifacts are present in the new synthesizer especially due to splat-based approach which does not generate any “sticky” triangles.

## TechnicolorHijack: no “click” artifacts

The pose traces are equivalent, at the exception of small temporal instabilities with TMIV2.02 synthesizer on the upper glass windows (of course not apparent on the still picture above).

## TechnicolorPainter: similar performances

## IntelFrog: similar performances

## OrangeKitchen: no “click” artifacts

There are some temporal instabilities on TMIV2.02 synthesizer (click effect) which do not exist on IDC synthesizer. Apart from that, synthesized pose traces are visually equivalent.

## PoznanFencing: no transparency artifacts

There are some transparencies on TMIV 2.02 synthesizer, for example on left fencer helmet which do exist with IDC synthesizer. The result is nevertheless not good to see anyway, considering the quality of the depth.

All in one, the IDC synthesizer works as good or better than current TMIV2.02 when using all plain source views.

# Comparative Subjective Performances on the TMIV2.02 chain

We now report the subjective performances obtained when synthesizing from the atlas. To that end, the IDC synthesizer have been inserted in the TMIV2.02 chain (as a replacement for the current one).

The synthesizer is challenged when fed with sparse and non-redundant information as provided with the decoded patch atlases. An overa

| Content | Observation | Explanation |
| --- | --- | --- |
| ClassroomVideo | equivalent |  |
| TechnicolorMuseum | slightly better | No transparency artifacts and a little less contour artifacts |
| TechnicolorHijack | equivalent |  |
| TechnicolorPainter | slightly worse | Bug related to inpainting still to find out |
| IntelFrog | equivalent |  |
| OrangeKitchen | better | No transparency artifacts |
| PoznanFencing | equivalent | Both pose traces are too bad anyway to make any ranking |

Here below are a couple of side-by-side images where typical differences have been identified.

All in one, the IDC synthesizer behaves slightly but not significantly better than TMIV2.02 when trying to recover pose traces from atlases. But there is not so much room for improvement as long as the pruning discards so much the visual stuff at the encoding side.

# Discussion and conclusion

So far, the synthesizer performance is not enough to be worth sharing. There are probably still a couple of bugs which leads to under-performance with respect to the current synthesizer. We will try to fix them in the coming weeks.

Would we come up with a more mature IDC synthesizer during the ad hoc meeting period, and would some partner volunteer to crosscheck its performances, this IDC synthesizer could be added to the TMIV3.0 before MPEG129, and Core Experimenters could choose to use it in addition to the current TMIV synthesizer.

In any case, we can still draw the following conclusions:

- Current TMIV2.02 synthesizer does an acceptable job even if it can still be improved to remove contour and transparency artifacts.

- It is only partly responsible for the poor quality of the pose traces that the MIV group can propose today. Coming up with a new synthesizer won’t radically change the results when synthesizing from the atlases / pruned views.

- The problem is very likely to come from: i) the pruning stage as it is currently implemented (inherited from Technicolor-Intel CfP answer but without using a central view) but also from ii) the poor quality of certain content (cf PoznanFencing) where camera calibration, depth estimation and color management have not been done optimally.

- PSNR improvement are definitely not a reliable indicator of the perceptual quality, especially in our “volumetric” context. Multiplying the number of metrics is not a good solution either:  past experiences (cf HDR) have shown the limitations of such a way to do, and the time schedule currently envisioned by the group is maybe too short to go in these kind of investigations. Further improvements need to improve pose traces first – as proposed in [2] - to convince the community to adopt our technology.

# References

- J. Jung, B. Kroon, J. Boyce, Common Test Conditions for Immersive Video, ISO/IEC JTC1/SC29/WG11 MPEG/N18563, July. 2019, Goeteborg, Switzerland.

- Renaud Doré, Gérard Briand, PoznanFencing&OthersContentsPose traces, m50602 Geneva MPEG128