UKTV Acquisitions Technical Delivery Specification

Please note that every programme submitted for transmission must satisfy an Automated Quality Control process specified by UKTV on whose behalf Red Bee Media (RBM) are acting. Any programme failing the QC process may be rejected and returned to the supplier for repair. Where specified the supplier must adhere to the specific technical delivery requirements of the broadcaster that form an integral part of the contract. If you wish to deliver in any other form than detailed in this document please contact uktv.media.prep@redbeemedia.com.

Notification of Delivery of Master Materials

Once a file(s) has been delivered to RBM, please send an email notification to uktv.media.prep@redbeemedia.com and complianceops@uktv.co.uk so they can start coordinating the UKTV workflow process.

*For information on delivery of Supporting Materials, please see the end of this document.

Technical Specifications for Master Materials

| Content Type | HD (preferred) | HD (alternative) | SD |
| --- | --- | --- | --- |
| Video Codec: | AVC Intra | XDCAMHD/MPEG | IMX 50 |
| Stream Type: | AVCi OP1A | XDCAMHD | OP1A |
| CBR or VBR: | CBR | CBR | CBR |
| Average Video Bit Rate: | 100000kbps | 50000kbps | 50000kbps |
| Chroma Sampling: | 422P@HL | 422P@HL | 422P PAL |
| Scan Type: | MBAFF | M3 N12/Interlaced/TFF | iFrame/Interlaced/TFF |
| Frame Rate: | 25 | 25 | 25 |
| Aspect Ratio: | 1.78:1 | 1.78:1 | 1.78:1/1.33:1 |
| Frame Size: | 1920x1080 | 1920x1080 | 720x576 |
| Audio Codec: | PCM | PCM | PCM |
| Audio Sample Rate: | 48k | 48k | 48k |
| Audio Bit Depth: | 24b | 24b | 24b |
| Audio Bit Rate: | 9216kbps | 9216kbps | 4608 |
| Audio Channels: | 8 | 8 | 4 |
| File Container: | MXF/OP1A/ | XDCAMHD | MXF/OP1A/ |
| File Extension: | mxf | mx | mxf |

Audio Configuration

| Track | Preferred for Stereo R123:4b | Preferred for Surround R123:16c |
| --- | --- | --- |
| 1 | Stereo Final Mix L | Stereo Final Mix L |
| 2 | Stereo Final Mix R | Stereo Final Mix R |
| 3 | Stereo M&E L | Stereo M&E L |
| 4 | Stereo M&E R | Stereo M&E R |
| 5 |  | 5.1 Final Mix L |
| 6 |  | 5.1 Final Mix R |
| 7 |  | 5.1 Final Mix C |
| 8 |  | 5.1 Final Mix LFE |
| 9 |  | 5.1 Final Mix Ls |
| 10 |  | 5.1 Final Mix Rs |
| 11 |  | 5.1 M&E L or Muted |
| 12 |  | 5.1 M&E R or Muted |
| 13 |  | 5.1 M&E C or Muted |
| 14 |  | 5.1 M&E LFE or Muted |
| 15 |  | 5.1 M&E Ls or Muted |
| 16 |  | 5.1 M&E Rs or Muted |

Please note that we require audio channels 1 + 2 to be in Full Stereo Mix. Dual Mono audio will be rejected and only acceptable if Full Stereo Mix is not available for the asset. Audio generated as mono must be presented on two phase-coherent tracks and flagged as stereo.

Expected audio track configuration should be clearly identified on the clock for digital file delivered content and on the VTR Paperwork for tape delivered content.

Line-Up

Preference for Stereo Line-Up tone is for GLITS at 1kHz for Final Mix and 2kHz for M&E at - 18dBFS (sinusoidal, free of distortion and phase coherent between channels).

Preference for Surround Line-Up tone is for BLITS (sinusoidal, free of distortion and phase coherent between channels).

Preference for Video Line-up is for 100% Bars.

All files and tapes (Digital Betacam, HDCam or HDCam SR) must include bars, tone and identifying clock lead in. The standards and references being used must be clearly identified on clock for digital file delivered content and on the VTR Paperwork for tape content.

Use of Non-HD Material

Some high-definition programmes will contain some material from standard definition originals and sources which are not considered to meet HD broadcast standards, such as domestic camcorders. This material is all called ‘non-HD’ in this document. To maintain a high standard and meet audience expectations the amount of non-HD material is limited to 25% of the programme’s total duration. Non-HD material must not be used for large uninterrupted sections of the programme, unless agreed by the broadcaster. This includes archive material.

Delivery of Supporting Materials

RBM do not handle the delivery of supporting materials. Instead please send supporting materials as instructed below -

- Episode Running Order (episode numbers and titles) -

compliance@uktv.co.uk

programmingexecutives@uktv.co.uk

**Please state in the email the Show and Season the content relates to.

- Stills / Caption Logs / Synopses / Press Packs -

frontdesk@uktv.co.uk

pressoffice@uktv.co.uk

creativeproduction@uktv.co.uk

**Please state in the email the Show and Season the content relates to.

- Music Cue Sheets –

uktvmusic@on-music.tv

frontdesk@uktv.co.uk

**Please state in the email the Show and Season the content relates to.

- Scripts –

rbm.uk.as.programme.materials@redbeemedia.com

**Please state in the email the Show and Season the content relates to.

- Audio Stems - please upload labelled WAV files to UKTV’s Aspera folder (files can be zipped up into a package).

https://aspera.pub/1ZOyRHE

**All uploads must be accompanied by a notification email to creativeproduction@uktv.co.uk stating in the email the Show and Season the content relates to.