Puzzle

Description

Puzzle blends performance, sustainability, and a bold visual language into one striking modular concept. The pendant luminaire features a shadow-free glowing light effect, smart system readiness, and modern aesthetics.

USP

- Pendant mounting options with driver in canopy for concrete & wooden ceilings and driver on ceiling for false ceilings

- Continuous glowing light effect through transparent body

- Various light recipes: 3000K, 4000K, CRI 90+

- Office compliant

- Heat resistant & impact resistant PET-G material for housing

- Printed with mass-balanced 50% recycled copolyester materials

- Driver options: PSU (Non Dimming) / DIA (Dali Interact Dimming) / WIA (Wireless Interact dimming) / SIA (Sensor ready driver)

- Interact ready

- Integrated sensor option for presence detection and daylight control

- EPD document available (Environmental Product Declaration)

- Disassembly instructions available

The Projector complies with the following standards:

- CE marked

- RoHS

Specifications:

- Efficacy up to 152 lm/W

- CRI90, Color temperature: 3000K / 4000K

- Lumen: 4000 and 5000

- Lumen maintenance: 72,000hrs to L80 @ 25°C Ta

- Driver Failure Rate <1% @ 5,000 hrs

- IP20 / IK02

- Electrical Class II

- Ambient range: +10°C to +35°C

- Glow wire test 650°C

- 5-year warranty

- Binning: CRI90 SDCM<2

Schematic light drawing

Dimensional drawing