Cewood

HOW TO WRITE UP YOUR SPECIFICATION

Please note that some options are available in stock and ready for immediate despatch and are listed below. Other items available on 20+ week lead time.

- Interior panelling shall be Cewood by Decor Systems.

- Supplier: Decor Systems Ph: 1800 835 035

- Thickness Panel: ____mm (15/25/35mm)

- Strand Structure to be ___________ [please choose a thickness strand option]:

  - • S — Super Fine, 1 mm (IN STOCK, AVAILABLE FOR IMMEDIATE DESPATCH)

  - • F — Fine, 1.5 mm

  - • R — Regular, 3 mm

- Edge Profile to be __________ please choose an edging option:

  - PO – Panel without Chamfer (IN STOCK, AVAILABLE FOR IMMEDIATE DESPATCH)

  - P5 – Panel with 5mm chamfer (IN STOCK, AVAILABLE FOR IMMEDIATE DESPATCH)

  - P11 – Panel with 11mm chamfer

  - PO-T24 Standard T24 ceiling profile

  - P0G-T24 Immersed Ceiling Profile

  - P5G-T24 – Immersed ceiling profile with 5mm chamfer

  - P5H - Hidden suspended ceiling profile with 5 mm chamfer

  - P5S - Hidden suspended ceiling profile with overhang and 5 mm chamfer

- Panels to be coloured to be ______ [Natural or RAL _______ (Insert Code)]

- Panels to be ​Fire Group 1 per BCA ​Specification ​C1.10 clause 4.

- Cewood cannot to be substituted unless agreed by Architect and Acoustic Engineer. Panels should be allowed to acclimatise on site for at least five days before installation. Decor Systems certificate #  (Contact your Decor Systems project consultant for your individual project number) to be supplied upon completion.

- Panel ​installation to ​be as per ​manufacturer'​s instructions ​and AS/NZS 2785.​ - Installation ​to be done ​by an approved ​ceiling and ​partition ​contractor. ​

- Product ​guarantee to be ​99 years as per Decor Systems guarantee.

- Please contact Decor Systems for your project specific completion number.