DecorZen – Contours of Country

HOW TO WRITE UP YOUR SPECIFICATION

- Interior panelling shall be DecorZen Contours of Country by Decor Systems.

- Supplier: Decor Systems Ph: 1800 835 035

- Profile: ___________ (Insert artwork name selected)

- Finish: ________________  (Choose from Natural timber veneers, 2 pack poly colours or the range of laminates available)

- Substrate of panels to be ___ (Choose from: FR MDF / MR MDF / Plywood / OSB Board / Black MDF / Other)

- Joint to be ___________ [please choose a joint option: (Square / Arrised)]

- Edging to be __________[please choose an edging option: (Matching / Black / N/A)]

- DecorZen panels shall be supplied with DecorSorb Integrated Acoustic Backing.

- Panels to be include DecorInsul behind the panels.

- Decor Systems to supply Black Backing Strips as part of system.

- DecorShield  to be applied.

- Panels to be ​Fire Group 3 / ​2 / 1 [delete ​as required] ​per BCA ​Specification ​C1.10 clause 4. ​[Note that fire ​groups 1 is ​only available ​in the ​DecorImage or SmartLook range of laminate ​finish type and ​Group 2 in ​Natural timber veneers]

- DecorZen cannot to be substituted unless agreed by Architect and Acoustic Engineer. Panels should be allowed to acclimatise on site for at least five days before installation. Decor Systems certificate #  (Contact your Decor Systems project consultant for your individual project number) to be supplied upon completion.

- Panel ​installation to ​be as per ​manufacturer'​s instructions ​and AS/NZS 2785.​ - Installation ​to be done ​by an approved ​ceiling and ​partition ​contractor. ​

- Product ​guarantee to be ​99 years as per Decor Systems guarantee.

- Please contact Decor Systems for your project specific completion number.

Cultural Intellectual Property Disclaimer

The designs featured in our Contours of Country range are the result of a unique collaboration with four Aboriginal artists. These artworks are integral to the aesthetic and functional quality of our acoustic panels and embody significant cultural heritage and intellectual property.

As such, these designs are protected by cultural intellectual property rights and cannot be reproduced, copied, or imitated in any form without explicit permission from the artists and Decor Systems. Unauthorised use of these designs may constitute a breach of these rights and could lead to legal action.

We appreciate your respect and understanding of the cultural significance of these artworks and your commitment to upholding the integrity of this collaboration.