DIVISION 26 00 00					ELECTRICAL

SECTION 26 50 00 				LIGHTING

SECTION 26 56 00 				EXTERIOR LIGHTING

LIGHT COLUMN PATHWAY BOLLARD

Part 1		General

- Summary

  - Section Includes:

    - Series 500 pathway bollard, 5" diameter column

    - Series 600 pathway bollard, 6" diameter column

- Submittals

  - Provide submittals in accordance with Section 01 33 00 – Submittal Procedures.

  - Product data:

    - Manufacturer’s standard product literature.

    - Shop drawings.

    - Installation instructions.

    - Maintenance instructions.

- Quality Assurance

  - Manufacturer Qualifications:

    - Minimum 15 years experience in the manufacture of outdoor lighting.

    - Provide reference list of at least ten major transportation authorities, municipalities, universities, or

other high-use public environments currently using outdoor lighting fabricated by the manufacturer.

- Delivery, Storage and Handling

  - Handle products in accordance with manufacturer’s instructions.

  - Store products in manufacturer’s original packaging until ready for installation.

  - Protect products from impacts and abrasion during storage.

- Warranty

  - Provide manufacturer’s standard warranty.

    - Warranty terms: three years from date of invoice against defects in materials and workmanship.

Part 2		Products

- Manufacturer

  - Basis-of-design product: provide bollards based on the product named.

    - Light Column Pathway Bollard by Forms+Surfaces

    - Manufacturer Contact:
Forms+Surfaces
1901 William Flynn Highway 
Glenshaw, PA 15116
phone: 800-451-0410

fax: 412-385-4715

email: sales@forms-surfaces.com

website: www.forms-surfaces.com

  - Pathway Bollards

    - Materials:

      - Body: tubular stainless steel.

      - Base: stainless steel plate.

      - Head: stainless steel cap.

      - Lens: 0.125" thick white-frosted acrylic tube.

      - LED Driver: Input power is 100-277 VAC. Includes high efficiency constant output current with over-voltage, short circuit, and overload protection, has 0-10V dimming capabilities. Certifications include: IP66 (waterproof) enclosure and Class 2 rated output (UL 8750).

      - Custom LED light engine (Choose one lamp below):

        - Luminaire lumens: Series 500: 886 Series 600 :808.

            - Color Temperature: (Choose one color temperature below):

              - 3,000K.

              - 4,000K.

    - Finish:

      - Stainless steel with Satin finish and Ceramiloc treatment.

      - Polyester powdercoat (Choose one color below):

        - Standard Texture/Gloss from Forms+Surfaces Powdercoat Chart.

        - Custom RAL powdercoat color.

    - Dimensions: (Choose model(s) below):

a. 	Series 500 pathway bollard:

- Surface Mount with J-bolts:

A) 5" diameter x 9.75" high

b. 		Series 600 pathway bollard:

        - Surface Mount with J-bolts:

A) 6" diameter x 9.75" high.

    - Mounting:

a. Surface Mount: 3/8" thick stainless steel base plate, slotted for rotational capability. Stainless steel mounting hardware sold separately. Template available upon request.

Part 3		Execution

- Examination

  - Verify that substrates are stable and capable of supporting the weight of items covered under this section.

  - Verify that substrates have been adequately prepared to securely anchor those items that will be surface

mounted.

- Installation

  - Install according to the manufacturer’s installation instructions.

  - Install in conformance to applicable ADA guidelines and End User’s established Accessibility policies.

END OF SECTION