DIVISION 12 00 00					FURNISHINGS

SECTION 12 93 00 				SITE FURNISHINGS

SECTION 12 93 23 				TRASH AND LITTER RECEPTORS

ORBIT LITTER & RECYCLING RECEPTACLE

Part 1		General

- Summary

  - Section Includes:

    - Single-Stream receptacle, Stainless Steel, (1) 30-gallon liner.

    - Split-Stream litter & recycling receptacle, Stainless Steel, (2) 13-gallon liners.

- Submittals

  - Provide submittals in accordance with Section 01 33 00 – Submittal Procedures.

  - Product data:

    - Manufacturer’s standard product literature.

    - Shop drawings.

    - Installation instructions.

    - Maintenance instructions.

- Quality Assurance

  - Manufacturer Qualifications:

    - Minimum 15 years experience in the manufacture of litter and recycling receptacles.

    - Provide reference list of at least ten major transportation authorities, municipalities, universities, or

other high-use public environments currently using litter and recycling receptacles fabricated by the manufacturer.

- Delivery, Storage and Handling

  - Handle products in accordance with manufacturer’s instructions.

  - Store products in manufacturer’s original packaging until ready for installation.

  - Protect products from impacts and abrasion during storage.

- Warranty

  - Provide manufacturer’s standard warranty.

    - Warranty terms: three years from date of invoice against defects in materials and workmanship.

Part 2		Products

- Manufacturer:

  - Basis-of-design product: provide receptacle(s) based on the product named:

    - Orbit Litter & Recycling Receptacle by Forms+Surfaces

    - Manufacturer Contact:
Forms+Surfaces
1901 William Flynn Highway 
Glenshaw, PA 15116
phone: 800-451-0410

fax: 412-385-4715

email: sales@forms-surfaces.com

website: www.forms-surfaces.com

  - Receptacles

    - Materials:

      - Frame: cast aluminum top and bottom rings with stainless steel supports.

      - Body: Stainless Steel.

      - Lid: cast aluminum.

      - Base: black recycled rubber.

      - Liners: black polyethylene with UL94HB fire rating.

      - Hardware and hinges: stainless steel.

    - Finishes:

      - Main panels (Choose one finish below. Main Panel material must be the same as Accent Panel material.):

        - Stainless Steel, Sandstone.

        - Stainless Steel, Seastone.

        - Stainless Steel, Linen.

        - Stainless Steel, Diamond.

        - Stainless Steel, Satin.

      - Accent panels (Choose one finish below. Accent Panel material must be the same as Main Panel material.):

        - Stainless Steel, Sandstone.

        - Stainless Steel, Sandstone, Scape Perforation.

        - Stainless Steel, Seastone.

        - Stainless Steel, Seastone, Scape Perforation.

        - Stainless Steel, Seastone, Seagrass Eco-Etch.

        - Stainless Steel, Linen.

        - Stainless Steel, Linen, Scape Perforation.

        - Stainless Steel, Diamond.

        - Stainless Steel, Diamond, Scape Perforation.

        - Stainless Steel, Satin.

        - Stainless Steel, Satin, Scape Perforation.

        - Stainless Steel, Satin, Seagrass Eco-Etch.

      - Lid and base (Select one option below):

        - Standard Texture from Forms+Surfaces Powdercoat Chart.

        - Custom RAL powdercoat color.

      - Base: black recycled rubber.

    - Dimensions (Select model(s) below):

      - Single-Stream receptacle:

        - Overall: 21.6" wide x 17.6" deep x 32.1" high (without levelers).

        - Nominal capacity: 30 gallons.

        - Weight:  77 lbs.

      - Split-Stream litter & recycling receptacle:

        - Overall: 21.6" wide x 17.6" deep x 32.1" high (without levelers).

        - Nominal capacity: 26 gallons (uses two 13-gallon liners).

        - Weight: 77 lbs.

    - Instructional Graphics:

      - Apply instructional graphics to lids as specified to indicate the intended waste or recycling stream.

        - Graphics type: back-printed polycarbonate.

        - Letters and symbols color: White.

        - Graphics background colors:

            - Black.

            - Blue.

            - Green.

    - Installation (Select one below):

      - Freestanding: levelers included.

      - Surface mount: levelers included. Stainless steel anchors and mounting screws sold separately.

Part 3		Execution

- Examination

  - Verify that substrates are stable and capable of supporting the weight of items covered under this section.

  - Verify that substrates have been adequately prepared to securely anchor those items that will be surface

mounted.

- Installation

  - Install according to the manufacturer’s installation instructions.

  - Install in conformance to applicable ADA guidelines and End User’s established Accessibility policies.

END OF SECTION