DIVISION 26 00 00					ELECTRICAL

SECTION 26 50 00 				LIGHTING

SECTION 26 56 00 				EXTERIOR LIGHTING

APTOS BOLLARD

Part 1		General

- Summary

  - Section Includes:

    - Aptos bollard, hardwired

    - Aptos bollard, solar

- Submittals

  - Provide submittals in accordance with Section 01 33 00 – Submittal Procedures.

  - Product data:

    - Manufacturer’s standard product literature.

    - Shop drawings.

    - Installation instructions.

    - Maintenance instructions.

- Quality Assurance

  - Manufacturer Qualifications:

    - Minimum 15 years experience in the manufacture of outdoor lighting.

    - Provide reference list of at least ten major transportation authorities, municipalities, universities, or

other high-use public environments currently using outdoor lighting fabricated by the manufacturer.

- Delivery, Storage and Handling

  - Handle products in accordance with manufacturer’s instructions.

  - Store products in manufacturer’s original packaging until ready for installation.

  - Protect products from impacts and abrasion during storage.

- Warranty

  - Provide manufacturer’s standard warranty.

    - Warranty terms: three years from date of invoice against defects in materials and workmanship.

Part 2		Products

- Manufacturer

  - Basis-of-design product: provide bollards based on the product named.

    - Aptos Bollard by Forms+Surfaces

    - Manufacturer Contact:
Forms+Surfaces
1901 William Flynn Highway 
Glenshaw, PA 15116
phone: 800-451-0410

fax: 412-385-4715

email: sales@forms-surfaces.com

website: www.forms-surfaces.com

  - Bollards

    - Materials:

      - Bollard Body:

        - Cast aluminum.

      - Head Cap (Select one option below):

        - Cast aluminum.

        - Cast aluminum with solar module of monocrystalline cells encapsulated in high-impact, UV resistant polyurethane.

      - Solar Lighting Controller (for solar bollards):

        - Fully integrated electronics controller and energy management system with self-learning, predictive power management capabilities. Integrated charger circuit with a maximum power point tracking charge controller.

      - Battery (for solar bollards):

        - High performance lithium ion with high temperature tolerance, over-discharge protection and over-current protection.

      - Driver (for hardwired bollards): Input power is 100-277 V. Includes protection from output open load, over-current and short-circuit, and over-temperature with auto recovery. Compatible with forward phase, reverse-phase, and 0-10V dimmers. Certifications include IP64 enclosure.

      - Lamps (Choose one lamp below):

        - 3000K LED:

            - High output CREE LEDs

            - Luminaire lumens: 205 (AC); 49 (solar)

            - Color temperature: 3,000K

        - 4000K LED:

            - High output CREE LEDs

            - Luminaire lumens: 220 (AC); 53 (solar)

            - Color temperature: 4,000K.

    - Finish (Choose one finish below):

      - Polyester powdercoat (Select one option below):

        - Standard Texture from Forms+Surfaces Powdercoat Chart.

        - Custom RAL powdercoat color.

    - Dimensions (Choose model(s) below):

      - Hardwired Bollard:

        - 6.4" diameter x 36.5" high.

      - Solar Bollard:

        - 6.5" diameter x 36.3" high.

    - Mounting:

      - Standard: Surface Mount with anchor bolts: 5.1" base plate with 1.8" conduit opening. Stainless steel mounting hardware sold separately.

Part 3		Execution

- Examination

  - Verify that substrates are stable and capable of supporting the weight of items covered under this section.

  - Verify that substrates have been adequately prepared to securely anchor those items that will be surface

mounted.

- Installation

  - Install according to the manufacturer’s installation instructions.

  - Install in conformance to applicable ADA guidelines and End User’s established Accessibility policies.

END OF SECTION