DIVISION 12 00 00					FURNISHINGS

SECTION 12 93 00 				SITE FURNISHINGS

SECTION 12 93 23 				TRASH AND LITTER RECEPTORS

APEX RECEPTACLE

Part 1		General

- Summary

  - Section Includes:

    - Litter and Recycling Receptacle, 24- and 36-gallons.

- Submittals

  - Provide submittals in accordance with Section 01 33 00 – Submittal Procedures.

  - Product data:

    - Manufacturer’s standard product literature.

    - Shop drawings.

    - Installation instructions.

    - Maintenance instructions.

  - Submit powdercoat finish samples for approval.

- Quality Assurance

  - Manufacturer Qualifications:

    - Minimum 15 years experience in the manufacture of litter and recycling receptacles.

    - Provide reference list of at least ten major transportation authorities, municipalities, universities, or

other high-use public environments currently using litter and recycling receptacles fabricated by the manufacturer.

- Delivery, Storage and Handling

  - Handle products in accordance with manufacturer’s instructions.

  - Store products in manufacturer’s original packaging until ready for installation.

  - Protect products from impacts and abrasion during storage.

- Warranty

  - Provide manufacturer’s standard warranty.

    - Warranty terms: three years from date of invoice against defects in materials and workmanship.

Part 2		Products

- Manufacturer

  - Basis-of-design product: provide receptacle based on the product named:

    - Apex Receptacle by Forms+Surfaces.

    - Manufacturer Contact:
Forms+Surfaces
1901 William Flynn Highway 
Glenshaw, PA 15116
phone: 800-451-0410

fax: 412-385-4715

email: sales@forms-surfaces.com

website: www.forms-surfaces.com

  - Receptacles

    - Materials:

      - Body frame and lid: solid corrosion-resistant aluminum.

      - Container walls: heavy corrosion-resistant aluminum.

      - Container wall inset material: (Choose one below):

1)		FSC® 100% Cumaru hardwood slats. (FSC License Code: FSC-C004453).

2)		Stainless Steel.

d.	Internal liner: black low-density polyethylene (LLDPE) with UL94HB fire rating.

e.	Latch: stainless steel.

f.	Hardware: stainless steel.

g.	Rain cover (optional): heavy corrosion-resistant aluminum.

    - Finishes:

      - Body frame and lid: polyester powdercoat (Select one option below):

              - Standard Texture from Forms+Surfaces Powdercoat Chart.

              - Custom RAL powdercoat color.

      - Insets: (Choose one below):

              - FSC 100% Cumaru hardwood slats.

              - Stainless Steel, Sandstone finish.

              - Stainless Steel, Seastone finish.

              - Stainless Steel, Diamond finish.

              - Stainless Steel, Satin finish.

c. 	Rain cover (optional): polyester powdercoat to match lid.

3.	Latch: (Choose one below):

1)  	Standard lift latch.

2)	Tool-operated security latch.

4.		Dimensions (Select models below):

      - Receptacles:

        - 24-gallon, single-stream receptacle:

            - 24-gallon, top opening, freestanding, without rain cover: 21.0" wide x 15.0" deep x 32.0" high.

            - 24-gallon, top opening, freestanding, with rain cover: 21.0" wide x 15.0" deep x 40.5" high.

            - 24-gallon, top opening, surface mount, without rain cover: 21.0" wide x 15.0" deep x 32.5" high.

            - 24-gallon, top opening, surface mount, with rain cover: 21.0" wide x 15.0" deep x 41.0" high.

        - 36-gallon, single-stream receptacle

            - 36-gallon, top opening, freestanding, without rain cover: 32.0" wide x 15.0" deep x 32.0" high.

            - 36-gallon, top opening, freestanding, with rain cover: 32.0" wide x 15.0" deep x 40.5" high.

            - 36-gallon, top opening, surface mount, without rain cover: 32.0" wide x 15.0" deep x 32.5" high.

            - 36-gallon, top opening, surface mount, with rain cover: 32.0" wide x 15.0" deep x 41.0" high.

        - 36-gallon, split-stream litter & recycling receptacle:

            - 36-gallon, top opening, freestanding, without rain cover: 32.0" wide x 15.0" deep x 32.0" high.

            - 36-gallon, top opening, freestanding, with rain cover: 32.0" wide x 15.0" deep x 40.5" high.

            - 36-gallon, top opening, surface mount, without rain cover: 32.0" wide x 15.0" deep x 32.5" high.

            - 36-gallon, top opening, surface mount, with rain cover: 32.0" wide x 15.0" deep x 41.0" high.

      - Lid openings:

        - Single-stream receptacle:

            - 24-gallon, single-stream lid opening: 15" wide x 5" deep.

        - Split-stream receptacle:

            - 36-gallon, split-stream lid openings: two openings of 12.4" wide x 5" deep.

    - Instructional Graphics:

      - Apply instructional graphics to lids as specified to indicate the intended waste or recycling stream.

        - Graphics type: back-printed polycarbonate.

        - Letters and symbols color: White.

        - Graphics background colors:

            - Black.

            - Blue.

            - Green.

6.		Installation (Select one below):

      - Freestanding: levelers provided.

      - Surface mount: levelers provided. Stainless steel anchors and mounting bolts sold separately.

Part 3		Execution

- Examination

  - Verify that substrates are stable and capable of supporting the weight of items covered under this section.

  - Verify that substrates have been adequately prepared to securely anchor those items that will be surface

mounted.

- Installation

  - Install according to the manufacturer’s installation instructions.

  - Install in conformance to applicable ADA guidelines and End User’s established accessibility policies.

END OF SECTION