DIVISION 12 00 00					FURNISHINGS

SECTION 12 93 00					SITE FURNISHINGS

SECTION 12 93 43					SITE SEATING AND TABLES

SECTION 12 93 43.13			SITE SEATING

SOLERIS SUNSHADE

Part 1		General

- Summary

  - Section Includes:

    - Sunshade, aluminum shade panels, aluminum frame, stainless steel pole, and mounting assembly.

- Submittals:

  - Provide submittals in accordance with Section 01 33 00 – Submittal Procedures.

  - Product data:

    - Manufacturer’s standard product literature.

    - Shop drawings.

    - Installation instructions.

    - Maintenance instructions.

  - Submit powdercoat finish samples for approval.

- Quality Assurance:

  - Manufacturer Qualifications:

    - Minimum 15 years experience in the manufacture of site seating.

    - Provide reference list of at least ten major transportation authorities, municipalities, universities, or other high-use public environments currently using site seating fabricated by the manufacturer.

- Delivery, Storage and Handling:

  - Handle products in accordance with manufacturer’s instructions.

  - Store products in manufacturer’s original packaging until ready for installation.

  - Protect products from impacts and abrasion during storage.

- Warranty:

  - Provide manufacturer’s standard warranty.

    - Warranty terms: three years from date of invoice against defects in materials and workmanship.

Part 2		Products

- Manufacturer

  - Basis-of-design product: 	provide sunshade based on the product named:

    - Soleris Sunshade by Forms+Surfaces.

    - Manufacturer Contact:
Forms+Surfaces
1901 William Flynn Highway 
Glenshaw, PA 15116
phone: 800-451-0410

fax: 412-385-4715

email: sales@forms-surfaces.com

website: www.forms-surfaces.com

  - Sunshade:

    - Materials:

      - Shade panels: Constructed of 0.06" aluminum. (Choose one option below):

        - Solid shade panels.

        - Shade panels with Slat Perforation.

b.		Frame: cast and extruded aluminum.

c. 		Fasteners:  stainless steel.

d.		Mounting Pole: 1.75" diameter stainless steel tubing.

e.		Mounting pole sleeve: 2.0" diameter stainless steel tubing.

f.			Locking pin: stainless steel.

g.		Base Plate (Choose one option below):

1)	Standalone or with tabletop: 0.50" stainless steel base plate, 14.5" diameter.

2)  Apex Table Ensemble, 0.50” stainless steel plate, 11”x11” square.

3)  Knight Table Ensemble, 0.50” stainless steel base plate, 11”x11” square.

4)	Tangent Table Ensemble, 4-seat: 0.50" stainless steel base plate, 9.25" diameter.

5)	Tangent Table Ensemble, 6-seat: 0.50" stainless steel base plate, 9.25" diameter.

6)  Trio Table Ensemble, 0.50” stainless steel base plate, 11”x11” square.

7) 	Other application as specified.

h.		Anchoring hardware: Stainless steel mounting screws.

    - Finishes:

      - Shade panels: Polyester powdercoat (Select one option below):

        - Standard Texture/ Gloss from Forms+Surfaces Powdercoat Chart.

        - Custom RAL powdercoat option.

      - Frame and mounting assembly: Polyester powdercoat (Select one option below):

        - Standard Texture from Forms+Surfaces Powdercoat Chart.

        - Custom RAL powdercoat option.

      - Mounting Pole:  Satin finish.

    - Dimensions:

      - Standalone:  96.9" wide x 86.0" deep x 92.7" high.

      - Apex Table Ensemble: 96.9" wide x 83.0" deep x 86.7" high.

      - Knight Table Ensemble: 96.9" wide x 83.0" deep x 86.7" high.

      - Tangent Table Ensemble: 96.9" wide x 86.0" deep x 86.7" high.

      - Trio Table Ensemble: 96.9" wide x 86.0" deep x 86.7" high.

      - Vector Table Ensemble: 96.9" wide x 86.0" deep x 86.7" high.

      - With round tabletop: 96.9" wide x 86.0" deep x 86.7" - 92.7” high.

      - With square tabletop: 96.9" wide x 83.0" deep x 86.7" high.

    - Installation:

      - Surface mount: Stainless steel anchors and mounting bolts are sold separately.

Part 3		Execution

- Examination

  - Verify that substrates are stable and capable of supporting the weight of items covered under this section.

  - Verify that substrates have been adequately prepared to securely anchor those items that will be surface mounted.

- Installation

  - Install according to the manufacturer’s installation instructions.

  - Install in conformance to applicable ADA guidelines and End User’s established Accessibility policies.

END OF SECTION