DIVISION 26 00 00					ELECTRICAL

SECTION 26 50 00 				LIGHTING

SECTION 26 56 00 				EXTERIOR LIGHTING

TRIO PEDESTRIAN

Part 1		General

- Summary

  - Section Includes:

    - Trio Pedestrian.

- Submittals

  - Provide submittals in accordance with Section 01 33 00 – Submittal Procedures.

  - Product data:

    - Manufacturer’s standard product literature.

    - Shop drawings.

    - Installation instructions.

    - Maintenance instructions.

  - Submit powdercoat finish samples for approval.

- Quality Assurance

  - Manufacturer Qualifications

    - Minimum 15 years experience in the manufacture of outdoor lighting.

    - Provide reference list of at least ten major transportation authorities, municipalities, universities, or

other high-use public environments currently using outdoor lighting fabricated by the manufacturer.

- Delivery, Storage and Handling

  - Handle products in accordance with manufacturer’s instructions.

  - Store products in manufacturer’s original packaging until ready for installation.

  - Protect products from impacts and abrasion during storage.

- Warranty

  - Provide manufacturer’s standard warranty:

    - Warranty terms: three years from date of invoice against defects in materials and workmanship.

Part 2		Products

- Manufacturer

  - Basis-of-design product: provide pedestrian(s) based on the product named:

    - Trio Pedestrian by Forms+Surfaces

    - Manufacturer Contact:
Forms+Surfaces
1901 William Flynn Highway 
Glenshaw, PA 15116
phone: 800-451-0410

fax: 412-385-4715

email: sales@forms-surfaces.com

website: www.forms-surfaces.com

  - Pedestrian Lighting:

    - Materials:

      - Body: formed aluminum extrusion and cast aluminum head.

      - Lens: white acrylic.

      - LED Driver: Input power is 120-277 VAC. Includes high efficiency, constant output current, with over-voltage, short circuit, overload protection, and has 0-10V dimming capabilities. Certifications include: IP66 (waterproof) enclosure, and Class 2 rated output (UL8750).

      - LED Light Engine (Select color temperature below):

        - 40W custom LED light engine

            - Luminaire lumens: 3197

            - Color temperature

              - 3,000K

              - 4,000K

      - Hardware: stainless steel.

    - Finish: polyester powdercoat (Select one option below):

      - Standard Texture from Forms+Surfaces Powdercoat Chart.

      - Custom RAL powdercoat color.

2.	Dimensions:

      - 135" high x 7.7" wide x 7.7" deep.

    - Weight:

      - 73 lbs.

    - Mounting: Surface mount with J-bolts: 0.25” thick stainless steel base plate. Stainless steel mounting hardware sold separately. Template available upon request.

Part 3		Execution

- Examination

  - Verify that substrates are stable and capable of supporting the weight of items covered under this section.

  - Verify that substrates have been adequately prepared to securely anchor those items that will be surface

mounted.

- Installation

  - Install according to the manufacturer’s installation instructions.

  - Install in conformance to applicable ADA guidelines and End User’s established accessibility policies.

END OF SECTION