SECTION 09 70 00

WALL FINISHES

- GENERAL

Summary

Section Includes: Pre-engineered wall panel and column system including wall panels, mounting extrusions, bases and frame. Wall panel system comes “ready to install” with fasteners, adhesives, and other materials required for a complete assembly. Fasteners shall be concealed type. Installer must provide mounting hardware appropriate for substrate.

Related Sections:

Section 14 27 00 – Elevators

Section 05 58 13 – Column Covers

References

ASTM E84 – Surface Burning Characteristics of Building Materials.

Submittals

Provide submittals in accordance with Section 01 30 50 – Submittal Procedures.

Submit manufacturer’s shop drawings, installation drawings, installation instructions and maintenance instructions.

Submit environmental impact data for all materials.

Submit samples no less than 4" x 4" for all specified material finishes.

Submit panel edge extrusion samples no less than 4" of specified finish.

Submit mock-up of wall system no less than 12" x 12".

Manufacturer information:

Provide overview literature describing manufacturer’s overall scope of products and manufacturing capabilities.

Provide URL for manufacturer’s web site; web site must provide access to technical data, images and general product information.

Quality Assurance

Manufacturer Qualifications

Minimum 10 years experience in the manufacture of architectural surface materials

Minimum 10 years experience in the fabrication of wall systems.

Provide reference list of at least 20 public space projects currently using walls fabricated by the manufacturer.

Installer Qualifications

Minimum three years experience in the installation of wall systems.

Delivery, Storage and Handling

Deliver materials to installation site in manufacturer’s original packaging. Handle products in accordance with manufacturer’s instructions. Store in dry, secure location, protected against direct sunlight and excessive heat. Protect finished surfaces with strippable film.

Warranty

Provide manufacturer’s standard warranty.

Warranty terms: one year against defects in materials and workmanship.

PRODUCT

Manufacturer

Forms+Surfaces
1901 William Flynn Highway
Glenshaw, PA 15116
Phone: 800-451-0410
Fax: 412-385-3715
Email: sales@forms-surfaces.com
Website: www.forms-surfaces.com

Wall Panels

General

Provide interlocking grid panel system with inset panels mounted to extruded aluminum frames on structural backer. The recommended substrate is 3/4" fire-rated OSB or plywood.

Panel configuration: Forms+Surfaces LEVELe Wall Panel & Column System with Minimal, Capture, Blind or Float style panel frames. Refer to F+S product support documentation to verify inset material options available per panel style. See drawings for panel layout and reveal spacing

Provide inset panels in the finishes specified.

Weight per square foot: average 2.40 lbs to 3.04 lbs

Frames are extruded aluminum. Aluminum is inherently non-combustible.

Inset Materials

Stainless Steel Panels

Material: Stainless Steel

Finish: Sandstone, Seastone, Linen, Diamond, Mist, Mirror, Satin (choose one)

Impression Pattern: Champagne, Chardonnay, Dallas, Denver, Dune, Kalahari (choose one)

Eco-Etch Pattern: City Lights, Current, Dash, Flicker, Glacier, Mica, Reeds, Seagrass, Sequence, Silkworm, Stacks, Stripe, Thatch, Trace (choose one)

Fire rating: Stainless Steel is inherently non-combustible and generally considered to be NFPA and IBC Class A fire rated and UBC Class 1 fire rated.

Substrate: fire-rated structural backer

Treatment: Provide Ceramiloc+™ treatment, an invisible, protective treatment that significantly increases the fingerprint resistance and maintenance ease of stainless steel. Patented technology molecularly bonds nano-silica particles to the surface of stainless steel.

Fused Metal Panels

Material: Fused Bronze, Fused Graphite, Fused Nickel Bronze, Fused Nickel Silver, Fused White Gold (choose one)

Finish: Sandstone, Seastone, Linen, Diamond, Mirror, Satin (choose one)

Impression Pattern: Champagne, Chardonnay, Dallas, Denver, Dune, Kalahari (choose one)

Eco-Etch Pattern: City Lights, Current, Dash, Flicker, Glacier, Mica, Reeds, Seagrass, Sequence, Silkworm, Stacks, Stripe, Thatch, Trace (choose one)

Fire rating: Fused Metal is inherently non-combustible and generally considered to be NFPA and IBC Class A fire rated and UBC Class 1 fire rated.

Substrate: fire-rated structural backer

Treatment: Provide Ceramiloc+™ treatment, an invisible, protective treatment that significantly increases the fingerprint resistance and maintenance ease of Fused Metal. Patented technology molecularly bonds nano-silica particles to the surface of Fused Metal.

Bonded Metal Panels

Material: Bonded Aluminum, Bonded Bronze (choose one)

Pattern: Austin, Charleston, Dash, Delta, Grass, Herringbone, Loft, Mara, Rain, Trevia, Vancouver, Undercurrent (choose one)

Patina: Natural, Dark (choose one)

Fire rating: Bonded Metal is NFPA and IBC Class B fired rated and UBC class 2 fire rated.

Substrate: structural backer

Bonded Quartz Panels

Material: Bonded White, Bonded Charcoal (choose one)

Pattern: Crinkle, Current, Dash, Grass, Herringbone, Loft, Mara, Trevia (choose one)

Fire rating: Bonded Quartz is NFPA and IBC Class B fired rated and UBC class 2 fire rated.

Substrate: structural backer

Laminated Glass Panels

Material: VividGlass: ViviChrome, ViviGraphix Gleam, ViviGraphix Graphica, ViviMuse Painterly, ViviMuse Shibori, ViviSpectra Spectrum, ViviSpectra VEKTR, ViviSpectra Zoom, ViviStone, ViviStrata, ViviTela Loom, ViviTela Mesh, Scribe Glass (choose one)

Material: CastGlass: Chromis, Elements (choose one)

Fire rating: VividGlass glass is Class A fire rated in accordance with ASTM designation E84-09, standard test method of surface burning characteristics of building materials. The foregoing test procedure is comparable to UL 723, ANSI/NFPA No. 255, and UBC No. 8-1.

Illuminated Glass Panels

Material: VividGlass: ViviChrome, ViviGraphix Gleam, ViviGraphix Graphica, ViviMuse Shibori, ViviSpectra Spectrum, ViviSpectra VEKTR, ViviSpectra Zoom, ViviStone, ViviStrata (choose one)

Wood Veneer Panels

Material: Champagne, Cherry, Ebony, Walnut, White Oak Plain, White Oak Rift (choose one)

Aluminum Panels

Material: Aluminum

Base Materials

Stainless Steel

Material: Stainless Steel

Finish: Sandstone, Seastone, Linen, Diamond, Mist, Mirror, Satin (choose one)

Fused Metal

Material: Fused Bronze, Fused Graphite, Fused Nickel Bronze, Fused Nickel Silver, Fused White Gold (choose one)

Finish: Sandstone, Seastone, Linen, Diamond, Mirror, Satin (choose one)

Panel Frames

Material: Extruded, anodized aluminum

Finish: Clear, Black or Antique Bronze anodized (choose one)

Corners and End Stops

Material: Extruded, anodized aluminum

Finish: Clear, Black or Antique Bronze anodized (choose one)

Reveals

Material: Surface applied colored reveal strip

Finish: Black, Brushed Chrome, Polished Chrome (choose one)

EXECUTION

Preparation

Protect column finishes, fixtures and equipment from damage caused by work of this Section.

Installation

Install in accordance with column system manufacturer’s instructions.

Cleaning and Protection

Remove strippable film. Clean exposed surfaces in accordance with manufacturer’s instructions.

Protect exposed surfaces from damage by subsequent construction.

- END OF SECTION