DIVISION 12 00 00					FURNISHINGS

SECTION 12 93 00					SITE FURNISHINGS

SECTION 12 93 43					SITE SEATING AND TABLES

SECTION 12 93 43.13			SITE SEATING

CAMBER BENCH

Part 1		General

- Summary

  - Section Includes:

    - Backed benches, 6 foot, cast aluminum frame, extruded aluminum slats.

    - Backed benches, 6 foot, cast aluminum frame, FSC 100% Cumaru hardwood slats.

    - Backed benches, 8 foot, cast aluminum frame, extruded aluminum slats.

- Submittals

  - Provide submittals in accordance with Section 01 33 00 – Submittal Procedures.

  - Product data:

    - Manufacturer’s standard product literature.

    - Shop drawings.

    - Installation instructions.

    - Maintenance instructions.

  - Submit powdercoat finish samples for approval.

- Quality Assurance

  - Manufacturer Qualifications:

    - Minimum 15 years experience in the manufacture of site seating.

    - Forest Stewardship Council (“FSC”) Certified Supplier. Provide manufacturer’s FSC certification number.

    - Provide reference list of at least ten major transportation authorities, municipalities, universities, or other high-use public environments currently using site seating fabricated by the manufacturer.

- Delivery, Storage and Handling

  - Handle products in accordance with manufacturer’s instructions.

  - Store products in manufacturer’s original packaging until ready for installation.

  - Protect products from impacts and abrasion during storage.

- Warranty

  - Provide manufacturer’s standard warranty:

    - Warranty terms: three years from date of invoice against defects in materials and workmanship.

Part 2		Products

- Manufacturer

  - Basis-of-design product: 	provide benches based on the product named:

    - Camber Bench by Forms+Surfaces.

    - Manufacturer Contact:
1901 William Flynn Highway 
Glenshaw, PA 15116
Pittsburgh, PA 15223
phone: 800-451-0410

fax: 412-385-4715

email: sales@forms-surfaces.com

website: www.forms-surfaces.com

  - Benches

    - Materials:

      - Bench frames: cast aluminum with integral armrests.

      - Intermediate armrests: cast aluminum. (optional)

      - Bench slats (Choose one below):

        - Extruded aluminum.

        - FSC 100% Cumaru hardwood.

      - Fasteners: stainless steel.

    - Finishes:

      - Bench frames: polyester powdercoat (Select one option below):

        - Standard Texture from Forms+Surfaces Powdercoat Chart.

        - Custom RAL powdercoat color.

      - Intermediate armrests (optional):

        - Polyester powdercoat color to match bench frame.

      - Slats (Choose one below):

        - Aluminum: anodized finish or polyester powdercoat (Choose one below):

            - Anodized finish: Clear Anodized.

            - Polyester powdercoat: Slate Texture.

        - Wood: penetrating oil finish.

            - Finish color: Penofin® hardwood formula “Transparent Natural.”

    - Dimensions: (Choose model(s) below):

      - 6 foot bench:

        - Overall: 77.3" long x 27.2" deep x 32.8" high.

        - Seat: 17.3" high x 21.0" deep.

        - Armrest: 22.8" high.

      - 8 foot bench:

        - Overall: 101.3" long x 27.2" deep x 32.8" high.

        - Seat: 17.3" high x 21.0" deep.

        - Armrest: 22.8" high.

    - Mounting (Choose one below):

      - Freestanding.

      - Surface mount. Stainless steel anchors and mounting bolts sold separately.

Part 3		Execution

- Examination

  - Verify that substrates are stable and capable of supporting the weight of items covered under this section.

  - Verify that substrates have been adequately prepared to securely anchor those items that will be surface mounted.

- Installation

  - Install according to the manufacturer’s installation instructions.

  - Install in conformance to applicable ADA guidelines and End User’s established Accessibility policies.

END OF SECTION