DIVISION 12 00 00					FURNISHINGS

SECTION 12 93 00 				SITE FURNISHINGS

SECTION 12 93 23 				TRASH AND LITTER RECEPTORS

URBAN RENAISSANCE RECEPTACLE

Part 1		General

- Summary

  - Section Includes: (Choose model below):

    - 36-gallon receptacle, top opening

    - 45-gallon receptacle, top opening

    - 36-gallon receptacle, side opening with dome lid

    - 45-gallon receptacle, side opening with dome lid

    - 36-gallon receptacle, side opening with integrated recycle bin

    - 36-gallon receptacle, side opening, split-stream with dome lid

    - 45-gallon receptacle, side opening, split-stream with dome lid

- Submittals

  - Provide submittals in accordance with Section 01 33 00 – Submittal Procedures.

  - Product data:

    - Manufacturer’s standard product literature.

    - Shop drawings.

    - Installation instructions.

    - Maintenance instructions.

  - Submit powdercoat finish samples for approval.

- Quality Assurance

  - Manufacturer Qualifications:

    - Minimum 15 years experience in the manufacture of litter and recycling receptacles.

    - Provide reference list of at least ten major transportation authorities, municipalities, universities, or

other high-use public environments currently using litter and recycling receptacles fabricated by the manufacturer.

- Delivery, Storage and Handling

  - Handle products in accordance with manufacturer’s instructions.

  - Store products in manufacturer’s original packaging until ready for installation.

  - Protect products from impacts and abrasion during storage.

- Warranty

  - Provide manufacturer’s standard warranty:

    - Warranty terms: three years from date of invoice against defects in materials and workmanship.

Part 2		Products

- Manufacturer

  - Basis-of-design product: provide receptacles based on the product named:

    - Urban Renaissance Receptacle by Forms+Surfaces

    - Manufacturer Contact:
Forms+Surfaces
1901 William Flynn Highway 
Glenshaw, PA 15116
phone: 800-451-0410

fax: 412-385-4715

email: sales@forms-surfaces.com

website: www.forms-surfaces.com

  - Receptacles

    - Materials:

      - Frame: square tubular stainless steel.

      - Grillwork: stainless steel sheet with perforated designs (Choose one design below):

        - Updrop

        - Vertical

        - Vista

      - Lids: spun aluminum with internal structural frame.

      - Liner: black polyethylene with UL94HB fire-rating.

      - Latch, hinges and hinge pins: Stainless Steel.

    - Finish:

      - Polyester powdercoat (Select one option below):

        - Standard Texture/Gloss from Forms+Surfaces Powdercoat Chart.

        - Premium Texture from Forms+Surfaces Powdercoat Chart.

        - Custom RAL powdercoat color.

    - Dimensions (Choose model below):

      - 36-gallon receptacle, top opening: 26.5" diameter x 39.5" high.

      - 45-gallon receptacle, top opening: 28.5" diameter x 44" high.

      - 36-gallon receptacle, side opening with dome lid: 26.5" diameter x 42.3" high.

      - 45-gallon receptacle, side opening with dome lid: 28.5" diameter x 47" high.

      - 36-gallon receptacle, side opening with integrated recycle bin: 26.5" diameter x 48.3" high.

      - 36-gallon receptacle, side opening, split-stream with dome lid: 26.5" diameter x 43.4" high.

      - 45-gallon receptacle, side opening, split-stream with dome lid: 28.3" diameter x 47.4" high.

    - Installation (Select one below):

      - Freestanding: levelers included.

      - Surface mount: levelers included. Stainless steel anchors and mounting screws sold separately.

Part 3		Execution

- Examination

  - Verify that substrates are stable and capable of supporting the weight of items covered under this section.

  - Verify that substrates have been adequately prepared to securely anchor those items that will be surface

mounted.

- Installation

  - Install according to the manufacturer’s installation instructions.

  - Install in conformance to applicable ADA guidelines and End User’s established Accessibility policies.

END OF SECTION