DIVISION 12 00 00					FURNISHINGS

SECTION 12 93 00					SITE FURNISHINGS

SECTION 12 93 43					SITE SEATING AND TABLES

SECTION 12 93 43.13			SITE SEATING

CIRCUIT BENCH

Part 1		General

- Summary

  - Section Includes:

    - Six-foot, backless benches with Powdercoated cast aluminum frames and FSC Recycled 100% Teak hardwood slats.

- Submittals

  - Provide submittals in accordance with Section 01 33 00 – Submittal Procedures.

  - Product data:

    - Manufacturer’s standard product literature.

    - Shop drawings.

    - Installation instructions.

    - Maintenance instructions.

  - Submit powdercoat finish samples for approval.

- Quality Assurance

  - Manufacturer Qualifications:

    - Minimum 15 years experience in the manufacture of site seating.

    - Forest Stewardship Council (FSC) Certified Supplier. Provide manufacturer’s FSC certification number.

    - Provide reference list of at least ten major transportation authorities, municipalities, universities, or other high-use public environments currently using site seating fabricated by the manufacturer.

- Delivery, Storage and Handling

  - Handle products in accordance with manufacturer’s instructions.

  - Store products in manufacturer’s original packaging until ready for installation.

  - Protect products from impacts and abrasion during storage.

- Warranty

  - Provide manufacturer’s standard warranty.

    - Warranty terms: three years from date of invoice against defects in materials and workmanship.

Part 2		Products

- Manufacturer

  - Basis-of-design product: 	provide backless benches based on the product named:

    - Circuit Bench by Forms+Surfaces.

    - Manufacturer Contact:
Forms+Surfaces
1901 William Flynn Highway 
Glenshaw, PA 15116
phone: 800-451-0410

fax: 412-385-4715

email: sales@forms-surfaces.com

website: www.forms-surfaces.com

  - Backless Benches

    - Materials:

      - Bench frames: cast aluminum.

      - Seats

        - FSC Recycled 100% Teak hardwood slats.

      - Fasteners: stainless steel.

    - Finishes:

      - Bench frames: polyester powdercoat (Choose one color below):

        - Standard Texture/Gloss from Forms+Surfaces Powdercoat Chart.

        - Custom RAL Powdercoat color.

      - Seats (Choose one below):

        - FSC Recycled 100% Teak: natural oiled finish.

      - Nominal dimensions: 72" long x 18" deep x 18" high.

    - Mounting (Choose one option below):

      - Freestanding: Provide nylon glides.

      - Surface mount: Stainless steel anchors and mounting screws are sold separately.

Part 3		Execution

- Examination

  - Verify that substrates are stable and capable of supporting the weight of items covered under this section.

  - Verify that substrates have been adequately prepared to securely anchor those items that will be surface mounted.

- Installation

  - Install according to the manufacturer’s installation instructions.

  - Install in conformance to applicable ADA guidelines and End User’s established Accessibility policies.

- END OF SECTION