DIVISION 12 00 00					FURNISHINGS

SECTION 12 93 00 				SITE FURNISHINGS

SECTION 12 93 23 				TRASH AND LITTER RECEPTORS

TONYO RECEPTACLE – 36-GALLON

Part 1		General

- Summary

  - Section Includes:

    - 36-gallon, single-stream receptacle with FSC 100% Cumaru hardwood body/door

    - 36-gallon, single-stream receptacle with powdercoated stainless steel body/door

    - 36-gallon, split-stream receptacle with FSC 100% Cumaru hardwood body/door

    - 36-gallon, split-stream receptacle with powdercoated stainless steel body/door

- Submittals

  - Provide submittals in accordance with Section 01 33 00 – Submittal Procedures.

  - Product data:

    - Manufacturer’s standard product literature.

    - Shop drawings.

    - Installation instructions.

    - Maintenance instructions.

- Quality Assurance

  - Manufacturer Qualifications:

    - Minimum 15 years experience in the manufacture of litter and recycling receptacles.

    - Provide reference list of at least ten major transportation authorities, municipalities, universities, or

other high-use public environments currently using litter and recycling receptacles fabricated by the manufacturer.

- Delivery, Storage and Handling

  - Handle products in accordance with manufacturer’s instructions.

  - Store products in manufacturer’s original packaging until ready for installation.

  - Protect products from impacts and abrasion during storage.

- Warranty

  - Provide manufacturer’s standard warranty.

    - Warranty terms: three years from date of invoice against defects in materials and workmanship.

Part 2		Products

- Manufacturer

  - Basis-of-design product: provide Tonyo Receptacle(s) based on the product named:

    - Tonyo Receptacle by Forms+Surfaces

    - Manufacturer Contact:
Forms+Surfaces
1901 William Flynn Highway 
Glenshaw, PA 15116
phone: 800-451-0410

fax: 412-385-4715

email: sales@forms-surfaces.com

website: www.forms-surfaces.com

  - Receptacles

    - Materials:

      - Internal frame: stainless steel.

      - Sidebars: extruded aluminum.

      - Lid & base: corrosion-resistant cast aluminum.

      - Rain hat (optional): corrosion-resistant cast aluminum.

      - Body & door (Select one option below):

        - Stainless steel.

        - FSC 100% Cumaru hardwood slats with cast aluminum supports.

      - Liner(s): black polyethylene.

      - Hardware & latch: stainless steel.

    - Finishes:

      - Body & door (and lid when applicable) (Select one option below):

        - Polyester powdercoat (Select one option below):

            - Standard Texture from Forms+Surfaces Powdercoat Chart.

            - Custom RAL powdercoat color.

        - FSC 100% Cumaru hardwood slats: Penofin® hardwood formula “Transparent Natural.”

      - Base, sidebars, optional rain hat (and lid when applicable):

        - Polyester powdercoat (Select one option below):

            - Standard Texture from Forms+Surfaces Powdercoat Chart.

            - Custom RAL powdercoat color.

    - Body/Door perforation pattern (for stainless steel body/door only) (Select one option below):

      - No perforation

      - Notch perforation

      - Plait perforation

      - Terra perforation

      - Warp perforation

    - Latch: (Select one option below):

      - Standard lift latch.

      - Limited-access latch (operated by flathead screwdriver or similar).

    - Overall Dimensions (Select one configuration below):

      - 36-gallon, single-stream receptacle, FSC 100% Cumaru hardwood body/door, with rain hat: 23.1" dia. x 47.1" high

      - 36-gallon, single-stream receptacle, FSC 100% Cumaru hardwood body/door, without rain hat: 23.1" dia. x 38.5" high

      - 36-gallon, single-stream receptacle with powdercoated stainless steel body/door, with rain hat: 23.1" dia. x 47.1" high

      - 36-gallon, single-stream receptacle with powdercoated stainless steel body/door, without rain hat: 23.1" dia. x 38.5" high

      - 36-gallon, split-stream receptacle with FSC 100% Cumaru hardwood body/door, with rain hat: 23.1" dia. x 47.1" high

      - 36-gallon, split-stream receptacle with FSC 100% Cumaru hardwood body/door, without rain hat: 23.1" dia. x 38.5" high

      - 36-gallon, split-stream receptacle with powdercoated stainless steel body/door, with rain hat: 23.1" dia. x 47.1" high

      - 36-gallon, split-stream receptacle with powdercoated stainless steel body/door, without rain hat: 23.1" dia. x 38.5" high

    - Instructional Graphics:

      - Apply instructional graphics to lids as specified to indicate the intended waste or recycling stream.

      - Graphics type: back-printed polycarbonate.

      - Letters and symbols color: White.

      - Graphics background colors:

            - Black.

            - Blue.

            - Green.

    - Installation (Select one below):

      - Freestanding: levelers included.

      - Surface mount: levelers included. Stainless steel anchors and mounting screws sold separately.

Part 3		Execution

- Examination

  - Verify that substrates are stable and capable of supporting the weight of items covered under this section.

  - Verify that substrates have been adequately prepared to securely anchor those items that will be surface

mounted.

- Installation

  - Install according to the manufacturer’s installation instructions.

  - Install in conformance to applicable ADA Guidelines and End User’s established Accessibility policies.

END OF SECTION