DIVISION 12 00 00					FURNISHINGS

SECTION 12 93 00					SITE FURNISHINGS

SECTION 12 93 43					SITE SEATING AND TABLES

SECTION 12 93 43.13			SITE SEATING

BOARDWALK BENCH

Part 1		General

- Summary

  - Section Includes:

    - Standalone, backless bench, 6 foot, extruded aluminum frame, cast aluminum ends, and FSC® Recycled 100% Cumaru hardwood seat slats. (FSC License Code: FSC-C004453)

- Submittals

  - Provide submittals in accordance with Section 01 33 00 – Submittal Procedures.

  - Product data:

    - Manufacturer’s standard product literature.

    - Shop drawings.

    - Installation instructions.

    - Maintenance instructions.

  - Submit powdercoat finish samples for approval.

- Quality Assurance

  - Manufacturer Qualifications:

    - Minimum 15 years experience in the manufacture of site seating.

    - Provide reference list of at least ten major transportation authorities, municipalities, universities, or other high-use public environments currently using site seating fabricated by the manufacturer.

- Delivery, Storage and Handling

  - Handle products in accordance with manufacturer’s instructions.

  - Store products in manufacturer’s original packaging until ready for installation.

  - Protect products from impacts and abrasion during storage.

- Warranty

  - Provide manufacturer’s standard warranty:

    - Warranty terms: three years from date of invoice against defects in materials and workmanship.

Part 2		Products

- Manufacturer

  - Basis-of-design product: 	provide backless bench(es) based on the product named:

    - Boardwalk Bench by Forms+Surfaces

    - Manufacturer Contact:
Forms+Surfaces
30 Pine Street
Pittsburgh, PA 15223
phone: 800-451-0410

fax: 412-385-4715

email: sales@forms-surfaces.com

website: www.forms-surfaces.com

  - Benches

    - Materials:

      - Bench frame: aluminum frame with cast-aluminum ends.

      - Seats: FSC Recycled 100% Cumaru hardwood slats.

      - Fasteners: stainless steel.

      - Optional intermediate seat dividers: solid aluminum.

    - Finishes:

      - Frames and optional seat dividers: polyester powdercoat (Select one option below):

        - Standard Texture from Forms+Surfaces Powdercoat Chart.

        - Custom RAL powdercoat color.

      - Seats: natural oiled finish.

    - Dimensions:

      - Freestanding bench without seat dividers: 72.2" long x 16.0" deep x 18.25" high.

      - Surface mount bench without seat dividers: 72.2" long x 16.0" deep x 18.44" high.

      - Freestanding bench with intermediate seat dividers:

        - Overall dimensions: 72.2" long x 16.0" deep x 20.55" high.

        - Seat dimensions: 22.0" long x 16.0" deep x 20.55" high.

        - Seat divider dimensions: 1.0" long x 15.0" deep x 2.3" high.

      - Surface mount bench with intermediate seat dividers:

        - Overall dimensions: 72.2" long x 16.0" deep x 20.7" high.

        - Seat dimensions: 22.0" long x 16.0" deep x 20.7" high.

        - Seat divider dimensions: 1.0" long x 15.0" deep x 2.3" high.

    - Mounting:

      - Freestanding.

      - Surface mount. Mounting plates are provided, stainless steel anchors and mounting hardware is sold separately.

Part 3		Execution

- Examination

  - Verify that substrates are stable and capable of supporting the weight of items covered under this section.

  - Verify that substrates have been adequately prepared to securely anchor those items that will be surface mounted.

- Installation

  - Install according to the manufacturer’s installation instructions.

  - Install in conformance to applicable ADA guidelines and End User’s established Accessibility policies.

END OF SECTION