DIVISION 12 00 00					FURNISHINGS

SECTION 12 93 00 				SITE FURNISHINGS

SECTION 12 93 43 				SITE SEATING AND TABLES

SECTION 12 93 43.13 		SITE SEATING

AVIVO CHAIR

Part 1		General

- Summary

  - Section Includes:

    - Chairs without armrests.

    - Chairs with armrests.

- Submittals

  - Provide submittals in accordance with Section 01 33 00 – Submittal Procedures.

  - Product data:

    - Manufacturer’s standard product literature.

    - Shop drawings.

    - Maintenance instructions.

  - Submit powdercoat finish samples for approval.

- Quality Assurance

  - Manufacturer Qualifications:

    - Minimum 15 years experience in the manufacture of site seating.

    - Provide reference list of at least ten major transportation authorities, municipalities, universities, or

other high-use public environments currently using site seating fabricated by the manufacturer.

- Delivery, Storage and Handling

  - Handle products in accordance with manufacturer’s instructions.

  - Store products in manufacturer’s original packaging until ready for installation.

  - Protect products from impacts and abrasion during storage.

- Warranty

  - Provide manufacturer’s standard warranty.

    - Warranty terms: three years from date of invoice against defects in materials and workmanship.

Part 2		Products

- Manufacturer

  - Basis-of-design product: provide chairs and/or armchairs based on the product named:

    - Avivo Chair by Forms+Surfaces.

    - Manufacturer Contact:
Forms+Surfaces
1901 William Flynn Highway 
Glenshaw, PA 15116
phone: 800-451-0410

fax: 412-385-4715

email: sales@forms-surfaces.com

website: www.forms-surfaces.com

  - Chairs

    - Materials:

      - Chair frames, armrests: cast aluminum.

      - Seats and seat backs: 1/8" formed aluminum.

      - Bumpers: UV-stabilized, wear resistant polypropylene.

      - Glides: Nylon.

    - Finishes:

      - Polyester powdercoat (Select one option below):

        - Standard Texture from Forms+Surfaces Powdercoat Chart.

        - Custom RAL powdercoat color.

      - Seat and seat back perforation options (Select one option below):

        - No perforation.

        - Riva perforation pattern.

    - Dimensions (Select model(s) below):

      - Chair without armrests: 19" overall width x 23" overall depth x 32.3" overall height. 16.5” seat width, 19" seat height, 19” seat depth.

      - Chair with armrests: 22" overall width x 23" overall depth x 32.3" overall height. 16.5” seat width, 19" seat height, 19” seat depth, 24.5" armrest height.

    - Installation: Freestanding.

Part 3		Execution

- Examination

  - Verify that substrates are stable and capable of supporting the weight of items covered under this section.

END OF SECTION