DIVISION 26 00 00					ELECTRICAL

SECTION 26 50 00 				LIGHTING

SECTION 26 56 00 				EXTERIOR LIGHTING

KNIGHT BOLLARD

Part 1		General

- Summary

  - Section Includes:

    - Knight Bollard.

- Submittals

  - Provide submittals in accordance with Section 01 33 00 – Submittal Procedures.

  - Product data:

    - Manufacturer’s standard product literature.

    - Shop drawings.

    - Installation instructions.

    - Maintenance instructions.

  - Submit powdercoat finish samples for approval.

- Quality Assurance

  - Manufacturer Qualifications:

    - Minimum 15 years experience in the manufacture of outdoor lighting.

    - Provide reference list of at least ten major transportation authorities, municipalities, universities, or

other high-use public environments currently using outdoor lighting fabricated by the manufacturer.

- Delivery, Storage and Handling

  - Handle products in accordance with manufacturer’s instructions.

  - Store products in manufacturer’s original packaging until ready for installation.

  - Protect products from impacts and abrasion during storage.

- Warranty

  - Provide manufacturer’s standard warranty.

    - Warranty terms: three years from date of invoice against defects in materials and workmanship.

Part 2		Products

- Manufacturer

  - Basis-of-design product: provide bollards based on the product named.

    - Knight Bollard by Forms+Surfaces

    - Manufacturer Contact:
Forms+Surfaces
1901 William Flynn Highway 
Glenshaw, PA 15116
phone: 800-451-0410

fax: 412-385-4715

email: sales@forms-surfaces.com

website: www.forms-surfaces.com

  - Bollards

    - Materials

      - Column: extruded aluminum with rounded corners.

      - Head: aluminum quadrant casting with

      - Lamp housing: aluminum with four borosilicate glass lenses.

      - Driver (Choose one below):

        - 0-10 dimming: Input power is 122-277V. Includes high efficiency constant output current with over-voltage, short circuit, overload protection. Certifications include: IP66 (waterproof) enclosure and Class 2 rated output (UL 8750).

        - Phase dimming: Input power is 120V. Includes high efficiency constant output current with over-voltage, short circuit, overload protection. Certifications include: IP66 (waterproof) enclosure and Class 2 rated output (UL 8750).

      - Lamp

      - LED (Choose one lamp below):

            - 3000K

              - 16W custom LED light engine

              - Luminaire lumens: 362

              - Color temperature: 3,000K

            - 4000K

              - 16W custom LED light engine

              - Luminaire lumens: 362

              - Color temperature: 4,000K

    - Finishes

      - Column and lamp housing: polyester powdercoat (Select one option below):

        - Standard Texture from Forms+Surfaces Powdercoat Chart.

        - Premium Texture from Forms+Surfaces Powdercoat Chart.

        - Custom RAL powdercoat color.

      - Quadrant casting: Aluminum Texture polyester powdercoat with polished outer edges. All surfaces are clear coated.

    - Dimensions

      - Overall: 8" square x 43.1" high.

    - Mounting (Choose one below):

      - Standard:

        - Surface Mount with 3/16" thick x 7.5 " square base plate with J-bolt anchors. Stainless steel mounting hardware sold separately. Template available upon request.

      - Optional:

        - S10-P1 Embedded Deep Set Security Core.

        - SC30-P1 Embedded Shallow Mount Security Core.

Part 3		Execution

- Examination

  - Verify that substrates are stable and capable of supporting the weight of items covered under this section.

  - Verify that substrates have been adequately prepared to securely anchor those items that will be surface

mounted.

- Installation

  - Install according to the manufacturer’s installation instructions.

  - Install in conformance to applicable ADA guidelines and End User’s established Accessibility policies.

END OF SECTION