DIVISION 26 00 00					ELECTRICAL

SECTION 26 50 00 				LIGHTING

SECTION 26 56 00 				EXTERIOR LIGHTING

KNIGHT BOLLARD WITH EMBEDDED SECURITY CORE

Part 1		General

- Summary

  - Section Includes:

    - Knight Bollard with embedded security core.

- References

  - ASTM A 53 – Standard Specification for Pipe, Steel, Black, and Hot-Dipped, Zinc Coated, Welded and Seamless.

  - ASTM A 123 – Specification for Zinc (hot-dip galvanized) Coatings on Iron and Steel Products.

- Performance Requirements

  - Bollards must meet minimum level of S10-P1 or SC30-P1 as identified by the Standard Test Method for Vehicle Crash Testing of Perimeter Barriers. Manufacturer must provide documentation confirming that a finite element analysis has been completed for this product. A S10-P1 is equivalent to stopping a 5,000 lb. vehicle traveling at 10 mph in less than 1 meter and A SC30-P1 rating is equivalent to stopping a 2,430 lb. vehicle traveling at 30 mph in less than 1 meter.

- Submittals

  - Provide submittals in accordance with Section 01 33 00 – Submittal Procedures.

  - Product data: Manufacturer’s standard data sheets on each product to be used, including:

    - Manufacturer’s standard product literature.

    - Shop Drawings.

    - Installation instructions.

    - Maintenance instructions.

  - Submit finish samples for approval.

- Quality Assurance

  - Manufacturer Qualifications:

    - Minimum 15 years experience in the manufacture of outdoor lighting.

    - Provide reference list of at least ten major transportation authorities, municipalities, universities, or

other high-use public environments currently using site products fabricated by the manufacturer.

- Delivery, Storage and Handling

  - Handle products in accordance with manufacturer’s instructions.

  - Store products in manufacturer’s original packaging until ready for installation.

  - Protect products from impacts and abrasion during storage.

- Warranty

  - Provide manufacturer’s standard warranty:

    - Warranty terms: three years from date of invoice against defects in materials and workmanship.

Part 2		Products

- Manufacturer

  - Basis-of-design product: provide bollards based on the product named.

    - Knight Security Bollard by Forms+Surfaces

      - Manufacturer Contact:
Forms+Surfaces
1901 William Flynn Highway 
Glenshaw, PA 15116
phone: 800-451-0410

fax: 412-385-4715

email: sales@forms-surfaces.com

website: www.forms-surfaces.com

B. Cast Aluminum Lighted Security Bollards:

    - Materials:

      - Column: Cast aluminum.

      - Head: Aluminum quadrant casting

      - Lamp housing: Aluminum with four borosilicate glass lenses.

      - LED driver (Choose one below):

        - 0-10 dimming: Input power is 120-277V. Includes high efficiency constant output current with over-voltage, short circuit, overload protection. Certifications include: IP66 (waterproof) enclosure and Class 2 rated output (UL 8750).

        - Phase dimming: Input power is 120V. Includes high efficiency constant output current with over-voltage, short circuit, overload protection. Certifications include: IP66 (waterproof) enclosure and Class 2 rated output (UL 8750).

      - Lamps

        - LED (Choose one lamp below):

            - 3000K

              - 16W custom LED light engine

              - Luminaire lumens: 362

              - Color temperature: 3,000K

            - 4000K

              - 16W custom LED light engine

              - Luminaire lumens: 362

              - Color temperature: 4,000K

      - Mounting: cast in place extra heavy wall steel security core with welded cross supports.  48" Security core minimum 24" embed into concrete and minimum 24" internal to bollard.

      - Heavy steel wall security core set in 18" by 18" by 30" concrete footer minimum.

    - Finishes:

        - Column and lamp housing: polyester powdercoat (Select one option below):

            - Standard Texture from Forms+Surfaces Powdercoat Chart.

            - Custom RAL powdercoat color.

      - Quadrant casting: Aluminum Texture polyester powdercoat with polished outer edges. All surfaces are clear coated.

    - Dimensions: Column: 8-inch nominal square x 43.1" high.

    - Mounting (Choose one below):

      - S10-P1 Embedded Deep Set Security Core.

      - SC30-P1 Embedded Shallow Mount Security Core.

Part 3		Execution

  - Examination

- Verify that substrates are stable and capable of supporting the weight of items covered under this section.

- Verify that foundation, applied finishes, and adjacent construction are ready to receive bollards and are level, plumb, and square within tolerances acceptable to manufacturer.

- Verify that required utilities are in correct location and are of correct capacities for specified products.

  - Installation

- Install in accordance with manufacturer's installation instructions.

- Install in conformance to applicable ADA guidelines and End User’s established accessibility policies.

END OF SECTION