DIVISION 12 00 00					FURNISHINGS

SECTION 12 93 00					SITE FURNISHINGS

SECTION 12 93 43					SITE SEATING AND TABLES

SECTION 12 93 43.13			SITE SEATING

VECTOR TABLE ENSEMBLE

Part 1		General

- Summary

  - Section Includes:

    - Table ensemble with aluminum frame and FSC® 100% Cumaru hardwood slats. (FSC License Code: FSC-C004453) or extruded aluminum slats

    - Table ensemble with ADA option, cast aluminum frame, and FSC 100% Cumaru hardwood slats or extruded aluminum slats

- Submittals

  - Provide submittals in accordance with Section 01 33 00 – Submittal Procedures.

  - Product data:

    - Manufacturer’s standard product literature.

    - Shop drawings.

    - Installation instructions.

    - Maintenance instructions.

  - Submit powdercoat finish samples for approval.

- Quality Assurance

  - Manufacturer Qualifications:

    - Minimum 15 years experience in the manufacture of site furniture.

    - Forest Stewardship Council (“FSC”) Certified Supplier. Provide manufacturer’s FSC certification number.

    - Provide reference list of at least ten major transportation authorities, municipalities, universities, or other high-use public environments currently using site seating fabricated by the manufacturer.

- Delivery, Storage and Handling

  - Handle products in accordance with manufacturer’s instructions.

  - Store products in manufacturer’s original packaging until ready for installation.

  - Protect products from impacts and abrasion during storage.

- Warranty

  - Provide manufacturer’s standard warranty.

    - Warranty terms: three years from date of invoice against defects in materials and workmanship.

Part 2		Products

- Manufacturer

  - Basis-of-design product: 	provide table ensembles based on the product named:

    - Vector Table Ensemble by Forms+Surfaces.

    - Manufacturer Contact:
Forms+Surfaces
1901 William Flynn Highway 
Glenshaw, PA 15116
phone: 800-451-0410

fax: 412-385-4715

email: sales@forms-surfaces.com

website: www.forms-surfaces.com

  - Table Ensembles

    - Materials:

      - Frames: aluminum.

      - Slats (Choose one below):

        - FSC 100% Cumaru hardwood (FSC License Code: FSC-C004453).

        - Extruded aluminum.

c.	 Seat backs: aluminum (Optional)

    - Finishes:

      - Frames: polyester powdercoat (Select one option below):

        - Standard Texture from Forms+Surfaces Powdercoat Chart.

        - Custom RAL powdercoat color.

      - Slats, wood: Penofin® hardwood formula, “Transparent Natural.”

      - Slats, metal: Polyester powdercoat (Select one option below):

            - Standard Texture from Forms+Surfaces Powdercoat Chart.

            - Custom RAL powdercoat color.

    - Seat backs: (Optional)

        - Polyester powdercoat to match bench frame.

    - Dimensions:

      - Vector Table Ensemble (set): 72.5" overall length, 63.4" overall depth, 30" height.

      - Vector Table Ensemble (bench): 72.5" length, 16.9" depth, 17.5" height.

      - Vector Table Ensemble (ADA bench): 25.7" length, 16.9" depth, 17.5" height.

      - Vector Table Ensemble (table): 72.5" length, 29.6" depth, 30" height.

    - Weight:

      - Vector Table Ensemble:

        - backed, Cumaru: 374.7 lb

        - backless, Cumaru: 323.8 lb

        - backed, aluminum slats: 409.2 lb

        - backless, aluminum slats: 358.3 lb

      - Vector Table Ensemble with ADA option:

        - backed, Cumaru: 318.5 lb

        - backless, Cumaru: 284.5 lb

        - backed, aluminum slats: 346.6 lb

        - backless, aluminum slats: 312.7 lb

    - Mounting (Choose one below):

      - Freestanding.

      - Surface mount. Stainless steel anchors and mounting hardware are sold separately. Required for tables and benches with seat backs.

Part 3		Execution

- Examination

  - Verify that substrates are stable and capable of supporting the weight of items covered under this section.

  - Verify that substrates have been adequately prepared to securely anchor those items that will be surface mounted.

- Installation

  - Install according to the manufacturer’s installation instructions.

  - Install in conformance to applicable ADA guidelines and End User’s established accessibility policies.

END OF SECTION