DIVISION 26 00 00					ELECTRICAL

SECTION 26 50 00 				LIGHTING

SECTION 26 56 00 				EXTERIOR LIGHTING

APTOS PEDESTRIAN

Part 1		General

- Summary

  - Section Includes:

    - Aptos Pedestrian, single luminaire

    - Aptos Pedestrian, double luminaire

- Submittals

  - Provide submittals in accordance with Section 01 33 00 – Submittal Procedures.

  - Product data:

    - Manufacturer’s standard product literature.

    - Shop drawings.

    - Installation instructions.

    - Maintenance instructions.

- Quality Assurance

  - Manufacturer Qualifications:

    - Minimum 15 years experience in the manufacture of outdoor lighting.

    - Provide reference list of at least ten major transportation authorities, municipalities, universities, or

other high-use public environments currently using outdoor lighting fabricated by the manufacturer.

- Delivery, Storage and Handling

  - Handle products in accordance with manufacturer’s instructions.

  - Store products in manufacturer’s original packaging until ready for installation.

  - Protect products from impacts and abrasion during storage.

- Warranty

  - Provide manufacturer’s standard warranty.

    - Warranty terms: one year from date of invoice against defects in materials and workmanship.

Part 2		Products

- Manufacturer

  - Basis-of-design product: provide pedestrian lighting based on the product named.

    - Aptos Pedestrian by Forms+Surfaces

    - Manufacturer Contact:
Forms+Surfaces
1901 William Flynn Highway 
Glenshaw, PA 15116
phone: 800-451-0410

fax: 412-385-4715

email: sales@forms-surfaces.com

website: www.forms-surfaces.com

  - Pedestrian Lighting

    - Materials:

      - Pole:

        - Aluminum tube

      - Luminaire head:

        - Cast aluminum with frosted acrylic lens

      - Driver: Input power is 120-277 VAC. Includes protection from output open load, over-current and short-circuit, and over-temperature with auto recovery. Compatible with forward phase, reverse-phase, and 0-10V dimmers. Certifications include DLC (DesignLight Consortium) and CA Title 24 technical requirements. ENERGY STAR compliant, IP66 enclosure.

      - Lamps (Choose one lamp below):

        - 3000K LED:

            - High output Cree LEDs

            - Luminaire lumens:

              - Single luminaire: 2,536

              - Double luminaire: 5,202

            - Color temperature: 3,000K

        - 4000K LED:

            - High output Cree LEDs

            - Luminaire lumens:

              - Single luminaire: 2,536

              - Double luminaire: 5,202

            - Color temperature: 4,000K

    - Finish (Choose one finish below):

      - Polyester powdercoat (Select one option below):

        - Standard Texture from Forms+Surfaces Powdercoat Chart.

        - Custom RAL powdercoat color.

    - Dimensions (Choose model(s) below):

      - Single Luminaire:

        - 4" diameter x 144" high pole with 30" L x 19.2" W luminaire

      - Double Luminaire

        - 4" diameter x 144" high pole with (2) 30" L x 19.2" W luminaires

    - Mounting:

      - Surface Mount: 12.5" diameter base plate with ¾"-10 x 24" J-bolt anchors. Mounting hardware sold separately. Template available upon request.

Part 3		Execution

- Examination

  - Verify that substrates are stable and capable of supporting the weight of items covered under this section.

  - Verify that substrates have been adequately prepared to securely anchor those items that will be surface

mounted.

- Installation

  - Install according to the manufacturer’s installation instructions.

  - Install in conformance to applicable ADA guidelines and End User’s established Accessibility policies.

END OF SECTION