DIVISION 12 00 00					FURNISHINGS

SECTION 12 93 00 				SITE FURNISHINGS

SECTION 12 93 23 				TRASH AND LITTER RECEPTORS

TRANSIT RECEPTACLE

Part 1		General

- Summary

  - Section Includes:

    - Stainless steel litter and recycling receptacles.

- Submittals

  - Provide submittals in accordance with Section 01 33 00 – Submittal Procedures.

  - Product data:

    - Manufacturer’s standard product literature.

    - Shop drawings.

    - Installation instructions.

    - Maintenance instructions.

  - Submit powdercoat finish samples for approval.

- Quality Assurance

  - Manufacturer Qualifications:

    - Minimum 15 years experience in the manufacture of litter and recycling receptacles.

    - Provide reference list of at least ten major transportation authorities, municipalities, universities, or

other high-use public environments currently using litter and recycling receptacles fabricated by the manufacturer.

- Delivery, Storage and Handling

  - Handle products in accordance with manufacturer’s instructions.

  - Store products in manufacturer’s original packaging until ready for installation.

  - Protect products from impacts and abrasion during storage.

- Warranty

  - Provide manufacturer’s standard warranty.

    - Warranty terms: Three years from date of invoice against defects in materials and workmanship.

Part 2		Products

- Manufacturer

  - Basis-of-design product: provide receptacles based on the product named:

    - Transit Receptacle by Forms+Surfaces

    - Manufacturer Contact:
Forms+Surfaces
1901 William Flynn Highway 
Glenshaw, PA 15116
phone: 800-451-0410

fax: 412-385-4715

email: sales@forms-surfaces.com

website: www.forms-surfaces.com

  - Receptacles

    - Materials:

      - Structural frame:

        - Base and top castings: solid corrosion-resistant cast aluminum.

        - Vertical supports: stainless steel.

      - Body and swing-out access doors: stainless steel.

      - Lids (Choose one below):

        - Polyethylene (dual-stream and tri-stream, waste-specific models only).

        - Cast aluminum.

      - Optional rain cover: cast aluminum with tubular stainless steel mounting posts. (Must be used with cast aluminum lid).

      - Liners: black polyethylene with UL94HB fire rating.

    - Finishes:

      - Body and swing-out access doors (Choose one finish below):

        - Sandstone.

        - Linen.

        - Diamond.

        - Satin.

      - Lids:

        - Polyethylene lids (dual-stream and tri-stream, waste-specific models only): light texture finish (Choose one color below):

            - Black.

            - Blue.

            - Green.

        - Cast aluminum lids: polyester powdercoat (Select one option below):

            - Standard Texture from Forms+Surfaces Powdercoat Chart.

            - Premium Texture from Forms+Surfaces Powdercoat Chart.

            - Custom RAL powdercoat color.

      - Optional rain cover: polyester powdercoat (to match cast aluminum lid).

    - Dimensions (Select models below):

      - Receptacles:

        - Dual-stream receptacle:

            - 60-gallon, top opening with one 20-gallon and one 40-gallon liner: 40.8" wide x 19.5" deep x 35.9" high.

            - 60-gallon, top opening with one 20-gallon and one 40-gallon liner, with rain cover: 40.8" wide x 19.5" deep x 46.5" high.

        - Tri-stream receptacle:

            - 60-gallon, top opening with three 20-gallon liners: 40.8" wide x 19.5" deep x 35.9" high.

            - 60-gallon, top opening with three 20-gallon liners, with rain cover: 40.8" wide x 19.5" deep x 46.5" high.

      - Lid openings:

        - Dual-stream receptacle:

            - Large opening: 18.4" wide x 8.6" deep.

            - Small opening: 7.1" wide x 8.1" deep.

        - Tri-stream receptacle, waste-specific openings:

            - Round opening (bottles & cans): 6.0" diameter.

            - Oval slot (paper): 2.0" wide x 12.0" deep.

            - Square opening (litter): 8.0" wide x 8.0" deep.

        - Tri-stream receptacle, rectangular openings:

            - Center opening: 6.0” wide x 9.6” deep.

            - Side openings: 8.1” wide x 8.7” deep.

    - Instructional Graphics:

      - Apply instructional graphics to lids as specified to indicate the intended waste or recycling stream.

        - Graphics type: back-printed polycarbonate.

        - Letters and symbols color: White.

        - Graphics background colors:

            - Dual-stream (SLTRA-4020):

              - Black.

              - Blue.

              - Green.

            - Tri-stream, waste-specific openings (SLTRA-180): One strip containing Black, Blue, and Green.

            - Tri-stream, rectangular openings (SLTRA-320):

              - Black.

              - Blue.

              - Green.

    - Installation (Select one below):

      - Freestanding: levelers included.

      - Surface mount: levelers included. Stainless steel anchors and mounting bolts sold separately.

Part 3		Execution

- Examination

  - Verify that substrates are stable and capable of supporting the weight of items covered under this section.

  - Verify that substrates have been adequately prepared to securely anchor those items that will be surface

mounted.

- Installation

  - Install according to the manufacturer’s installation instructions.

  - Install in conformance to applicable ADA guidelines and End User’s established Accessibility policies.

END OF SECTION