DIVISION 12 00 00					FURNISHINGS

SECTION 12 93 00 				SITE FURNISHINGS

SECTION 12 93 43 				SITE SEATING AND TABLES

SECTION 12 93 43.13 		SITE SEATING

HUDSON BENCH

Part 1		General

- Summary

  - Section Includes:

    - Six-foot backless benches, freestanding, FSC 100% Cumaru hardwood

    - Six-foot backless benches, surface mounted, FSC 100% Cumaru hardwood

    - Eight-foot backless benches, freestanding, FSC 100% Cumaru hardwood

    - Eight-foot backless benches, surface mounted, FSC 100% Cumaru hardwood

- Submittals

  - Provide submittals in accordance with Section 01 33 00 – Submittal Procedures.

  - Product data:

    - Manufacturer’s standard product literature.

    - Shop drawings.

    - Installation instructions.

    - Maintenance instructions.

- Quality Assurance

  - Manufacturer Qualifications:

    - Minimum 15 years experience in the manufacture of site seating.

    - Forest Stewardship Council (“FSC”) Certified Supplier. Provide manufacturer’s FSC certification number.

    - Provide reference list of at least ten major transportation authorities, municipalities, universities, or

other high-use public environments currently using site seating fabricated by the manufacturer.

- Delivery, Storage and Handling

  - Handle products in accordance with manufacturer’s instructions.

  - Store products in manufacturer’s original packaging until ready for installation.

  - Protect products from impacts and abrasion during storage.

- Warranty

  - Provide manufacturer’s standard warranty:

    - Warranty terms: three years from date of invoice against defects in materials and workmanship.

Part 2		Products

- Manufacturer

  - Basis-of-design product: provide benches based on the product named:

    - Hudson Benches by Forms+Surfaces.

    - Manufacturer Contact:
Forms+Surfaces
1901 William Flynn Highway 
Glenshaw, PA 15116
phone: 800-451-0410

fax: 412-385-4715

email: sales@forms-surfaces.com

website: www.forms-surfaces.com

  - Benches

    - Materials:

      - Seat slats: nominal 1.5" x 4" solid FSC 100% Cumaru hardwood.

      - Bench frame: FSC 100% Cumaru hardwood slats locked together with concealed 1/2" diameter threaded stainless steel rods.

    - Finish:

      - FSC 100% hardwood slats: penetrating oil finish:

        - Finish color: Penofin® hardwood formula “Transparent Natural.”

    - Dimensions (Select model(s) below):

      - Six-foot bench: 72" long x 18" deep x 16" high.

      - Eight-foot bench: 96" long x 18" deep x 16" high.

    - Installation (Choose mounting option below depending on model):

      - Freestanding.

      - Surface mount: Stainless steel anchors and mounting bolts are sold separately.

Part 3		Execution

- Examination

  - Verify that substrates are stable and capable of supporting the weight of items covered under this section.

  - Verify that substrates have been adequately prepared to securely anchor those items that will be surface

mounted.

- Installation

  - Install according to the manufacturer’s installation instructions.

  - Install in conformance to applicable ADA guidelines and End User’s established Accessibility policies.

END OF SECTION