DIVISION 12 00 00					FURNISHINGS

SECTION 12 93 00 				SITE FURNISHINGS

SECTION 12 93 43 				SITE SEATING AND TABLES

SECTION 12 93 43.13 		SITE SEATING

VISTA CHAIR

Part 1		General

- Summary

  - Section Includes:

    - Stacking chairs with stainless steel frames and perforated seats.

    - Stacking armchairs with stainless steel frames and perforated seats.

- Submittals

  - Provide submittals in accordance with Section 01 33 00 – Submittal Procedures.

  - Product data:

    - Manufacturer’s standard product literature.

    - Shop drawings.

    - Maintenance instructions.

  - Submit powdercoat finish samples for approval.

- Quality Assurance

  - Manufacturer Qualifications:

    - Minimum 15 years experience in the manufacture of site seating.

    - Provide reference list of at least ten major transportation authorities, municipalities, universities, or

other high-use public environments currently using site seating fabricated by the manufacturer.

- Delivery, Storage and Handling

  - Handle products in accordance with manufacturer’s instructions.

  - Store products in manufacturer’s original packaging until ready for installation.

  - Protect products from impacts and abrasion during storage.

- Warranty

  - Provide manufacturer’s standard warranty.

    - Warranty terms: three years from date of invoice against defects in materials and workmanship.

Part 2		Products

- Manufacturer

  - Basis-of-design product: provide chairs and/or armchairs based on the product named:

    - Vista Chair by Forms+Surfaces.

    - Manufacturer Contact:
Forms+Surfaces
1901 William Flynn Highway 
Glenshaw, PA 15116
phone: 800-451-0410

fax: 412-385-4715

email: sales@forms-surfaces.com

website: www.forms-surfaces.com

  - Chairs

    - Materials:

      - Chair frames, armrests: 7/8" diameter heavy wall stainless steel tubing.

      - Perforated seats: 14 gauge stainless steel sheet with overall pattern of .25" diameter perforations.

      - Glides: nylon.

    - Finishes:

      - Polyester powdercoat (Select one option below):

        - Standard Texture/ Gloss from Forms+Surfaces Powdercoat Chart.

        - Premium Texture from Forms+Surfaces Powdercoat Chart.

        - Custom RAL powdercoat color.

    - Dimensions (Select model(s) below):

      - Stacking chair: 20" wide x 22" deep x 32" high. Seat height 18.5."

      - Stacking armchair: 24.5" wide x 22" deep x 32" high. Seat height 18.5." Armrest height 25."

    - Installation: Freestanding.

Part 3		Execution

- Examination

  - Verify that substrates are stable and capable of supporting the weight of items covered under this section.

END OF SECTION