DIVISION 12 00 00					FURNISHINGS

SECTION 12 93 00					SITE FURNISHINGS

SECTION 12 93 43					SITE SEATING AND TABLES

SECTION 12 93 43.13			SITE SEATING

TRIO TABLE ENSEMBLE

Part 1		General

- Summary

  - Section Includes:

    - Table ensemble with cast aluminum frame and FSC 100% Cumaru hardwood slats.

    - Table ensemble with ADA option, cast aluminum frame, and FSC 100% Cumaru hardwood slats.

    - Table ensemble with cast aluminum frame and extruded aluminum slats.

    - Table ensemble with ADA option, cast aluminum frame, and extruded aluminum slats.

- Submittals

  - Provide submittals in accordance with Section 01 33 00 – Submittal Procedures.

  - Product data:

    - Manufacturer’s standard product literature.

    - Shop drawings.

    - Installation instructions.

    - Maintenance instructions.

  - Submit powdercoat finish samples for approval.

- Quality Assurance

  - Manufacturer Qualifications:

    - Minimum 15 years experience in the manufacture of site furniture.

    - Forest Stewardship Council (“FSC”) Certified Supplier. Provide manufacturer’s FSC certification number.

    - Provide reference list of at least ten major transportation authorities, municipalities, universities, or other high-use public environments currently using site seating fabricated by the manufacturer.

- Delivery, Storage and Handling

  - Handle products in accordance with manufacturer’s instructions.

  - Store products in manufacturer’s original packaging until ready for installation.

  - Protect products from impacts and abrasion during storage.

- Warranty

  - Provide manufacturer’s standard warranty.

    - Warranty terms: three years from date of invoice against defects in materials and workmanship.

Part 2		Products

- Manufacturer

  - Basis-of-design product: 	provide table ensembles based on the product named:

    - Trio Table Ensemble by Forms+Surfaces.

    - Manufacturer Contact:
Forms+Surfaces
1901 William Flynn Highway 
Glenshaw, PA 15116
phone: 800-451-0410

fax: 412-385-4715

email: sales@forms-surfaces.com

website: www.forms-surfaces.com

  - Table Ensembles

    - Materials:

      - Frames: cast aluminum.

      - Slats: FSC 100% Cumaru hardwood.

      - Slats: extruded aluminum with integral longitudinal grooves on seating surfaces.

      - Fasteners: stainless steel.

    - Finishes:

      - Frames: polyester powdercoat (Select one option below):

        - Standard Texture from Forms+Surfaces Powdercoat Chart.

        - Custom RAL powdercoat color.

      - Slats, wood: penetrating oil finish.

      - Slats, metal: anodized finish or polyester powdercoat (Choose one option below):

        - Anodized finish: Clear Anodized.

        - Polyester powdercoat: Slate Texture

    - Dimensions:

      - Trio Table Ensemble (set): 74.5" overall length, 65.2" overall depth, 29.7" overall height.

      - Trio Table Ensemble (bench): 74.5" length, 16.5" depth, 18.0" height.

      - Trio Table Ensemble (ADA bench): 37.3" length, 16.5" depth, 18.0" height.

      - Trio Table Ensemble (table): 74.2" length, 29.4" depth, 29.7" height.

    - Weight:

      - Trio Table Ensemble: 340 lbs.

      - Trio Table Ensemble with ADA option: 318 lbs.

    - Mounting: Surface mount. Stainless steel anchors and mounting bolts are sold separately.

Part 3		Execution

- Examination

  - Verify that substrates are stable and capable of supporting the weight of items covered under this section.

  - Verify that substrates have been adequately prepared to securely anchor those items that will be surface mounted.

- Installation

  - Install according to the manufacturer’s installation instructions.

  - Install in conformance to applicable ADA guidelines and End User’s established accessibility policies.

END OF SECTION