DIVISION 12 00 00					FURNISHINGS

SECTION 12 93 00					SITE FURNISHINGS

SECTION 12 93 43					SITE SEATING AND TABLES

SECTION 12 93 43.13			SITE SEATING

TANGENT RAIL SEATING

Part 1		General

- Summary

  - Section Includes:

    - Backed seats, Straight rail, extruded aluminum slats.

    - Backed seats, Straight rail, FSC 100% Cumaru hardwood slats. (FSC License Code: FSC-C004453)

    - Backed seats, Straight rail, Stainless Steel pans.

    - Backless seats, Straight rail, extruded aluminum slats.

    - Backless seats, Straight rail, FSC 100% Cumaru hardwood slats.

    - Backless seats, Straight rail, Stainless Steel pans.

- Submittals

  - Provide submittals in accordance with Section 01 33 00 – Submittal Procedures.

  - Product data:

    - Manufacturer’s standard product literature.

    - Shop drawings.

    - Installation instructions.

    - Maintenance instructions.

  - Submit powdercoat finish samples for approval.

- Quality Assurance

  - Manufacturer Qualifications:

    - Minimum 15 years experience in the manufacture of site seating.

    - Provide reference list of at least ten major transportation authorities, municipalities, universities, or other high-use public environments currently using site seating fabricated by the manufacturer.

- Delivery, Storage and Handling

  - Handle products in accordance with manufacturer’s instructions.

  - Store products in manufacturer’s original packaging until ready for installation.

  - Protect products from impacts and abrasion during storage.

- Warranty

  - Provide manufacturer’s standard warranty.

      - Warranty terms: three years from date of invoice against defects in materials and workmanship.

Part 2		Products

- Manufacturer

  - Basis-of-design product: 	provide rail seating based on the product named:

    - Tangent Rail Seating by Forms+Surfaces.

    - Manufacturer Contact:
Forms+Surfaces
1901 William Flynn Highway 
Glenshaw, PA 15116
phone: 800-451-0410

fax: 412-385-4715

email: sales@forms-surfaces.com

website: www.forms-surfaces.com

  - Rail Seating

    - Materials:

      - Seat frames: die-cast aluminum.

      - Intermediate armrests: die-cast aluminum. (Optional)

      - Connection castings: die-cast aluminum.

      - Legs: Extruded aluminum and cast aluminum post leg.

      - Rail: extruded aluminum.

      - Rail end caps: die-cast aluminum.

      - Seat Type (Choose one option below):

        - Backed.

        - Backless.

      - Seats (Choose one option below):

        - Extruded aluminum slats.

        - FSC 100% Cumaru hardwood slats.

        - Stainless steel seat pan.

      - Fasteners: stainless steel.

    - Finishes

      - Seat frames, intermediate armrests (Optional), connection castings, surface mount feet (Optional), rail, legs: polyester powdercoat (Select one option below):

        - Standard Texture from Forms+Surfaces Powdercoat Chart.

        - Custom RAL powdercoat color.

      - Seats (Choose one option below):

        - Extruded aluminum slats: polyester powdercoat (Select one option below):

            - Standard Texture from Forms+Surfaces Powdercoat Chart.

            - Custom RAL powdercoat color.

        - FSC 100% Cumaru hardwood slats: natural oiled finish.

        - Stainless steel pan (Choose one finish below):

            - Polyester powdercoat (Select one option below):

              - Standard Texture from Forms+Surfaces Powdercoat Chart.

              - Custom RAL powdercoat color.

            - Stainless steel finish: Sandstone.

            - Stainless steel perforation: Fade.

    - Dimensions (Choose one option below):

      - One backed seat: 19.4" long x 20.2" deep x 32.1" high.

      - Two backed seats: 43.4" long x 20.2" deep x 32.1" high.

      - Three backed seats: 67.4" long x 20.2" deep x 32.1" high.

      - Four backed seats: 91.4" long x 20.2" deep x 32.1" high.

      - Five backed seats: 115.4" long x 20.2" deep x 32.1" high.

      - One backless seat: 19.4" long x 16.2" deep x 21.8" high.

      - Two backless seats: 43.4" long x 16.2" deep x 21.8" high.

      - Three backless seats: 67.4" long x 16.2" deep x 21.8" high.

      - Four backless seats: 91.4" long x 16.2" deep x 21.8" high.

      - Five backless seats: 115.4" long x 16.2" deep x 21.8" high.

NOTE: Dimensions shown are for surface mounted models. 11 inches are added to the leg length for

cast-in-place models for the portion to be embedded underground.

    - Mounting (Choose one option below):

      - Surface mount. Stainless steel anchors and mounting bolts are sold separately.

      - Cast-in-place. Stainless steel anchors and mounting bolts are sold separately.

Part 3		Execution

- Examination

  - Verify that substrates are stable and capable of supporting the weight of items covered under this section.

  - Verify that substrates have been adequately prepared to securely anchor those items that will be surface mounted.

- Installation

  - Install according to the manufacturer’s installation instructions.

  - Install in conformance to applicable ADA guidelines and End User’s established Accessibility policies.

END OF SECTION