DIVISION 12 00 00 				FURNISHINGS

SECTION 12 93 00 				SITE FURNISHINGS

SECTION 12 93 43 				SITE SEATING AND TABLES

SECTION 12 93 43.13 		SITE SEATING

KNIGHT BENCH

Part 1		General

- Summary

  - Section Includes:

    - Backed bench, 6-foot, aluminum frame, FSC 100% Cumaru hardwood slats.

    - Backless bench, 6-foot, aluminum frame, FSC 100% Cumaru hardwood slats.

    - Backed bench, 8-foot, aluminum frame, FSC 100% Cumaru hardwood slats.

    - Backless bench, 8-foot, aluminum frame, FSC 100% Cumaru hardwood slats.

    - Backed bench, 6-foot, aluminum frame, FSC Recycled 100% Teak hardwood slats.

    - Backless bench, 6-foot, aluminum frame, FSC Recycled 100% Teak hardwood slats.

    - Backed bench, 6-foot, aluminum frame, extruded aluminum slats.

    - Backless bench, 6-foot, aluminum frame, extruded aluminum slats.

    - Backed bench, 8-foot, aluminum frame, extruded aluminum slats.

    - Backless bench, 8-foot, aluminum frame, extruded aluminum slats.

    - Backed bench, 6-foot, wall-top mount, aluminum frame, FSC 100% Cumaru hardwood slats.

    - Backless bench, 6-foot, wall-top mount, aluminum frame, FSC 100% Cumaru hardwood slats.

    - Backed bench, 8-foot, wall-top mount, aluminum frame, FSC 100% Cumaru hardwood slats.

    - Backless bench, 8-foot, wall-top mount, aluminum frame, FSC 100% Cumaru hardwood slats.

    - Backed bench, 6-foot, wall-top mount, aluminum frame, FSC Recycled 100% Teak hardwood slats.

    - Backless bench, 6-foot, wall-top mount, aluminum frame, FSC Recycled 100% Teak hardwood slats.

    - Backed bench, 6-foot, wall-top mount, aluminum frame, extruded aluminum slats.

    - Backless bench, 6-foot, wall-top mount, aluminum frame, extruded aluminum slats.

    - Backed bench, 8-foot, wall-top mount, aluminum frame, extruded aluminum slats.

    - Backless bench, 8-foot, wall-top mount, aluminum frame, extruded aluminum slats.

- Submittals

  - Provide submittals in accordance with Section 01 33 00 – Submittal Procedures.

  - Product data:

    - Manufacturer’s standard product literature.

    - Shop drawings.

    - Installation instructions.

    - Maintenance instructions.

  - Submit powdercoat finish samples for approval.

- Quality Assurance

  - Manufacturer Qualifications:

    - Minimum 15 years experience in the manufacture of site seating.

    - Forest Stewardship Council (“FSC”) Certified Supplier. Provide manufacturer’s FSC certification number.

    - Provide reference list of at least ten major transportation authorities, municipalities, universities, or

other high-use public environments currently using site seating fabricated by the manufacturer.

- Delivery, Storage and Handling

  - Handle products in accordance with manufacturer’s instructions.

  - Store products in manufacturer’s original packaging until ready for installation.

  - Protect products from impacts and abrasion during storage.

- Warranty

  - Provide manufacturer’s standard warranty:

    - Warranty terms: three years from date of invoice against defects in materials and workmanship.

Part 2		Products

- Manufacturer

  - Basis-of-design product: provide backed and/or backless benches based on the product named:

    - Knight Bench by Forms+Surfaces.

    - Manufacturer Contact:
Forms+Surfaces
1901 William Flynn Highway 
Glenshaw, PA 15116
phone: 800-451-0410

fax: 412-385-4715

email: sales@forms-surfaces.com

website: www.forms-surfaces.com

  - Benches

    - Materials:

      - Seat slats (Choose one below):

        - FSC 100% Cumaru hardwood.

        - FSC 100% Recycled Teak hardwood.

        - Extruded aluminum.

      - Bench frame: solid aluminum with invisible welds.

      - Slat fasteners: recessed stainless steel.

      - Optional armrests: solid aluminum.

    - Finishes

      - Bench slats: (Choose one below):

        - Wood: Penofin® hardwood formula “Transparent Natural.”

        - Aluminum: Polyester Powder coat (Select one option below):

            - Standard Texture from Forms+Surfaces Powder coat Chart.

            - Custom RAL Powder coat color

      - Bench frame (outer frame edges): polished aluminum.

      - Bench frame (inner surfaces): polyester powdercoat (Select one option below):

        - Standard Texture from Forms+Surfaces Powder coat Chart.

        - Custom RAL Powder coat color.

      - Armrests (outer edges): polished aluminum.

      - Armrests (flat surfaces): polyester Powdercoat to match frame.

    - Dimensions (Choose model below):

      - Backed bench, six-foot, surface mount

        - Overall dimensions: 72" long x 22.7" deep x 31.1" high.

        - Seat dimensions: 15.2" deep x 18" high.

        - Back height: 15.2".

        - Armrest height: 20.8".

      - Backless bench, six-foot, surface mount

        - Overall dimensions: 72" long x 19.8" deep x 18" high.

        - Seat dimensions: 19.8" deep x 18" high.

        - Armrest height: 19.5".

      - Backed bench, eight-foot, surface mount

        - Overall dimensions: 96" long x 22.7" deep x 31.1" high.

        - Seat dimensions: 15.2" deep x 18" high.

        - Back height: 15.2".

        - Armrest height: 20.75".

      - Backless bench, eight-foot, surface mount

        - Overall dimensions: 96" long x 19.8" deep x 18" high.

        - Seat dimensions: 19.8" deep x 18" high.

        - Armrest height: 19.5".

      - Backed bench, six-foot, wall-top mount

        - Overall dimensions: 72" long x 22.7" deep x 19.2" high.

        - Seat dimensions: 15.2" deep x 17.9" high.

        - Back height: 15.2".

        - Armrest height: 20.8".

      - Backless bench, six-foot, wall-top mount

        - Overall dimensions: 72" long x 19.8" deep x 3.3" high.

        - Seat dimensions: 19.8" deep x 3.3" high.

        - Armrest height: 19.5".

      - Backed bench, eight-foot, wall-top mount

        - Overall dimensions: 96" long x 22.7" deep x 19.2" high.

        - Seat dimensions: 15.2" deep x 17.9" high.

        - Back height: 15.2".

        - Armrest height: 20.75".

      - Backless bench, eight-foot, wall-top mount

        - Overall dimensions: 96" long x 19.8" deep x 3.3" high.

        - Seat dimensions: 19.8" deep x 3.3" high.

        - Armrest height: 19.5".

    - Mounting:

      - Surface mount. Stainless steel anchors and mounting bolts are sold separately.

      - Wall-top. Provide mounting hardware based on installation.

Part 3		Execution

- Examination

  - Verify that substrates are stable and capable of supporting the weight of items covered under this section.

  - Verify that substrates have been adequately prepared to securely anchor those items that will be surface

mounted.

- Installation

  - Install according to the manufacturer’s installation instructions.

  - Install in conformance to applicable ADA guidelines and End User’s established Accessibility policies.

END OF SECTION