DIVISION 12 00 00					FURNISHINGS

SECTION 12 93 00 				SITE FURNISHINGS

SECTION 12 93 43 				SITE SEATING AND TABLES

SECTION 12 93 43.53 		SITE TABLES

COLUMN TABLE

Part 1		General

- Summary

  - Section Includes: (Select model(s) below):

    - 24" base, 24" round top, table height.

    - 24" base, 30" round top, table height.

    - 30" base, 36" round top, table height.

    - 30" base, 42" round top, table height.

    - 30" base, 36" x 36" square top, table height.

- Submittals

  - Provide submittals in accordance with Section 01 33 00 – Submittal Procedures.

  - Product data:

    - Manufacturer’s standard product literature.

    - Shop drawings.

    - Installation instructions.

    - Maintenance instructions.

- Quality Assurance

  - Manufacturer Qualifications:

    - Minimum 15 years experience in the manufacture of outdoor tables.

    - Provide reference list of at least ten major transportation authorities, municipalities, universities, or

other high-use public environments currently using outdoor tables fabricated by the manufacturer.

- Delivery, Storage and Handling

  - Handle products in accordance with manufacturer’s instructions.

  - Store products in manufacturer’s original packaging until ready for installation.

  - Protect products from impacts and abrasion during storage.

- Warranty

  - Provide manufacturer’s standard warranty.

    - Warranty terms: three years from date of invoice against defects in materials and workmanship.

Part 2		Products

- Manufacturer

  - Basis-of-design product: provide table pedestals based on the product named:

    - Column Table by Forms+Surfaces.

    - Manufacturer Contact:
Forms+Surfaces
1901 William Flynn Highway 
Glenshaw, PA 15116
phone: 800-451-0410

fax: 412-385-4715

email: sales@forms-surfaces.com

website: www.forms-surfaces.com

  - Table Tops

    - Material: spun stainless steel.

    - Finish (Select one finish below):

      - Stainless steel finish:

        - Sandstone.

        - Diamond.

        - Satin.

      - Polyester powdercoat (Select one option below):

        - Standard Texture/Gloss from Forms+Surfaces Powdercoat Chart.

        - Custom RAL powdercoat color.

      - With or without a center hole to receive a 1.5" (38 mm) diameter umbrella pole. When an umbrella hole is specified, a nylon collar and cap are included.

    - Perforation pattern: Concentric pattern of ¼" round perforations. (Optional. For round or square table tops only).

  - Table Bases and Columns

    - Materials:

      - Vertical supports: 3" diameter stainless steel tubing.

      - Base plates: stainless steel plate.

    - Finish (Select one finish below):

      - Stainless steel finish: Satin.

      - Polyester powdercoat (Select one option below):

        - Standard Texture/Gloss from Forms+Surfaces Powdercoat Chart.

        - Custom RAL powdercoat color.

    - Dimensions (Select model(s) below):

      - Table height: 28.3" high.

      - Bar height: 41.8" high.

    - Mounting (Select one below):

      - Freestanding.

      - Surface mount. Stainless steel anchors and tamper-resistant stainless steel bolts are sold separately. Surface mount is mandatory for certain models. See Product Data Sheet or Quote/Order Form for details.

Part 3		Execution

- Examination

  - Verify that substrates are stable and capable of supporting the weight of items covered under this section.

  - Verify that substrates have been adequately prepared to securely anchor those items that will be surface

mounted.

- Installation

  - Install according to the manufacturer’s installation instructions.

  - Install in conformance to applicable ADA guidelines and End User’s established Accessibility policies.

END OF SECTION