DIVISION 26 00 00					ELECTRICAL

SECTION 26 50 00 				LIGHTING

SECTION 26 56 00 				EXTERIOR LIGHTING

LIGHT COLUMN BOLLARD WITH EMBEDDED SECURITY CORE

Part 1		General

- Summary

  - Section Includes:

    - Series 600 S10-P1 & SC30-P1 illuminated security bollard.

    - Series 600 S10-P1 & SC30-P1 non-illuminated security bollard.

- References

  - ASTM A 53 – Standard Specification for Pipe, Steel, Black, and Hot-Dipped, Zinc Coated, Welded and Seamless.

  - ASTM A 123 – Specification for Zinc (hot-dip galvanized) Coatings on Iron and Steel Products.

- Performance Requirements

  - Bollards must meet minimum level of S10- P1 or SC30-P1 as identified by the Standard Test Method for Vehicle Crash Testing of Perimeter Barriers. Manufacturer must provide documentation confirming that a finite element analysis has been completed for this product. A S10- P1 is equivalent to stopping a 5,000 lb. vehicle traveling at 10 mph in less than 1 meter and A SC30-P1 rating is equivalent to stopping a 2,430 lb. vehicle traveling at 30 mph in less than 1 meter.

- Submittals

  - Provide submittals in accordance with Section 01 33 00 – Submittal Procedures.

  - Product data: Manufacturer’s standard data sheets on each product to be used, including:

    - Manufacturer’s standard product literature.

    - Shop Drawings.

    - Installation instructions.

    - Maintenance instructions.

  - Submit finish samples for approval.

- Quality Assurance

  - Manufacturer Qualifications:

    - Minimum 15 years of experience in the manufacture of outdoor lighting.

    - Provide reference list of at least ten major transportation authorities, municipalities, universities, or

other high-use public environments currently using site products fabricated by the manufacturer.

- Delivery, Storage and Handling

  - Handle products in accordance with manufacturer’s instructions.

  - Store products in manufacturer’s original packaging until ready for installation.

  - Protect products from impacts and abrasion during storage.

- Warranty

  - Provide manufacturer’s standard warranty:

    - Warranty terms: one year from date of invoice against defects in materials and workmanship.

Part 2		Products

- Manufacturer

  - Basis-of-design product: provide bollards based on the product named.

    - Light Column Security Bollard by Forms+Surfaces

    - Manufacturer Contact:
Forms+Surfaces
1901 William Flynn Highway 
Glenshaw, PA 15116
phone: 800-451-0410

fax: 412-385-4715

email: sales@forms-surfaces.com

website: www.forms-surfaces.com

  - Security Bollards:

    - Materials:

      - Light Column Body: tubular stainless steel.

      - Light Column Head (for illuminated bollards): .125" thick white-frosted acrylic tube with stainless steel cap.

      - Optional standard shield configurations (Choose one below, if desired):

        - Sector:

- Series 600, 180° shield.

- Series 600, 360° shield.

        - Strum:

            - Series 600, 180° shield.

            - Series 600, 360° shield.

        - Scale:

            - Series 600, 180° shield.

            - Series 600, 360° shield.

        - Stria:

            - Series 600, 180° shield.

            - Series 600, 360° shield.

        - Solid:

            - Series 600, 180° shield.

      - Driver Input power is 120-277 V. Includes high efficiency constant output current with over-voltage, short circuit, and overload protection. Driver has forward-phase, reverse-phase, and 0-10V dimming capabilities. Certifications include: IP66 (waterproof) enclosure and Class 2 rated output (UL 8750).

      - LED light engine (for illuminated bollards) (Choose one below):

        - 10W custom LED light engine

            - Luminaire lumens: Series 600: 990

            - Color temperature: (Choose one color temperature below):

              - 3,000K.

              - 4,000K.

    - Finish (Choose one finish below):

      - Satin with Ceramiloc treatment

      - Polyester powdercoat (Select one option below):

        - Standard Texture/Gloss from Forms+Surfaces Powdercoat Chart.

        - Custom RAL powdercoat color.

            - Dimensions: 6" diameter x 45" high

    - Mounting (Choose one below):

      - S10-P1 Embedded Deep Set Security Core.

      - SC30-P1 Embedded Shallow Mount Security Core.

Part 3		Execution

  - Examination

- Verify that substrates are stable and capable of supporting the weight of items covered under this section.

- Verify that foundation, applied finishes, and adjacent construction are ready to receive bollards and are level, plumb, and square within tolerances acceptable to manufacturer.

- Verify that required utilities are in correct location and are of correct capacities for specified products.

  - Installation

- Install in accordance with manufacturer's installation instructions.

- Install in conformance to applicable ADA guidelines and End User’s established accessibility policies.

END OF SECTION