DIVISION 26 00 00					ELECTRICAL

SECTION 26 50 00 				LIGHTING

SECTION 26 56 00 				EXTERIOR LIGHTING

LIGHT COLUMN BOLLARD

Part 1		General

- Summary

  - Section Includes:

    - Series 500 bollard, 5" diameter column, illuminated

    - Series 500 bollard, 5" diameter column, non-illuminated

    - Series 600 bollard, 6" diameter column, illuminated

    - Series 600 bollard, 6" diameter column, non-illuminated

- Submittals

  - Provide submittals in accordance with Section 01 33 00 – Submittal Procedures.

  - Product data:

    - Manufacturer’s standard product literature.

    - Shop drawings.

    - Installation instructions.

    - Maintenance instructions.

- Quality Assurance

  - Manufacturer Qualifications:

    - Minimum 15 years experience in the manufacture of outdoor lighting.

    - Provide reference list of at least ten major transportation authorities, municipalities, universities, or

other high-use public environments currently using outdoor lighting fabricated by the manufacturer.

- Delivery, Storage and Handling

  - Handle products in accordance with manufacturer’s instructions.

  - Store products in manufacturer’s original packaging until ready for installation.

  - Protect products from impacts and abrasion during storage.

- Warranty

  - Provide manufacturer’s standard warranty.

    - Warranty terms: three years from date of invoice against defects in materials and workmanship.

Part 2		Products

- Manufacturer

  - Basis-of-design product: provide bollards based on the product named.

    - Light Column Bollard by Forms+Surfaces

    - Manufacturer Contact:
Forms+Surfaces
1901 William Flynn Highway 
Glenshaw, PA 15116
phone: 800-451-0410

fax: 412-385-4715

email: sales@forms-surfaces.com

website: www.forms-surfaces.com

  - Bollards

    - Materials:

      - Light Column Body: tubular stainless steel.

      - Light Column Head (for illuminated bollards): .125" thick white-frosted acrylic tube with stainless steel cap.

      - Optional standard shield configurations (Choose one below, if desired):
1)   Sector:
       A)   Series 500, 180° shield.
       B)   Series 600, 180° shield.
       C)   Series 500, 360° shield.
       D)   Series 600, 360° shield.

2)   Strum: 
       A)   Series 500, 180° shield.
       B)   Series 600, 180° shield.
       C)   Series 500, 360° shield.
       D)   Series 600, 360° shield.
3)   Scale:
       A)   Series 500, 180° shield.
       B)   Series 600, 180° shield.
       C)   Series 500, 360° shield.
       D)   Series 600, 360° shield.

4)    Stria:

A)   Series 500, 180° shield.
                                        B)   Series 600, 180° shield.
                                        C)   Series 500, 360° shield.
                                        D)   Series 600, 360° shield.

4)    Solid:

A)   Series 500, 180° shield.
                                        B)   Series 600, 180° shield.

      - Driver (3000K, 4000K, RGBW LED): Input power is 100-277 V. Includes high efficiency constant output current with over-voltage, short circuit, and overload protection. Driver for 3000K and 4000K LED has 0-10V dimming capabilities. Certifications include: IP66 (waterproof) enclosure and Class 2 rated output (UL 8750).

      - LED light engine (for illuminated bollards) (Choose one below):

        - 10W custom LED light engine

            - Luminaire lumens: Series 500: 1,006, Series 600: 990.

            - Color temperature: (Choose one color temperature below):

              - 3,000K.

              - 4,000K.

        - 78W custom RGBW LED light engine.

            - Luminaire lumens: Varies

            - Color temperature: 4,000K white, RGBW, dynamic white

      - GFCI outlet (Series 600 illuminated, non-RGBW, non-security bollard only) (Optional)

    - Finish (Choose one finish below):

      - Satin with Ceramiloc treatment

      - Polyester powdercoat (Select one option below):

        - Standard Texture/Gloss from Forms+Surfaces Powdercoat Chart.

        - Custom RAL powdercoat color.

    - Dimensions (Choose model(s) below):

      - Series 500 bollards:

        - 5" diameter x 45" high.

      - Series 600 bollards:

        - 6" diameter x 45" high.

    - Mounting (Choose one below):

      - Standard:

        - Surface Mount with .50" thick base plate with J-bolts, slotted for rotational capability and stainless steel cover provided. Stainless steel mounting hardware sold separately. Template available upon request.

      - Optional:

        - S10-P1 Embedded Deep Set Security Core. (not available in Series 500 or RGBW)

        - SC30-P1 Embedded Shallow Mount Security Core. (not available in Series 500 or RGBW)

    - RGBW Requirements

      - DMX512 protocol via onboard decoder.

      - DMX512 compatible controller required (provided by others).

Part 3		Execution

- Examination

  - Verify that substrates are stable and capable of supporting the weight of items covered under this section.

  - Verify that substrates have been adequately prepared to securely anchor those items that will be surface

mounted.

- Installation

  - Install according to the manufacturer’s installation instructions.

  - Install in conformance to applicable ADA guidelines and End User’s established Accessibility policies.

END OF SECTION