DIVISION 26 00 00					ELECTRICAL

SECTION 26 50 00 				LIGHTING

SECTION 26 56 00 				EXTERIOR LIGHTING

TANGENT BOLLARD

Part 1		General

- Summary

  - Section Includes:

    - Tangent Bollard.

- Submittals

  - Provide submittals in accordance with Section 01 33 00 – Submittal Procedures.

  - Product data:

    - Manufacturer’s standard product literature.

    - Shop drawings.

    - Installation instructions.

    - Maintenance instructions.

  - Submit powdercoat finish samples for approval.

- Quality Assurance

  - Manufacturer Qualifications:

    - Minimum 15 years experience in the manufacture of outdoor lighting.

    - Provide reference list of at least ten major transportation authorities, municipalities, universities, or

other high-use public environments currently using outdoor lighting fabricated by the manufacturer.

- Delivery, Storage and Handling

  - Handle products in accordance with manufacturer’s instructions.

  - Store products in manufacturer’s original packaging until ready for installation.

  - Protect products from impacts and abrasion during storage.

- Warranty

  - Provide manufacturer’s standard warranty:

    - Warranty terms: three years from date of invoice against defects in materials and workmanship.

Part 2		Products

- Manufacturer

  - Basis-of-design product: provide bollard(s) based on the product named:

    - Tangent Bollard by Forms+Surfaces.

    - Manufacturer Contact:
Forms+Surfaces
1901 William Flynn Highway 
Glenshaw, PA 15116
phone: 800-451-0410

fax: 412-385-4715

email: sales@forms-surfaces.com

website: www.forms-surfaces.com

  - Bollards:

    - Materials:

      - Body: two aluminum castings with an edge-lit LED light panel.

      - Lens: Edge-lit, light-conducting PMMA.

      - Diffuser: tempered-glass.

      - LED Driver:  Input power is 90 – 305 VAC. Includes high efficiency, constant output current, with over-voltage, short circuit, overload protection, and has 0-10V dimming capabilities. Certifications include: IP66 (waterproof) enclosure, and Class A sound rating.

      - LED Light Engine

        - 22W custom LED light engine.

        - Luminaire lumens: 294.

        - Color temperature: 4,000K.

      - Hardware: stainless steel.

    - Finish: polyester powdercoat (Select one option below):

      - Standard Texture from Forms+Surfaces Powdercoat Chart.

      - Custom RAL Powdercoat color.

    - Dimensions: 40.9" high x 7.6" wide x 2.5" deep.

    - Mounting: Surface mount with 7.6” x 7” base using four tamper-resistant bolts. Stainless steel mounting hardware sold separately. Templates available upon request.

Part 3		Execution

- Examination

  - Verify that substrates are stable and capable of supporting the weight of items covered under this section.

  - Verify that substrates have been adequately prepared to securely anchor those items that will be surface

mounted.

- Installation

  - Install according to the manufacturer’s installation instructions.

  - Install in conformance to applicable ADA guidelines and End User’s established accessibility policies.

END OF SECTION