DIVISION 26 00 00					ELECTRICAL

SECTION 26 50 00 				LIGHTING

SECTION 26 56 00 				EXTERIOR LIGHTING

LIGHT COLUMN PEDESTRIAN

Part 1		General

- Summary

  - Section Includes:

    - Series 500 pedestrian, 5" diameter column.

    - Series 600 pedestrian, 6" diameter column.

- Submittals

  - Provide submittals in accordance with Section 01 33 00 – Submittal Procedures.

  - Product data:

    - Manufacturer’s standard product literature.

    - Shop drawings.

    - Installation instructions.

    - Maintenance instructions.

- Quality Assurance

  - Manufacturer Qualifications:

    - Minimum 15 years experience in the manufacture of outdoor lighting.

    - Provide reference list of at least ten major transportation authorities, municipalities, universities, or

other high-use public environments currently using outdoor lighting fabricated by the manufacturer.

- Delivery, Storage and Handling

  - Handle products in accordance with manufacturer’s instructions.

  - Store products in manufacturer’s original packaging until ready for installation.

  - Protect products from impacts and abrasion during storage.

- Warranty

  - Provide manufacturer’s standard warranty.

    - Warranty terms: three years from date of invoice against defects in materials and workmanship.

Part 2		Products

- Manufacturer:

  - Basis-of-design product: provide pedestrian lighting based on the product named:

    - Light Column Pedestrian Lighting by Forms+Surfaces.

    - Manufacturer Contact:
Forms+Surfaces
1901 William Flynn Highway 
Glenshaw, PA 15116
phone: 800-451-0410

fax: 412-385-4715

email: sales@forms-surfaces.com

website: www.forms-surfaces.com

  - Pedestrian Lighting

    - Materials

      - Body: tubular stainless steel.

      - Lens: .125" thick white-frosted acrylic tube with stainless steel cap.

      - Optional standard stainless steel shield configurations (Choose from below as required):
1)   Kente: 
       A)   Series 600, 180° shield.

2)   Perforation: 
       A)   Series 500, 180° shield.
       B)   Series 600, 180° shield.
       C)   Series 500, 360° shield.
       D)   Series 600, 360° shield.
3)   Scape:
       A)   Series 500, 180° shield.
       B)   Series 600, 180° shield.
       C)   Series 500, 360° shield.
       D)   Series 600, 360° shield.

4)    Bubbles:

            - Series 500, 180° shield.

            - Series 600, 180° shield.

            - Series 500, 360° shield.

            - Series 600, 360° shield.

5)   Ribbon:

            - Series 500, 360° shield.

            - Series 600, 360° shield.

6)    Argyle:

            - Series 500, 360° shield.

            - Series 600, 360° shield.

      - Driver: Input power is 90-305 V. Includes high efficiency constant output current with over-voltage, short circuit, overload protection, and has 0-10V dimming capabilities. Certifications include: IP67 (waterproof) enclosure and Class 2 rated output.

      - LED light engine (for illuminated bollards) (Choose one below):

        - 10W custom LED light engine

            - Luminaire lumens: Series 500: 3783, Series 600: 3790.

            - Color temperature: (Choose one color temperature below):

              - 3,000K.

              - 4,000K.

      - GFCI outlet (Series 600 only) (Optional)

    - Finish:

      - Stainless steel with Satin finish and Ceramiloc treatment.

      - Polyester Powdercoat (Choose one color option below):

        - Standard Texture/Gloss from Forms+Surfaces Powdercoat Chart.

        - Custom RAL Powdercoat color.

    - Dimensions (Select model(s) below):

      - Series 500:

        - 5" diameter x 132.5" high.

      - Series 600.

        - 6" diameter x 132.5" high.

    - Mounting: .50" thick base plate, slotted for rotational capability. Provide stainless steel cover. Stainless steel hardware sold separately. Template available upon request.

Part 3		Execution

- Examination

  - Verify that substrates are stable and capable of supporting the weight of items covered under this section.

  - Verify that substrates have been adequately prepared to securely anchor those items that will be surface

mounted.

- Installation

  - Install according to the manufacturer’s installation instructions.

  - Install in conformance to applicable ADA guidelines and End User’s established Accessibility policies.

END OF SECTION