DIVISION 26 00 00					ELECTRICAL

SECTION 26 50 00 				LIGHTING

SECTION 26 56 00 				EXTERIOR LIGHTING

RINCON PATHWAY BOLLARD

Part 1		General

- Summary

  - Section Includes:

    - Rincon Pathway Bollard.

- Submittals

  - Provide submittals in accordance with Section 01 33 00 – Submittal Procedures.

  - Product data:

    - Manufacturer’s standard product literature.

    - Shop drawings.

    - Installation instructions.

    - Maintenance instructions.

  - Submit powdercoat finish samples for approval.

- Quality Assurance

  - Manufacturer Qualifications:

    - Minimum 15 years experience in the manufacture of outdoor lighting.

    - Provide reference list of at least ten major transportation authorities, municipalities, universities, or

other high-use public environments currently using outdoor lighting fabricated by the manufacturer.

- Delivery, Storage and Handling

  - Handle products in accordance with manufacturer’s instructions.

  - Store products in manufacturer’s original packaging until ready for installation.

  - Protect products from impacts and abrasion during storage.

- Warranty

  - Provide manufacturer’s standard warranty:

    - Warranty terms: one year from date of invoice against defects in materials and workmanship.

Part 2		Products

- Manufacturer

  - Basis-of-design product: provide bollards based on the product named.

    - Rincon Pathway Bollard by Forms+Surfaces

Manufacturer Contact:
Forms+Surfaces

1901 William Flynn Highway 
Glenshaw, PA 15116
phone: 800-451-0410

fax: 412-385-4715

email: sales@forms-surfaces.com

website: www.forms-surfaces.com

  - Bollards

    - Materials:

      - Body: .06" (1.5 mm) stainless steel.

      - Base: stainless steel casting.

      - Head Cap: stainless steel casting.

      - Lens: .25" (6.35 mm) thick white-frosted acrylic.

      - LED Driver: Input power is 100-277 VAC. Includes high efficiency constant output current with over-voltage, short circuit, overload protection, and has 0-10V dimming capabilities. Certifications include: IP66 (waterproof) enclosure and Class 2 rated output (UL 8750.

      - LED Light Engine (Select color temperature below):

        - 14W custom LED light engine

            - Luminaire lumens: 780

2.   Color temperature

              - 3,000K.

              - 4,000K

2. 	Finish:

a. 	Body and Head Cap:

        - Stainless steel with Satin finish and Ceramiloc treatment..

        - Polyester powdercoat (Select one option below):

            - Standard Texture from Forms+Surfaces Powdercoat Chart.

            - Custom RAL powdercoat color.

    - Dimensions: 9.5” high x 4.5" wide x 4.5" deep.

      - Mounting: Surface mount with 3/8”-16 x 8” J-bolt anchors: 3/8” thick stainless steel base plate, slotted for rotational capability. Necessary hardware is included, template available upon request.

Part 3		Execution

- Examination

  - Verify that substrates are stable and capable of supporting the weight of items covered under this section.

  - Verify that substrates have been adequately prepared to securely anchor those items that will be surface

mounted.

- Installation

  - Install according to the manufacturer’s installation instructions.

  - Install in conformance to applicable ADA guidelines and End User’s established accessibility policies.

END OF SECTION