DIVISION 12 00 00 				FURNISHINGS

SECTION 12 93 00 				SITE FURNISHINGS

SECTION 12 93 23 				TRASH AND LITTER RECEPTORS

CORDIA LITTER & RECYCLING RECEPTACLE

Part 1		General

- Summary

  - Section Includes:

    - Litter and Recycling Receptacle, 36-gallon.

- Submittals

  - Provide submittals in accordance with Section 01 33 00 – Submittal Procedures.

  - Product data:

    - Manufacturer’s standard product literature.

    - Shop drawings.

    - Installation instructions.

    - Maintenance instructions.

  - Submit powdercoat finish samples for approval.

- Quality Assurance

  - Manufacturer Qualifications:

    - Minimum 15 years experience in the manufacture of litter and recycling receptacles.

    - Forest Stewardship Council (FSC) Certified Supplier. Provide manufacturer’s FSC certification number.

    - Provide reference list of at least ten major transportation authorities, municipalities, universities, or

other high-use public environments currently using litter and recycling receptacles fabricated by the manufacturer.

- Delivery, Storage, and Handling

  - Handle products in accordance with manufacturer’s instructions.

  - Store products in manufacturer’s original packaging until ready for installation.

  - Protect products from impacts and abrasion during storage.

- Warranty

  - Provide manufacturer’s standard warranty:

    - Warranty terms: three years from date of invoice against defects in materials and workmanship.

Part 2		Products

- Manufacturer

  - Basis-of-design product: provide receptacles based on the product named:

1.	Cordia Receptacle by Forms+Surfaces.

2.	Manufacturer Contact:
Forms+Surfaces
1901 William Flynn Highway 
Glenshaw, PA 15116
phone: 800-451-0410

fax: 412-385-4715

email: sales@forms-surfaces.com

website: www.forms-surfaces.com

  - Receptacles

    - Materials:

      - Body frame, lid, and optional rain cover: corrosion-resistant aluminum.

      - Container walls: corrosion-resistant aluminum.

      - Container wall inset material (Choose one below):

        - FSC 100% Cumaru hardwood slats (FSC License Code: FSC-C004453).

        - Aluminum.

    - Finishes

      - Body frame, and lid (Select one option below):

        - Standard Texture from Forms+Surfaces Powdercoat Chart.

        - Custom RAL powdercoat color.

      - Insets (Choose one below):

        - FSC 100% Cumaru hardwood slats.

        - Aluminum inset, Aluminum Texture powdercoat.

        - Aluminum inset, Argento Texture powdercoat.

        - Aluminum inset, Slate Texture powdercoat.

        - Aluminum inset, other color as specified.

      - Rain cover (optional): polyester powdercoat to match lid and body.

3.	Latch (Choose one below):

        - Standard lift latch.

        - Tool-operated security latch.

- Dimensions (Choose model below):

a. 	Single-Stream receptacle:

1)	36-gallon, top opening, freestanding, without rain cover: 22.1" wide x 22.1" deep x 35.2" high

2)	36-gallon, top opening, surface mount, without rain cover: 22.1" wide x 22.1" deep x 35.2" high

3)	36-gallon, side opening, freestanding, with rain cover: 22.1" wide x 22.1" deep x 43.4" high

4)	36-gallon, side opening, surface mount, with rain cover: 22.1" wide x 22.1" deep x 43.4" high

b. 	Split-Stream receptacle:

1)	36-gallon, top opening, freestanding, without rain cover: 22.1" wide x 22.1" deep x 35.2" high

2)	36-gallon, top opening, surface mount, without rain cover: 22.1" wide x 22.1" deep x 35.2" high

3)	36-gallon, side opening, freestanding, with rain cover: 22.1" wide x 22.1" deep x 43.4" high

4)	36-gallon, side opening, surface mount, with rain cover: 22.1" wide x 22.1" deep x 43.4" high

    - Instructional Graphics:

      - Apply instructional graphics to receptacles, as specified, to indicate the intended waste or recycling stream.

        - Graphics type:

            - Hardwood inset: back-printed acrylic.

            - Aluminum inset: back-printed polycarbonate.

        - Letters and symbols color: White.

        - Graphics background colors:

            - Black.

            - Blue.

            - Green.

6.		Installation (Select one below):

      - Freestanding: levelers provided.

      - Surface mount: levelers provided. Stainless steel anchors and mounting bolts sold separately.

Part 3		Execution

- Examination

  - Verify that substrates are stable and capable of supporting the weight of items covered under this section.

  - Verify that substrates have been adequately prepared to securely anchor those items that will be surface

mounted.

- Installation

  - Install according to the manufacturer’s installation instructions.

  - Install in conformance to applicable ADA guidelines and End User’s established Accessibility policies.

END OF SECTION