DIVISION 26 00 00					ELECTRICAL

SECTION 26 50 00 				LIGHTING

SECTION 26 56 00 				EXTERIOR LIGHTING

CORDIA BOLLARD

Part 1		General

- Summary

  - Section Includes:

    - Cordia Bollard.

- Submittals

  - Provide submittals in accordance with Section 01 33 00 – Submittal Procedures.

  - Product data:

    - Manufacturer’s standard product literature.

    - Shop drawings.

    - Installation instructions.

    - Maintenance instructions.

  - Submit powdercoat finish samples for approval.

- Quality Assurance

  - Manufacturer Qualifications:

    - Minimum 15 years experience in the manufacture of outdoor lighting.

    - Provide reference list of at least ten major transportation authorities, municipalities, universities, or

other high-use public environments currently using outdoor lighting fabricated by the manufacturer.

- Delivery, Storage and Handling

  - Handle products in accordance with manufacturer’s instructions.

  - Store products in manufacturer’s original packaging until ready for installation.

  - Protect products from impacts and abrasion during storage.

- Warranty

  - Provide manufacturer’s standard warranty:

    - Warranty terms: three years from date of invoice against defects in materials and workmanship.

Part 2		Products

- Manufacturer

  - Basis-of-design product: provide bollards based on the product named.

    - Cordia Bollard by Forms+Surfaces

    - Manufacturer Contact:
Forms+Surfaces
1901 William Flynn Highway 
Glenshaw, PA 15116
phone: 800-451-0410

fax: 412-385-4715

email: sales@forms-surfaces.com

website: www.forms-surfaces.com

  - Bollards

    - Materials:

      - Body: corrosion-resistant cast aluminum with a powdercoat finish.

      - Base: corrosion-resistant aluminum with a powdercoat finish.

      - Head: corrosion-resistant aluminum with a powdercoat finish.

      - Lens: optically clear acrylic lens.

      - Optional directional shield with 180° coverage.

      - Driver: Input power is 100 - 277 VAC. Includes high efficiency constant output current with over-voltage, short circuit, overload protection, and has 0-10V dimming capabilities. Certifications include: IP66 (waterproof) enclosure and Class 2 rated output (UL 8750).

      - LED Light Engine (Select temperature below):

        - LED 3000K

            - 35W

            - Luminaire lumens: 873

3. 	Color temperature: 3,000K

2)	LED 4000K

1.	35W

2.	Luminaire lumens: 988

3. 	Color temperature: 4,000K

    - Finishes: polyester powdercoat (Select one option below):

      - Standard Texture from Forms+Surfaces Powdercoat Chart.

      - Custom RAL powdercoat colors.

    - Dimensions (Select model(s) below):

      - 37.9" high x 9.0" wide x 8.0" deep

    - Mounting: Surface mount with 6.38" diameter base plate using four tamper-resistant bolts. Stainless steel mounting hardware sold separately. Templates available upon request.

Part 3		Execution

- Examination

  - Verify that substrates are stable and capable of supporting the weight of items covered under this section.

  - Verify that substrates have been adequately prepared to securely anchor those items that will be surface

mounted.

- Installation

  - Install according to the manufacturer’s installation instructions.

  - Install in conformance to applicable ADA guidelines and End User’s established accessibility policies.

END OF SECTION