DIVISION 12 00 00					FURNISHINGS

SECTION 12 93 00					SITE FURNISHINGS

SECTION 12 93 43					SITE SEATING AND TABLES

SECTION 12 93 43.13			SITE SEATING

APEX BENCH

Part 1		General

- Summary

  - Section Includes:

    - Standalone backless bench with aluminum frame and aluminum slats.

    - Standalone backless bench with aluminum frame and FSC® 100% Cumaru hardwood slats.

    - Backless bench for use with add-on sections with aluminum frame and aluminum slats.

    - Backless bench for use with add-on sections with aluminum frame and FSC 100% Cumaru hardwood slats.

    - Intermediate add-on section with aluminum frame and aluminum slats.

    - Intermediate add-on section with aluminum frame and FSC 100% Cumaru hardwood slats.

    - End add-on section with aluminum frame and aluminum slats.

    - End add-on section with aluminum frame and FSC 100% Cumaru hardwood slats.

    - Wall-top mount bench with aluminum frame and FSC® 100% Cumaru hardwood slats.

    - Wall-top mount bench with aluminum frame and aluminum slats.

(FSC License Code: FSC-C004453)

- Submittals

  - Provide submittals in accordance with Section 01 33 00 – Submittal Procedures.

  - Product data:

    - Manufacturer’s standard product literature.

    - Shop drawings.

    - Installation instructions.

    - Maintenance instructions.

  - Submit powdercoat finish samples for approval.

- Quality Assurance

  - Manufacturer Qualifications:

    - Minimum 15 years experience in the manufacture of site seating.

    - Provide reference list of at least ten major transportation authorities, municipalities, universities, or other high-use public environments currently using site seating fabricated by the manufacturer.

- Delivery, Storage and Handling

  - Handle products in accordance with manufacturer’s instructions.

  - Store products in manufacturer’s original packaging until ready for installation.

  - Protect products from impacts and abrasion during storage.

- Warranty

  - Provide manufacturer’s standard warranty:

    - Warranty terms: three years from date of invoice against defects in materials and workmanship.

Part 2		Products

- Manufacturer

  - Basis-of-design product: 	provide backless bench(es) based on the product named.

    - Apex Bench by Forms+Surfaces

    - Manufacturer Contact:
Forms+Surfaces
1901 William Flynn Highway 
Glenshaw, PA 15116
phone: 800-451-0410

fax: 412-385-4715

email: sales@forms-surfaces.com

website: www.forms-surfaces.com

  - Benches & add-on sections

    - Materials:

      - Frames: aluminum.

      - Seats (Select one option below):

        - Extruded aluminum slats.

        - FSC® 100% Cumaru hardwood slats.

      - Fasteners: stainless steel.

    - Finishes:

      - Frames: polyester powdercoat (Select one option below):

        - Standard Texture from Forms+Surfaces Powdercoat Chart.

        - Custom RAL powdercoat color.

      - Seats (Select one option below):

        - Extruded aluminum slats: polyester powdercoat (Select one option below):

            - Standard Texture from Forms+Surfaces Powdercoat Chart.

            - Custom RAL powdercoat color.

        - FSC® 100% Cumaru: natural oiled finish.

    - Dimensions (Choose model below):

      - Stand-alone bench: 72.0" long x 17.7" deep x 17" high.

      - Bench for use with add-on sections: 72.3" long x 17.7" deep x 17" high.

      - Intermediate add-on section: 72.3" long x 17.7" deep x 17" high.

      - End add-on section: 72.0" long x 17.7" deep x 17" high.

      - If specifying benches with add-on sections, add 0.25" to overall length of final unit per add-on section.

      - Wall-top mount bench: 72.0" long x 17.7" deep x 2.5" high.

    - Mounting (Select one option below):

      - Surface mount with narrow or square ends. Stainless steel anchors and stainless steel mounting hardware sold separately.

      - Freestanding with square ends (for standalone benches only).

      - Wall-top. Provide mounting hardware based on installation.

Part 3		Execution

- Examination

  - Verify that substrates are stable and capable of supporting the weight of items covered under this section.

  - Verify that substrates have been adequately prepared to securely anchor those items that will be surface mounted.

- Installation

  - Install according to the manufacturer’s installation instructions. \

  - Install in conformance to applicable ADA guidelines and End User’s established Accessibility policies.

END OF SECTION