SECTION 14 27 00

ELEVATOR CAB INTERIOR SYSTEMS

- GENERAL

Summary

Section Includes: Elevator cars and elevator car finishes for electric traction and hydraulic elevators. Pre-engineered elevator interior system including rear and side wall panels, handrail and crash rail, stiles and rails, ceiling and lighting. Elevator interior system comes “ready to install” with fasteners, adhesives, and other materials required for a complete assembly. Fasteners shall be concealed type.

Related Sections:

Section 09 77 00 – Special Wall Surfacing

Section 05 75 00 – Decorative Formed Metal

Division 26 – Electrical. General requirements for electrical and lighting specified in this Section.

References

ASTM E84 – Surface Burning Characteristics of Building Materials.

ASME A17.1 – Safety Code for Elevators and Escalators.

CBC 2007 – California Building Code

Submittals

Provide submittals in accordance with Section 01 30 50 and Section 14 21 23 – Submittal Procedures.

Submit manufacturer’s shop drawings, installation drawings, installation instructions and maintenance instructions.

Submit environmental impact data for all materials.

Submit samples no less than 4" x 4" for all specified material finishes.

Submit panel edge extrusion samples no less than 4” of specified finish.

Manufacturer information:

Provide overview literature describing manufacturer’s overall scope of products and manufacturing capabilities.

Provide URL for manufacturer’s web site; web site must provide access to technical data, images and general product information.

Quality Assurance

Manufacturer Qualifications

Minimum 10 years experience in the manufacture of architectural surface materials.

Minimum 10 years experience in the fabrication of elevator cab interiors.

Provide reference list of at least 20 public space projects currently using elevator interiors fabricated by the manufacturer.

Installer Qualifications

Minimum three years experience in the installation of elevators or elevator interiors.

Delivery, Storage and Handling

Deliver materials to installation site in manufacturer’s original packaging. Handle products in accordance with manufacturer’s instructions. Store in dry, secure location, protected against direct sunlight and excessive heat. Protect finished surfaces with strippable film.

Warranty

Provide manufacturer’s standard warranty.

Warranty terms: one year against defects in materials and workmanship.

PRODUCT

Manufacturer

Forms+Surfaces
1901 William Flynn Highway
Glenshaw, PA 15116
Phone: 800-451-0410
Fax: 412-385-3715
Email: sales@forms-surfaces.com
Website: www.forms-surfaces.com

Elevator Rear and Side Wall Panels

General

Pre-engineered elevator interior system including clad panels over fire-rated MDF backer with .020” aluminum sealer sheet and Z-clip mounting system, bases, handrail and ceiling, along with installation template. Wall panels to consist of vertical and horizontal stile and rail framing elements surrounding contrasting insets.

Panel configuration: 2000. See drawing elevations for panel layout.

Provide inset panels in the finishes specified.

Panel Inset Materials

Stainless Steel Insets

Material: Stainless Steel

Finish: Sandstone, Seastone, Linen, Diamond, Mist, Mirror, Satin (choose one)

Impression Pattern: Champagne, Chardonnay, Dallas, Denver, Dune, Kalahari (choose one)

Eco-Etch Pattern: City Lights, Current, Dash, Flicker, Glacier, Mica, Seagrass, Sequence, Silkworm, Stacks, Stripe, Thatch, Trace (choose one)

Fire rating: Stainless Steel is inherently non-combustible and generally considered to be NFPA and IBC Class A fire rated and UBC Class 1 fire rated.

Treatment: Provide Ceramiloc+™ treatment, an invisible, protective treatment that significantly increases the fingerprint resistance and maintenance ease of stainless steel. Patented technology molecularly bonds nano-silica particles to the surface of stainless steel.

Fused Metal Insets

Material: Fused Bronze, Fused Graphite, Fused Nickel Bronze, Fused Nickel Silver, Fused White Gold (choose one)

Finish: Sandstone, Seastone, Linen, Diamond, Mirror, Satin (choose one)

Impression Pattern: Champagne, Chardonnay, Dallas, Denver, Dune, Kalahari (choose one)

Eco-Etch Pattern: City Lights, Current, Dash, Flicker, Glacier, Mica, Reeds, Seagrass, Sequence, Silkworm, Stacks, Stripe, Thatch, Trace (choose one)

Fire rating: Fused Metal is inherently non-combustible and generally considered to be NFPA and IBC Class A fire rated and UBC Class 1 fire rated.

Treatment: Provide Ceramiloc+™ treatment, an invisible, protective treatment that significantly increases the fingerprint resistance and maintenance ease of Fused Metal. Patented technology molecularly bonds nano-silica particles to the surface of Fused Metal.

Bonded Metal Insets

Material: Material: Bonded Aluminum, Bonded Bronze (choose one)

Pattern: Austin, Charleston, Dash, Delta, Grass, Herringbone, Loft, Mara, Rain, Trevia, Undercurrent, Vancouver (choose one)

Patina: Natural, Dark (choose one)

Fire rating: Bonded Metal is NFPA and IBC Class B fired rated and UBC class 2 fire rated.

Bonded Quartz Insets

Material: Bonded White, Bonded Charcoal (choose one)

Pattern: Crinkle, Current, Dash, Grass, Loft, Mara, Trevia (choose one)

Fire rating: Bonded Quartz is NFPA and IBC Class B fired rated and UBC class 2 fire rated.

Laminated Glass Insets

Material: VividGlass: ViviChrome, ViviGraphix Gleam, ViviGraphix Graphica, ViviMuse Painterly, ViviMuse Shibori, ViviSpectra Spectrum, ViviSpectra VEKTR, ViviSpectra Zoom, ViviStone, ViviStrata, ViviTela Loom, ViviTela Mesh, Scribe Glass (choose one)

Material: CastGlass: Chromis, Elements (choose one)

Fire rating: VividGlass glass is Class A fire rated in accordance with ASTM designation E84-09, standard test method of surface burning characteristics of building materials. The foregoing test procedure is comparable to UL 723, ANSI/NFPA No. 255, and UBC No. 8-1.

Wood Veneer Insets

Material: Champagne, Cherry, Ebony, Walnut, White Oak Plain, White Oak Rift (choose one)

Stile & Rail Materials

Stainless Steel

Material: Stainless Steel

Finish: Sandstone, Seastone, Linen, Diamond, Mist, Mirror, Satin (choose one)

Substrate: fire-rated particleboard with aluminum backer.

Fused Metal

Material: Fused Bronze, Fused Graphite, Fused Nickel Bronze, Fused Nickel Silver, Fused White Gold (choose one)

Finish: Sandstone, Seastone, Linen, Diamond, Satin, Mirror (choose one)

Substrate: fire-rated particleboard backer.

Corner Reveals

Material: Anodized aluminum

Finish: Clear, Black (choose one)

Handrails

General

Provide handrails on rear (and side) walls of single opening cars, and on side walls of dual opening cars. For elevator door openings that are flush with the adjacent side wall(s), a returned handrail configuration is recommended.

Style: Modular Series, Round Series, Sextant Series, Rectangular Series, Quadrant Series, Compass Series (choose one)

Material: Insert material per Style chosen above

Finish: Insert finish per Style chosen above

Crash Rails

General

Provide crash rail on rear (and side) walls of single opening cars, and on side walls of dual opening cars. For elevator door openings that are flush with the adjacent side wall(s), a returned crash rail configuration is recommended.

Style: 4” x 3/8” flat rail

Material: Stainless Steel

Finish: Satin

Ceiling System

General

Provide Forms+Surfaces Ceiling with inset panels mounted in extruded aluminum ceiling frame on fire-rated polycarbonate backer and integral lighting system.

Provide removable exit hatch as per ASME A17.1 for emergency egress.

Ceiling Inset Material

Stainless Steel

Material: Stainless Steel

Finish: Sandstone, Seastone, Linen, Diamond, Mist, Mirror, Satin (choose one)

Fused Metal

Material: Fused Bronze, Fused Graphite, Fused Nickel Bronze, Fused Nickel Silver, Fused White Gold (choose one)

Finish: Sandstone, Seastone, Linen, Diamond, Mirror, Satin (choose one)

LED Lighting

Configuration: Recessed low-voltage LED downlights per drawings, Recessed low-voltage LED downlights with low-voltage LED perimeter lighting, High-output LED perimeter lighting (choose one)

Color Temperature: 3000K, 5000K (choose one)

Associated Components

Materials for car operating panels, return panels, doors, jambs, transoms, and floor are not included in this section.

EXECUTION

Preparation

Protect elevator finishes, fixtures and equipment from damage caused by work of this Section.

Installation

Do not make structural changes to elevator cab. Do not install work in a manner that interferes with the safe operation of elevator.

Install in accordance with elevator interior system manufacturer’s instructions.

Cleaning and Protection

Remove strippable film. Clean exposed surface in accordance with manufacturer’s instructions.

Protect exposed surfaces from damage by subsequent construction.

- END OF SECTION