DIVISION 12 00 00					FURNISHINGS

SECTION 12 93 00 				SITE FURNISHINGS

SECTION 12 93 43 				SITE SEATING AND TABLES

SECTION 12 93 43.13 		SITE SEATING

LINIA CAFÉ CHAIR

Part 1		General

- Summary

  - Section Includes:

    - Café chair.

- Submittals

  - Provide submittals in accordance with Section 01 33 00 – Submittal Procedures.

  - Product data:

    - Manufacturer’s standard product literature.

    - Shop drawings.

    - Maintenance instructions.

  - Submit powdercoat finish samples for approval.

- Quality Assurance

  - Manufacturer Qualifications:

    - Minimum 15 years experience in the manufacture of site seating.

    - Provide reference list of at least ten major transportation authorities, municipalities, universities, or

other high-use public environments currently using site seating fabricated by the manufacturer.

- Delivery, Storage and Handling

  - Handle products in accordance with manufacturer’s instructions.

  - Store products in manufacturer’s original packaging until ready for installation.

  - Protect products from impacts and abrasion during storage.

- Warranty

  - Provide manufacturer’s standard warranty.

    - Warranty terms: three years from date of invoice against defects in materials and workmanship.

Part 2		Products

- Manufacturer

  - Basis-of-design product: provide armchairs based on the product named:

    - Linia Café Chair by Forms+Surfaces.

    - Manufacturer Contact:
Forms+Surfaces
1901 William Flynn Highway 
Glenshaw, PA 15116
phone: 800-451-0410

fax: 412-385-4715

email: sales@forms-surfaces.com

website: www.forms-surfaces.com

  - Chairs

    - Materials:

      - Frames:

        - 12mm 304 stainless steel rod.

      - Seats and seat backs (Select one option below):

        - 2mm stainless steel.

      - Glides/bumpers:

        - Nylon.

    - Finishes:

      - Chair frame: (Select one option below):

        - Electropolished.

        - Standard Texture from Forms+Surfaces Powdercoat Chart.

        - Custom RAL powdercoat color.

      - Seats and seat backs:

        - Polyester powdercoat (Select one option below):

            - Standard Texture from Forms+Surfaces Powdercoat Chart.

            - Custom RAL powdercoat color.

        - Seat perforation options (Select one option below):

            - No perforation.

            - Sweep perforation.

    - Dimensions:

        - Overall dimensions: 21.3" wide, 21.76" deep, 30.9" high

        - Armrest height: 26.2"

    - Mounting

      - Installation: Freestanding with nylon glides.

Part 3		Execution

- Examination

  - Verify that substrates are stable and capable of supporting the weight of items covered under this section.

  - Verify that substrates have been adequately prepared to securely anchor those items that will be surface

mounted.

- Installation

  - Install according to the manufacturer’s installation instructions.

  - Install in conformance to applicable ADA guidelines and End User’s established Accessibility policies.

END OF SECTION