DIVISION 12 00 00					FURNISHINGS

SECTION 12 93 00					SITE FURNISHINGS

SECTION 12 93 43					SITE SEATING AND TABLES

SECTION 12 93 43.13			SITE SEATING

TANGENT TABLE ENSEMBLE

Part 1		General

- Summary

  - Section Includes:

    - Four-seat ensemble, backed seats, extruded aluminum slats.

    - Four-seat ensemble, backed seats, FSC® 100% Cumaru. hardwood slats.

(FSC License Code: FSC-C004453)

    - Four-seat ensemble, backed seats, stainless steel pans.

    - Four-seat ensemble, backless seats, extruded aluminum slats.

    - Four-seat ensemble, backless seats, FSC 100% Cumaru hardwood slats.

    - Four-seat ensemble, backless seats, stainless steel pans.

    - Six-seat ensemble, backed seats, extruded aluminum slats.

    - Six-seat ensemble, backed seats, FSC 100% Cumaru hardwood slats.

    - Six-seat ensemble, backed seats, stainless steel pans.

    - Six-seat ensemble, backless seats, extruded aluminum slats.

    - Six-seat ensemble, backless seats, FSC 100% Cumaru hardwood slats.

    - Six-seat ensemble, backless seats, stainless steel pans.

- Submittals:

  - Provide submittals in accordance with Section 01 33 00 – Submittal Procedures.

  - Product data:

    - Manufacturer’s standard product literature.

    - Shop drawings.

    - Installation instructions.

    - Maintenance instructions.

  - Submit powdercoat finish samples for approval.

- Quality Assurance:

  - Manufacturer Qualifications:

    - Minimum 15 years experience in the manufacture of site seating.

    - Provide reference list of at least ten major transportation authorities, municipalities, universities, or other high-use public environments currently using site seating fabricated by the manufacturer.

- Delivery, Storage and Handling:

  - Handle products in accordance with manufacturer’s instructions.

  - Store products in manufacturer’s original packaging until ready for installation.

  - Protect products from impacts and abrasion during storage.

- Warranty:

  - Provide manufacturer’s standard warranty.

    - Warranty terms: three years from date of invoice against defects in materials and workmanship.

Part 2		Products

- Manufacturer

  - Basis-of-design product: 	provide table ensemble based on the product named:

    - Tangent Table Ensemble by Forms+Surfaces.

    - Manufacturer Contact:
Forms+Surfaces
1901 William Flynn Highway 
Glenshaw, PA 15116
phone: 800-451-0410

fax: 412-385-4715

email: sales@forms-surfaces.com

website: www.forms-surfaces.com

  - Ensembles:

    - Materials:

      - Frames: cast aluminum and extruded aluminum.

      - Table Tops: 48" across, stainless steel, with or without a center hole to receive a 1.75" diameter umbrella pole.

      - Seat Type(Choose one option below):

        - Backed.

        - Backless.

      - Seats (Choose one option below):

        - Extruded aluminum slats.

        - FSC 100% Cumaru hardwood slats.

        - Stainless steel pans.

      - Fasteners: stainless steel.

    - Finishes:

      - Ensemble frame:

1)	  Cast aluminum and extruded aluminum: polyester powdercoat (Select one option below):

1.	Standard Texture from Forms+Surfaces Powdercoat Chart.

2.	Custom RAL powdercoat color.

      - Seats (Choose one option below):

        - Extruded aluminum slats: polyester powdercoat (Select one option below):

            - Standard Texture from Forms+Surfaces Powdercoat Chart.

            - Custom RAL powdercoat color.

        - FSC 100% Cumaru hardwood slats: natural oiled finish.

        - Stainless steel pan (Choose one finish below):

            - Polyester powdercoat (Select one option below):

              - Standard Texture from Forms+Surfaces Powdercoat Chart.

              - Custom RAL powdercoat color.

            - Stainless steel finish: Sandstone

3. 	Stainless steel perforation: Fade.

      - Table Tops (Choose one option below):

1) 	Stainless steel finish: Sandstone.

        - Powdercoat finish: (Choose one finish below):

            - Polyester powdercoat (Select one option below):

              - Standard Texture from Forms+Surfaces Powdercoat Chart.

              - Custom RAL powdercoat color.

3)	With or without a center hole to receive a 1.5" (38 mm) diameter umbrella pole. When an umbrella hole is specified, a nylon collar and cap are included.

    - Dimensions (Select model(s) below):

      - Four-seat ensembles with backed seats:

        - Overall: 57.2" diameter, 32.9" height, 29.3" table height.

        - Seats: 18.0" wide x 20.2" deep x 18.1" high.

      - Four-seat ensembles with backless seats:

        - Overall: 54.4 " diameter, 29.3" height, 29.3" table height.

        - Seats: 18.0" wide x 20.2" deep x 18.1" high.

      - Six-seat ensembles with backed seats:

        - Overall: 85.3" diameter, 32.9" height, 29.3" table height.

        - Seats: 18.0" wide x 20.2" deep x 18.1" high.

      - Six-seat ensembles with backless seats:

        - Overall: 80.6" diameter, 30.2" height, 29.3" table height.

        - Seats: 18.0" wide x 20.2" deep x 18.1" high.

    - Installation (Choose one option below):

      - Freestanding.

      - Surface mount. Stainless steel anchors and mounting bolts are sold separately.

Part 3		Execution

- Examination

  - Verify that substrates are stable and capable of supporting the weight of items covered under this section.

  - Verify that substrates have been adequately prepared to securely anchor those items that will be surface mounted.

- Installation

  - Install according to the manufacturer’s installation instructions.

  - Install in conformance to applicable ADA guidelines and End User’s established Accessibility policies.

END OF SECTION